3GPP SA WG2 Meeting #164	 S2-2408070

# Maastricht, NL, 19-23 August 2024											                 (was S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1425 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Supporting of network initiated IMS Data Channel |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The conclusions of KI#2 on " Impact on IMS architecture, interfaces and procedures to support IMS capability exposure in the context of IMS data channel session " are agreed: 1. IMS network exposes its capabilities through the NEF, which can be provided by a specific NEF instance serving the IMS network exposure only or by the NEF instance serving the 5GC network exposure simultaneously. 2. The following capabilities need to be exposed by the IMS network: - establishing, updating, terminating of bootstrap data channels; - establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels. 3. The Nnef and Nimsas services and/or service operations need to be enhanced to support the capabilities mentioned above. NOTE: The detailed impacts to the services and/or service operations are defined in normative phase. 4. In Rel-19, the only service provider is IMS AS. The service consumer needs to discover the IMS AS for a given user via HSS before sending a request to IMS AS. If the user is not registered yet, the request is rejected. This CR implements the approved conclusions to support network initiated IMS data channel. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New services and operations in NEF and IMS AS. New procedures on network initiated IMS DC (including BDC and ADC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network initiated IMS DC is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.1, AA.2.4.X, AA.2.Y, AC.2.2.4, AC.2.2.Z, AC.7.3a |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

### AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_DCControl | Create | Request/Response | NEF, DC AS |
|  | Update | Request/Response | NEF, DC AS |
|  | Delete | Request/Response | NEF, DC AS |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

# >>>> NEXT CHANGES (ALL NEW)<<<<

### AA.2.4.X	Nimsas_DCControl Service

#### AA.2.4.X.1	General

Service description: This service enables the consumer to request IMS AS handling of data channel within an IMS session, including establishment, update and release of bootstrap data channel and/or application data channel. The service can be used by the consumer after receiving notification of an IMS session related event.

#### AA.2.4.X.2	Nimsas_DCControl_Create service operation

Service operation name: Nimsas_DCControl_Create

Description: This operation provides requests to the IMS AS for data channels a consumer wants to create based on the received IMS session event. The requested IMS data channel can be bootstrap data channel and/or application data channel.

Inputs, Required: Calling ID, Called ID, Session ID, Session case, Data Channel Mapping and Configuration Information.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session ID specifies the IMS session for which the DCControl operation applies. Session case indicates if this is an originating or terminating IMS session.

Data Channel Mapping and Configuration Information includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.X.3	Nimsas_DCControl_Update service operation

Service operation name: Nimsas_DCControl_Create

Description: This operation provides requests to the IMS AS for data channels a consumer wants to update based on the received IMS session event. The requested IMS data channel can be bootstrap data channel and/or application data channel.

Inputs, Required: Calling ID, Called ID, Session ID, Session case, Data Channel Mapping and Configuration Information.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.X.4	Nimsas_DCControl_Delete service operation

Service operation name: Nimsas_DCControl_Delete

Description: This operation provides requests to the IMS AS for data channels a consumer wants to delete based on the received IMS session event. The requested IMS data channel can be bootstrap data channel and/or application data channel.

Inputs, Required: Calling ID, Called ID, Session ID, Session case, Data Channel Mapping and Configuration Information.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

Outputs, Optional: None.

# >>>> NEXT CHANGES (ALL NEW)<<<<

## AA.2.Y	NEF Services

### AA.2.Y.1	General

In addition to those defined in TS 23.501 [93] clause 7.2.8 and TS 23.502 [94] clause 5.2.6, the following table illustrates additional NEF services to support IMS data channel.

Table AA.2.Y.1-1: NF services provided by the NEF

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_DCControl | Create | Request/Response | DC AS |
|  | Update | Request/Response | DC AS |
|  | Delete | Request/Response | DC AS |

### AA.2.Y.2	Nnef_DCControl service

#### AA.2.Y.2.1	General

Service description: This service enables the consumer to request NEF for handling of data channel within an IMS session, including establishment, update and release of bootstrap data channel and/or application data channel. The service can be used by the consumer after receiving notification of an IMS session related event.

#### AA.2.4.X.2	Nnef_DCControl_Create service operation

Service operation name: Nnef_DCControl_Create

Description: This operation provides requests to the NEF for data channels a consumer wants to create based on the received IMS session event. The requested IMS data channel can be bootstrap data channel and/or application data channel.

Inputs, Required: Calling ID, Called ID, Session ID, Session case, Data Channel Mapping and Configuration Information.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session ID specifies the IMS session for which the DCControl operation applies. Session case indicates if this is an originating or terminating IMS session.

Data Channel Mapping and Configuration Information includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

Inputs, Optional: None.

Outputs, Required: Transaction Reference ID, Result indication.

Transaction Reference ID is assigned by NEF for the specific AF request.

Outputs, Optional: None.

#### AA.2.4.X.3	Nnef_DCControl_Update service operation

Service operation name: Nnef_DCControl_Create

Description: This operation provides requests to the NEF for data channels a consumer wants to update based on the received IMS session event. The requested IMS data channel can be bootstrap data channel and/or application data channel.

Inputs, Required: Calling ID, Called ID, Session ID, Session case, Data Channel Mapping and Configuration Information, Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

#### AA.2.4.X.4	Nnef_DCControl_Delete service operation

Service operation name: Nnef_DCControl_Delete

Description: This operation provides requests to the NEF for data channels a consumer wants to delete based on the received IMS session event. The requested IMS data channel can be bootstrap data channel and/or application data channel.

Inputs, Required: Calling ID, Called ID, Session ID, Session case, Data Channel Mapping and Configuration Information, Transaction Reference ID.

Inputs, Optional: None.

Outputs, Required: Result indication.

Outputs, Optional: None.

Outputs, Optional: None.

# >>>> NEXT CHANGES<<<<

### AC.2.2.4	IMS AS

The IMS AS is enhanced to support the following functionalities:

-	The IMS AS interacts with the DCSF via DC1 for event notifications;

-	The IMS AS receives the data channel control instructions from the DCSF and accordingly interacts with the MF via DC2 or wit MRF via Mr'/Cr for data channel media resource management;

-	The IMS AS interacts with HSS for retrieving and storing DC enhanced IMS AS service specific data via N71/Sh.

-	The IMS AS receives data channel control request from NEF/DC AS for establishment, update or release of bootstrap data channel and/or application data channel.

# >>>> NEXT CHANGES (ALL NEW)<<<<

### AC.2.2.Z	NEF

The existing NEF exposure services is enhanced to support the following functionalities in IMS:

-	The NEF receives data channel request from DC AS for establishment, update or release of bootstrap data channel and/or application data channel.

-	The NEF interacts with IMS AS for the control of data channel upon the request of DC AS.

-	The NEF discovers the serving IMS AS of an IMS session via HSS.

# >>>> NEXT CHANGES (ALL NEW)<<<<

## AC.7.3a	Network Initiated Data Channel Setup Signaling Procedure

Figure AC.7.3a-1 depicts IMS DC establishment procedures requested by a DC AS, including the establishment of both bootstrap data channel and/or application data channel.

## In this procedure, UE-A represents the customer of an enterprise. UE-B represents service representative or automatic IVR in the enterprise network. DC AS is deployed by the enterprise and accesses operator's IMS network via NEF. The IMS session can be originated from UE-A or UE-B. This procedure only demonstrates a UE-A originated IMS session.

Figure AC.7.3a-1: Network Initiated Data Channel set up Signalling Procedure

0a.	UE-A successfully establishes an IMS Session to UE-B.

0b.	DC AS determines to trigger the establishment data channel to UE-A based on local policy or event notification of the related IMS Session.

1.	The DC AS initiates a Nnef_DCControl service request to establish data channel (including bootstrap data channel and/or application data channel). If the service request is authorized by the NEF, the NEF forwards the request to the serving IMS AS via Nimsas_DCControl service and operation. If the DCAS is deployed in the trust domain of IMS, DC AS may send Nimsas_DCControl directly to the IMS AS.

NOTE 1:	NEF or DC AS may discover the address of IMS AS from IMS event notification, or via HSS.

2.	The IMS AS validates the user subscription data and sends a notification to the DCSF for the policy of handling the requested data channel. The IMS AS instruction MF/MRF to allocate the resources in MF/MRF and sends the SDP Offer with data channel request to UE-A. The requested data channel is successfully established when UE-A accepts the SDP Offer.

NOTE 2:	This procedure only presents the data channel between UE-A and the DCSF. Whether a data channel between UE-B and the DCSF is required should be determined by DC AS.

3.	The IMS AS replies with Nimsas_DCControl service response to notify the successful establishment data channel. The NEF forwards the response to DC AS via Nnef_DCControl response.

# >>>>END OF CHANGES<<<<