3GPP TSG-SA2 Meeting #164	S2-2408076

# Maastricht, Netherlands, August 19 – 23, 2024	(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0245 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Local Handling of Edge Computing support with I-SMF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per the conclusion for KI#1 : Enhancements for EAS (re)discovery and UPF (re)selection with reducing impact on central 5GC NFs in TR 23.700-49 of FS_eEDGE_5GC_Ph3, Local handling of Edge Computing with the I-SMF needs to be supported for Session Breakout model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §3.2 - Add abbreviations for the SMF and the I-SMF. §4.2 - Add reference architecture providing access to EAS with I-SMF in the local network. §6.2.5, 6.2.5.1, 6.2.5.2 - Define Local Handling of Edge Computing support with I-SMF procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Local Handling of Edge Computing support with I-SMF is not specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 4.2, 6.2.5 (new), 6.2.5.1 (new), 6.2.5.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 4925, TS 23.503 1348 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [2].

C-DNS	Central DNS

C-NEF	Central NEF

C-PSA UPF	Central PSA UPF

EAS	Edge Application Server

EASDF	Edge Application Server Discovery Function

ECS	Edge Configuration Server

EDC	Edge DNS Client

EEC	Edge Enabler Client

EES	Edge Enabler Server

EHE	Edge Hosting Environment

I-SMF 	Intermediate SMF

L-DN	Local part of DN

L-DNS	Local DNS

L-NEF	Local NEF

L-PSA UPF	Local PSA UPF

HR-SBO	Home Routed Session BreakOut

* * * * Start of 2nd Change * * * *

4.2	Reference Architecture for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501 [2]. The following reference architectures for non-roaming, LBO roaming and HR with Session Breakout (HR-SBO) roaming scenarios further depict the relationship between the 5GS and a DN where Edge Application Servers (EASs) are deployed in an EHE.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

Figure 4.2-5 depicts 5GS architecture for HR-SBO roaming scenario supporting Edge Computing with UL CL/BP.

Figure 4.2-5: 5GS providing access to EAS with UL CL/BP for HR-SBO roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

NOTE 3:	For the HR-SBO roaming scenario, there can be other UPF(s) located in VPLMN between the UPF acting UL CL/BP and the UPF acting as remote PSA in HPLMN.

Figure 4.2-6 depicts 5GS architecture supporting Edge Computing with UL CL/BP and L-PSA controlled by the I-SMF in the local network.

Figure 4.2-6: 5GS providing access to EAS with I-SMF in the local network

* * * * Start of 3rd Change * * * *

6.2.5	Local Handling of Edge Computing support with I-SMF

6.2.5.1	General

For PDU Session with Session Breakout connectivity model, in order to reduce the burden of the central SMF , the AMF may add an I-SMF controls the ULCL/BP/Local PSA (and their associated traffic filters and forwarding rules) and provides access to EAS in the local network.

For the case that the I-SMF is selected to handle the edge computing related traffic, the following applies.

-	The EASDF discovery and selection is performed by the I-SMF instead of the SMF.

-	The DNS message handling rule is configured by the I-SMF.

-	The EAS deployment information may be retrieved from the NEF or locally configured to the I-SMF.

-	The I-SMF may be configured with or receive the Local Offloading Policy from PCF via SMF which indicates IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN.

NOTE 1:	The detailed information of Local Offloading Policy is described in clause 6.4 of TS 23.503 [4].

-	The I-SMF retrieves and subscribes to EAS Deployment Information from the NEF as in clause 6.2.3.4.3.

-	The SMF may forward the AF traffic influence information to the I-SMF.

6.2.5.2	Local Handling of Edge Computing with I-SMF procedure

Figure 6.2.5.2-1: Procedure for Local Handling of Edge Computing with I-SMF

- During the Registration procedure, the AMF receives the Local handling of Edge Computing allowed indication per DNN/S-NSNAI from the UDM in the step 14b of clause 4.2.2.2.2 of TS 23.502 [3].

2.	The AMF selects an I-SMF and sends an Local handling of Edge Computing allowed indication to the I-SMF as in clause 4.23.5.1 of TS 23.502 [3].

If the I-SMF receives the Local handling of Edge Computing allowed indication from AMF, the I-SMF:

-	selects a EASDF and obtains the EASDF IP address and EASDF DNS security information via interaction with EASDF; and

-	includes the EASDF IP address and (if exists) DNS security information of EASDF together with the Local handling of Edge Computing allowed indication to the SMF in the Nsmf_PDUSession_Create Request.

The SMF based on the received information (i.e. Local handling of Edge Computing allowed indication) skips the steps related to support Edge Computing (e.g. EASDF discovery and selection, EAS Deployment Information management, etc.) and:

-	includes Local handling of Edge Computing allowed indication during the SM Policy Association Establishment/Modification procedure and may receive Local Offloading Policy (i.e. IP range(s) and/or FQDN(s) allowed to be routed to the local part of DN) from the PCF as in clause 5.2.5.4.2 of TS 23.502 [3] and provides the Local Offloading Policy in the Nsmf_PDUSession_Create Response; and

-	includes the EASDF IP address as DNS server address and the DNS security information of EASDF to be sent to the UE via PCO; and

-	Forwards AF provided traffic influence information to the I-SMF if exists.

3.	The I-SMF invokes Neasdf_DNSContext_Update Request including UE IP address to complete the configuration of the context in the EASDF and configures the DNS handling rules using the Local Offloading Policy received from the SMF.

The I-SMF selects and configures the UL CL UPF and L-PSA UPF to forward DNS messages to the EASDF.

4. 	EAS Discovery procedure with EASDF is performed as described in steps 1-2 of clause 6.7.2.3 with following difference:

-	by replacing V-SMF and H-SMF with I-SMF and SMF, V-UPF and H-UPF with L-PSA UPF and PSA UPF, V-EASDF with EASDF, UL CL UPF in the VPLMN with UL CL UPF selected by the I-SMF and VPLMN Specific Offloading Information with Local Offloading Policy respectively.

-	For the DNS query with target FQDN not part of the FQDN authorized in the Local Offloading Policy, the EASDF may send the DNS query to the DNS server according to the DNS message handling rules provided by the I-SMF or to the default DNS server configured in the EASDF and may include address information as the EDNS Client Subnet option based on the instruction from I-SMF.

-	The UL CL/BP UPF may also be instructed by the I-SMF to send the DNS query to the DNS server via I-UPF and UPF by modifying the packet's destination IP address corresponding to the EASDF IP address to the DNS server address.

-	The EASDF may also be instructed by the I-SMF to construct and to send another DNS query with the same FQDN and the PSA UPF address information as the EDNS Client Subnet option to the default DNS server.

* * * * End of Changes * * * *