3GPP TSG-WG SA2 Meeting #164	S2-2408086

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Update of conclusion on KI#9

Document for:	Approval

Agenda Item:	19.3

Work Item / Release:	FS_XRM_Ph2 / Rel-19

Abstract: Remove the EN for the conclusion on KI#9 on exposure of available data rate.

1. Introduction

During last SA2#163 meeting, the interim agreement on the conclusion of KI#9 is reached while an EN remains to be resolved. This paper intends to add clarification to resolve the EN as following:

Editor's note:	This may be refined based on the potential feedback from RAN WG3.

Following the interim conclusion, the available data rate for a GBR QoS Flow could be requested by the AF for exposure. As instructed by the SMF, the NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes it to the AF directly or via the local NEF.

In S2-2406867/R3-243958, RAN3 already indicates that the available data rate reporting by the NG-RAN is feasible.

Besides, the local exposure of RAN-reported network information by the UPF has already been supported since R17, e.g., the packet latency, congestion information.

Therefore, it’s proposed to remove the EN directly.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First change * * * *

8.9	Conclusion for Key Issue #9

The following aspects are concluded for the normative work:

-	For GBR QoS Flow, exposure of available data rate:

-	Based on the AF requests, the PCF instructs SMF to enable the available data rate report and exposure for a GBR QoS Flow.

-	Following the instruction of SMF:

-	The NG-RAN reports the available data rate to the UPF via the GTP-U header of UL packets and UPF further exposes, based on mechanisms defined in clause 5.8.2.17 of TS 23.501 [2], the available data rate to the AF directly or via the local NEF.

NOTE:	It is recommended that the available data rate exposure is not reported very frequently.

# * * * * End of changes * * * *