3GPP TSG-SA2 Meeting #164	S2-2408093

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1151 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#4: Massive UE signaling strom mitigation and prevention |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on the conclusion of TR 23.700-84 Key Issue #4, a new Analytics ID is proposed to address signaling storms caused by massive UE signaling. This paper analyzes UE handover and paging signaling to mitigate signaling storms from a large number of UEs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In TS 23.288, under clause 6.7's UE-related analysis, we propose a new UE handover and paging signaling analysis ID aimed at preventing and mitigating signaling storms. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The analysis of handover and paging signaling in terms of UE mobility and connectivity will remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7, 6.7.1, 6.7.6 (new), 6.7.6.1 (new), 6.7.6.2 (new), 6.7.6.3 (new), 6.7.6.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.7	UE related analytics

6.7.1	General

This clause specifies the UE related analytics which can be provided by NWDAF:

-	UE mobility analytics;

-	UE communication analytics;

-	Expected UE behavioural parameters related network data analytics; and

-	Abnormal behaviour related network data analytics.

-    UE Handover and Paging Signaling Analytics.

The NWDAF service consumer may request for these analytics separately, or in a combined way. As an example, an NWDAF service consumer may learn from the NWDAF the expected UE behaviour parameters as defined in clause 4.15.6.3 of TS 23.502 [3], by requesting analytics for both UE mobility (see clause 6.7.2) and for UE communication (see clause 6.7.3).

Depending on local regulations, the NWDAF retrieves user consent for the UE with UDM prior to data collection as defined in clause 6.2.2.2 or clause 6.2.2.3. If user consent to collect data is not granted by the UE, the NWDAF rejects/cancels any analytics subscriptions to any of the UE related analytics with target for analytics set to the SUPI or GPSI of that UE. If the target for analytics is either an Internal or External Group Id or a list of SUPIs or "any UE", the NWDAF skips those SUPIs that do not grant user consent for the purpose of analytics or model training.

NOTE:	Possible uses of such analytics is for the AMF to learn about expected UE behaviour to derive appropriate MICO mode configuration, or for an AF to learn about expected UE behaviour to further provision 5GC with appropriate UE parameters.

END CHANGE

SECOND CHANGE

6.7.6	UE Handover and Paging Signaling Analytics

6.7.6.1	General

NWDAF can provide support by analyzing UE handover and paging signaling to offer statistics and predictions, which can assist in mitigating and preventing signaling storms caused by massive UEs.

The handover and paging signaling statistics and predictions can be used to forecast real-time trends in UE handovers and paging, helping to prevent and mitigate signaling storms in network slices and specific areas (e.g., AoI, TAI, etc.).

NWDAF can utilize this analysis in conjunction with NF load, network performance, and UE communication analysis. To support UE handover and paging signaling analysis, NWDAF can collect relevant information through OAM (via MDT), AMF, and SMF.

The NWDAF provides statistics and/or prediction analytics to consumer NFs.

The consumer of these analytics may indicate in the request:

- Analytics ID = "UE Handover and Paging Signaling";

- Target of Analytics Reporting: any UE (i.e all UEs).

- Analytics Filter Information optionally including:

  - Area of Interst;

  - List of NF Instance IDs

  - S-NSSAI;

  - DNN;

- Optional preferred level of accuracy of the analytics;

- An Analytics target period indicates the time period over which the statistics or predictions are requested;

6.7.6.2	Input Data

For UE signaling analytics, NWDAF may collect the information listed in Tables 6.7.6.2-1 and 6.7.6.2-2 for the relevant instances.

Table 6.7.6.2-1: Data Collected by NWDAF for Handover Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI. |
| Reference Signal Received Power (RSRP) | OAM | Cell signal strength: Handover is triggered when the signal strength of the cell connected to the UE falls below a defined threshold. (via MDT) |
| Reference Signal Received Quality (RSRQ) | OAM | Indicates signal quality, defined by the ratio of RSRP to RSSI. A low RSRQ value signifies poor signal quality, potentially triggering a handover. (via MDT) |
| Signal-to-Interference-plus-Noise Ratio (SINR) | OAM | Signal-to-Interference-plus-Noise Ratio: Beyond signal strength, signal quality is crucial. Handover is triggered when signal quality drops below a certain level. (via MDT) |
| Frequency band | OAM | Frequency Band (e.g., Indoor vs. Outdoor): The network uses various frequency bands, and as the UE moves, handover to a different band may be required. For instance, moving from indoors to outdoors may necessitate a handover to a different frequency band. (via MDT) |
| UE velocity | OAM | UE Speed (Higher speed increases handover likelihood): Fast-moving UEs may require more frequent handovers, necessitating optimal timing. Examples include UEs in fast-moving vehicles or trains. (via MDT) |
| Traking Area Identity (TAI) | OAM | A variable that identifies the tracking area where the UE is registered, including PLMN ID and TAC. |
| Cell(s) load infromation | OAM | When the current cell experiences high load and becomes overloaded, the UE is handed over to another cell to distribute the load. This handover optimizes the utilization of network resources. (via MDT) |
| Requests Handover | OAM | RRC connection reconfiguration |
| UE CM state | AMF | Relates to the UE's network connection status, indicating whether the UE is actively connected to the network. - CM-IDLE - CM-CONNECTED |
| S-NSSAI | AMF | Details related to the network slice the UE is associated with. |
| DNN | AMF | Information about the data network connected to the UE. |
| Timestamp | AMF | Records the time for tracking handover trends. |

Table 6.7.6.2-2: Data Collected by NWDAF for Paging Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI. |
| UE CM state | AMF | Relates to the UE's network connection status, indicating whether the UE is actively connected to the network. - CM-IDLE - CM-CONNECTED |
| Cell(s) load infromation | OAM | When the current cell experiences high load and becomes overloaded, the UE is handed over to another cell to distribute the load. This handover optimizes the utilization of network resources. (via MDT) |
| Traking Area Identity (TAI) | OAM | A variable that identifies the tracking area where the UE is registered. Paging messages are broadcast to all cells within the last known TAI of the UE. Includes PLMN ID and TAC. |
| Paging Occasion (PO) | OAM | A specific moment when a paging message is transmitted, synchronized with the DRX cycle. It represents the time slot during which the UE can receive paging messages. |
| Requests Paing | AMF/OAM | Paging Request |
| S-NSSAI | AMF | Details related to the network slice the UE is associated with. |
| DNN | AMF | Information about the data network connected to the UE. |
| Timestamp | AMF | Records the time to track and analyze paging trends. |
| Number of SMS, Call, PDU session Update requests | AMF | Indicates the events that trigger paging, such as Call, SMS, or PDU Session Update, as specified in TS 23.501 and TS 24.501. |

NOTE 1:

UE input data collection for a specific UE from OAM (via MDT) is detailed in clause 6.2.3.1.

6.7.6.3 Output Analytics

The output analytics for UE handover and paging signaling provided by NWDAF are defined in Tables 6.7.6.3-1, 6.7.6.3-2, and 6.7.6.3-3.

Table 6.7.6.3-1: UE Handover Singaling Statistics

| Information | Description |
| --- | --- |
| UE ID | SUPI |
| Handover signals (0..max) | Observed Handover Signal List |
| > Reference Signal Received Power (RSRP) | Signal Strength |
| > Reference Signal Received Quality (RSRQ) | Signal Quality |
| > Signal-to-Interference-plus-Noise Ratio (SINR) | Signal-to-Interference-plus-Noise Ratio |
| > Frequency band | Frequency Band |
| > UE velocity | UE Speeed |
| > Traking Area Identity | UE Registered Location |
| > Cell(s) load infromation | Current and Neighboring Cell Load Information |
| > UE CM state | UE Connection Status Information |
| >Timestamp | Timestamp for Handover Trend Analysis |

Table 6.7.6.3-2: UE Paging Singaling Statistics

| Information | Description |
| --- | --- |
| UE ID | SUPI |
| Paging signals (0..max) | Observed Paging Signal List |
| > UE CM state | UE Connection Status Information |
| > Cell(s) load infromation | Current and Neighboring Cell Load Information |
| > Traking Area Identity | UE Registered Location |
| > Paging Occasion | Paging Message Transmission Cycle |
| > Timestamp | Timestamp for Paging Trend Analysis |
| > Number of SMS, Call, PDU session Update requests | Paging Trigger Information Pattern |

Table 6.7.6.3-3: UE Handover Singaling Prediction

| Information | Description |
| --- | --- |
| UE ID | SUPI |
| Handover signals (0..max) | Predicted Handover Signal List |
| > Reference Signal Received Power (RSRP) | Signal Strength |
| > Reference Signal Received Quality (RSRQ) | Signal Quality |
| > Signal-to-Interference-plus-Noise Ratio (SINR) | Signal-to-Interference-plus-Noise Ratio |
| > Frequency band | Frequency Band |
| > UE velocity | UE Speeed |
| > Traking Area Identity | UE Registered Location |
| > Cell(s) load infromation | Current and Neighboring Cell Load Information |
| > UE CM state | UE Connection Status Information |
| >Timestamp | Timestamp for Handover Trend Analysis |
| >Confidence | Confidence of this prediction. |

Table 6.7.6.3-4: UE Paging Singaling Prediction

| Information | Description |
| --- | --- |
| UE ID | SUPI |
| Paging signals (0..max) | Predicted Paging Signal List |
| > UE CM state | UE Connection Status Information |
| > Cell(s) load infromation | Current and Neighboring Cell Load Information |
| > Traking Area Identity | UE Registered Location |
| > Paging Occasion | Paging Message Transmission Cycle |
| > Timestamp | Timestamp for Paging Trend Analysis |
| > Number of SMS, Call, PDU session Update requests | Paging Trigger Information Pattern |
| > Confidence | Confidence of this prediction. |

6.7.6.4	Procedures

The procedure in Figure 6.7.6.4.1-1 is used by the NF to retrieve trend analysis for UE handover and paging signals. It supports both one-time and continuous reporting of signal analysis.

Figure 6.7.6.4.1-1: Procedure for One-Time or Continuous Reporting of Analytics for UE Handover and Paging Signaling"

For one-time reporting:

1.	Request: The NF sends an Nnwdaf_AnalyticsInfo_Request to NWDAF, indicating a request for analytics related to UE signaling. The NF can request statistics, predictions, or both. The Analytics ID is set to "UE Handover and Paging Signaling," and the Target of Analytics Reporting is set to "any UE."

2-3. Authorization and Data Collection: If the request is authorized, NWDAF may request measurement information from OAM services by following the data collection procedure outlined in 6.2.3.2. NWDAF may obtain measurements by invoking management services as defined in TS 28.532 [6] and TS 28.550 [7].

4.	Analytics Derivation: The NWDAF derives the requested analytics.

5.	Response: The NWDAF provides the analytics related to UE signaling information to the NF.

For continuous reporting:

6.	Subscription Request: The NF sends an Nnwdaf_AnalyticsSubscription_Subscribe request to NWDAF, indicating a request for continuous analytics. The NF can request statistics, predictions, or both.

7-8. Subscription and Data Collection: The NWDAF subscribes to OAM services, following the data collection procedure outlined in 6.2.3.2, to obtain measurement information. NWDAF may invoke management services as defined in TS 28.532 [6] and TS 28.550 [7].

9.	Analytics Derivation: The NWDAF derives the requested analytics.

10.	Initial Response: The NWDAF provides the analytics related to UE signaling information to the NF.

11a. OAM Notification: The OAM notifies NWDAF with input data as defined in tables 6.7.6.2-1 and 6.7.6.2-2.

11b. AMF Notification: The AMF notifies NWDAF with input data as defined in tables 6.7.6.2-1 and 6.7.6.2-2.

12. New Analytics Derivation: The NWDAF derives new analytics based on the updated data.

13.	Notification: The NWDAF provides a notification with updated analytics related to UE signaling information to the NF.

END CHANGE