3GPP TSG-SA WG2 Meeting #164	S2-2408106

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	CATT

Title:	Update the Conclusion for KI#1

Document for:	Discussion/Approval

Agenda Item:	19.3.1

Work Item / Release:	FS_XRM Ph2 / Rel-19

Abstract of the contribution: this contribution proposes to update the conclusion for KI#1.

1.	Introduction

This contribution proposes to update the conclusion for KI#1.

If the multiplexing stream traffic can be mapped into different QoS Flows, the PDU Set information can be provided to the 5G RAN for each QoS Flow. Whether to define a new indication or to reuse existing indication for DL PDU Set information Marking can be decided in the normative stage. So it is proposed to change clause 8.1.1 and remove the EN.

The PSDB and PSER are used with GFBR to determine QoS Notification Control "GFBR cannot be guaranteed or can be guaranteed again". The QoS Notification to the AF just with Control "GFBR cannot be guaranteed or can be guaranteed again", does not include PSDB/PSER information, so the AF does not need to understand the PSDB/PSER. it is proposed such changes in clauses 8.1.2 and 8.1.3 and remove the ENs.

AF can also request to add the new PDU Set QoS requirements or remove the PDU Set QoS requirements for existing XR media flows, which the SMF/PCF to update the QoS profile and/or Alternative QoS Profiles to the PDU Set QoS parameters.

Change all interim agreements to conclusions for clause 8.1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-70.

# * * * * First Change * * * *

## 8.1	Conclusions for Key Issue #1

### 8.1.1	Conclusions for KI#1_other PDU Set handling

The following aspects are concluded for #1 for other PDU Set handling:

1.	According to the Protocol Description and/or additional Packet Filter as described in clause 8.4, the 5GC sends NG-RAN an indicator indicating the DL PDU Set Information Marking is supported. Based on the indicator , the NG RAN may indicate the 5GC to perform the PDU Set Information marking.

### 8.1.2	Conclusions for KI#1_QoS Notification without AQP

The following aspects are concluded for #1_PDU Set handling support or not indications in UL and/or DL and QoS notification:

1.	In order to support QoS Notification Control "GFBR cannot be guaranteed or can be guaranteed again" (see clause 5.7.2.4 in 23.501 [2]), for a GBR QoS flow with the QoS profile containing UL and/or DL PSDB and UL and/or DL PSER, instead of PDB and PER, the fulfilment or not of UL and/or DL PSDB or UL and/or DL PSER in the QoS profile are also used with the GFBR by the NG-RAN to determine QoS notification if the PDU Set QoS handling is applied in the corresponding direction..

### 8.1.3	Conclusions for KI#1_AQP

The following conclusion principles are agreed for key issue #1_AQP.

1.	The AF may provide Alternative Service requirements to 5GC which may contain the UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER).

2.	Based on the Alternative Service Requirements from AF, the PCF generates PCC rule with Alternative QoS parameters in a prioritized order, which can include UL and/or DL PDU Set QoS Parameters (i.e. UL PSDB, DL PSDB, UL PSER and/or DL PSER) and the corresponding reference to each Alternative QoS parameter Set.

3.	SMF provides one or more Alternative QoS Profile(s) with UL and/or DL PDU Set QoS parameters to NG-RAN. When the QoS profile includes UL and/or DL PDU Set QoS parameter, the Alternative QoS Profile shall include the corresponding PDU Set QoS parameters;.

4.	If QoS notification is determined as "GFBR cannot be guaranteed", the NG-RAN determines the AQP that can be used as a reference to the currently fulfilled QoS considering the priority order: If the PDU Set QoS handling is applied in the corresponding direction, the UL and/or DL PSDB and UL and/or DL PSER in the AQP are also used with the associated GFBR in the AQP by the NG RAN to determine the AQP that can be used as a reference to the currently fulfilled QoS.

5.	During PDU Session Establishment/Modification procedures, the SMF/PCF may update the QoS profile and/or Alternative QoS Profiles to include/exclude the PDU Set QoS parameters based on the PDU Set Based Handling Support Indication from the NG-RAN and or the new QoS requirements from the AF.

# * * * * End of change* * * *