3GPP TSG-SA WG2#164	S2-2408133

19-23 August 2024, Maastricht, Netherlands	(was S2-2407087)

Source:	Apple

Title:	Conclusion for KI#1.3

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.3.

# 1.	Introduction

This contribution proposes conclusion for KI#1.3 based on NMW discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.3

The following principles are concluded for the normative work:

-	The DualSteer PDU Session is a multi-access PDU Session that is pre-established on both 3GPP accesses.

-	The DualSteer Device establishes one leg of the DualSteer PDU session over one access network for and it establishes the second leg of the DualSteer PDU session over the second access network.

-	In case the DualSteer Device has two SUPIs, the DualSteer Device uses different SUPIs for different legs.

-	The Session Correlation Information for the two legs is stored in the UDM and provided to the H-SMF. When the PDU Session Establishment Request is received by the network for the second leg of the DualSteer PDU session, a Session Correlation Information ensures the same SMF is selected.

-	The DualSteer PDU session can be established even if only a single access is available. Once the second access becomes available, the DualSteer Device can initiate the PDU Session Establishment procedure to activate the UP resources for the second access.

-	A DualSteer Device can support simultaneous transmission to transfer different services via different access. A DualSteer Device that supports simultaneous transmission can use either of the SUPIs to establish the DualSteer PDU Session if both SUPIs are registered to 5GC.

-	Access Network resources only need to be set up for one leg of the DualSteer PDU Session at the time of PDU Session Establishment based on the DualSteer steering/switching policy.

-	The DualSteer PDU session is anchored in a common UPF (PGW-U) and managed by a common SMF (PGW-C).

-	The DualSteer PDU Session is applicable to specific DNNs/S-NSSAIs. The applicable DNNs/S-NSSAIs are controlled via URSP and SM subscription data.

-	A DualSteer Device that supports simultaneous transmission can establish the DualSteer PDU Session including different services/applications with different steering/switching conditions.

-	The DualSteer traffic is managed by a Steering Functionality in the UE for the uplink traffic and a Steering Functionality in the UPF for the downlink traffic.

Editor’s Note: Whether MPQUIC steering functionality is needed if the DualSteer Device supports simultaneous transmission needs more discussion / SoH. MPTCP steering functionality can be potentially de-prioritized.

-	For a DualSteer Device that is only capable for non-simultaneous transmission across all PDU sessions, only the Active-Standby steering mode is allowed and the active access is the same for all services.

-	For a DualSteer Device that supports simultaneous transmission, only the Active-Standby steering mode is allowed. This ensures only one access is used for a service.

-	The SMF manages the DualSteer PDU Session using a single PCF association that provides unified policies and single PFCP association/N4 rules towards UPF.

-	Steering of services for LBO sessions is not in the scope for DualSteer.

# >>>>END OF CHANGES<<<<