3GPP TSG-SA WG2#164	S2-2408134

19-23 August 2024, Maastricht, Netherlands	(was S2-2406827)

Source:	Apple

Title:	Conclusion for KI#1.4

Document for:	Discussion/Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: this contribution proposes conclusion for KI#1.4.

# 1.	Introduction

This contribution proposes conclusion for KI#1.4 based on NMW discussion report in S2-2408126 and S2-2406791.

# 2.	Proposal

It is proposed to agree the following changes vs. TR 23.700-54 v1.0.0:

# >>>>BEGINNING OF CHANGES <<<<

## 8.1	Conclusions for DualSteer

### 8.1.x 	Conclusions for KI#1.4

The following principles are concluded for the normative work:

-	The DualSteer Device uses URSP rules to link the service to the DualSteer PDU session.

-	The same URSP rules are provisioned for the two SUPIs of the DualSteer Device.

-	The Access Type preference in Route Selection Descriptor is extended to include a DualSteer indication.

-	The DualSteer UE rules are used to enforce DualSteer policies in the uplink direction and provided by the SMF to the UE.

-	The DualSteer UPF rules are used to enforce DualSteer policies in the downlink direction and provided by the SMF to the UPF.

-	The DualSteer UE Rules are provided during the DualSteer PDU Session establishment and defined to control the steering and switching of services in the DualSteer Device.

-	For a DualSteer Device that supports simultaneous transmission, the DualSteer UE rules identify the UE/SUPI for steering and switching decision by using an Access Descriptor in the rules.

-	The DualSteer UE and UPF Rules are defined with flow-level granularity to enable more efficient and flexible control.

-	The PCF can take into account UE's PLMN, RAT, or Access Type to generate PCC rules that is used by SMF to generate the DualSteer UE rules and the DualSteer UPF rules.

-	The PCF, based on PCRT, can update the PCC rules and that will trigger the SMF to update the DualSteer UE rules and the DualSteer UPF rules.

# >>>>END OF CHANGES<<<<