SA WG2 Meeting #164	S2-2408135

19-23 August 2024, Maastricht, Netherlands	(Revision of SP-240994)

# Source:	Apple (Primary Rapporteur)

# Title:	Updated WID on Multi-Access (DualSteer and ATSSS_Ph4)

# Document for:	Approval

Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	Multi-Access (DualSteer and ATSSS_Ph4)

Acronym:	MASSS

Unique identifier:	1040034

Potential target Release:	Rel-19

# Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  | X |
| Don’t know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_MASSS | SA2 | 1020070 | Study on Multi-Access (DualSteer and ATSSS_Ph4) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 940070 | Access Traffic Steering, Switch and Splitting support in the 5G system architecture; Phase 3 | Rel-18 Work Item |
| 960018 | Study on upper layer traffic steering, switching and split over dual 3GPP access | Stage 1 study for DualSteer in Rel-19 |
| 1020031 | Upper layer traffic steering and switching over dual 3GPP access | Stage 1 work item for DualSteer in Rel-19 |

# 3	Justification

1)	In Rel-18, a high-layer steering functionality “MPQUIC steering functionality using UDP proxying over HTTP” was defined that enables steering, switching, and splitting UDP traffic based on IETF protocols. It would be beneficial to specify how to enable the MPQUIC steering functionality to also steer, switch, and split non-UDP traffic (IP & Ethernet traffic).

2)	As identified in the SA1 study and service requirements on DualSteer (see TR 22.841 and TS 22.261), it is beneficial for a DualSteer Device to apply traffic steering and/or switching between two 3GPP access networks connected to the same or different PLMN networks. Use cases cover examples of diverse combinations of 3GPP access networks using the same or different RATs, including terrestrial NR plus NR, or NR plus E-UTRA (e.g. using a combined EPC and 5GC), a mix of terrestrial and non-terrestrial NR, as well as dual non-terrestrial NR access (using same or different NTN orbits, e.g., GEO/MEO/LEO).

3)	The Rel-16 to Rel-18 Access Traffic Steering, Switching and Splitting feature requires that MA PDU Sessions require integrated trusted or untrusted non-3GPP accesses. This means that to enable ATSSS either a TNGF (Trusted Non-3GPP Gateway Function) or an N3IWF (Non-3GPP InterWorking Function) is deployed in the PLMN. At the same time, many network deployments do not have such nodes and it is therefore beneficial to specify how to support a limited set of access traffic aggregation and steering features applicable to non-integrated non-3GPP access not based on TNGF/N3IWF.

# 4	Objective

The objective of this work item is to specify the architecture enhancements, functionalities and procedures to support MASSS features based on conclusions of TR 23.700-54 (clause 8).

Specifically, the detailed objectives include:

1)	New functionalities and procedures to support MPQUIC steering functionalities in order to steer, switch, and split non-UDP traffic.

2)	Enhancements of subscription aspects to support DualSteer.

3)	Registration and mobility management enhancements to support DualSteer.

4)	Session management enhancements to support DualSteer.

5) 	Definition of policies to support DualSteer traffic steering and DualSteer traffic switching.

6) 	URSP extensions to support DualSteer.

7)	Enhancements to functionalities and procedures to support Simplified ATSSS architecture over non-3GPP access.

# 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.501 | Description of new MPQUIC steering functionalities to support steering, switching and splitting of IP and Ethernet traffic. Description of DualSteer functionalities. System architecture for Simplified ATSSS. | TSG#106 (Dec. 2024) |  |
| TS 23.502 | Procedures to support steering, switching and splitting of IP and Ethernet traffic using MPQUIC steering functionality. Procedures to support DualSteer functionalities. Procedures to support Simplified ATSSS functionalities. | TSG#106 (Dec. 2024) |  |
| TS 23.503 | Policy control to support steering, switching and splitting of IP and Ethernet traffic using MPQUIC steering functionality. Policy control to support DualSteer functionalities. Policy control to support Simplified ATSSS functionalities. | TSG#106 (Dec. 2024) |  |

# 6	Work item Rapporteur(s)

Primary Rapporteur: Krisztian Kiss (Apple), krisztian@apple.com

Secondary Rapporteur: Zhuoyi Chen (China Telecom), chenzy34@chinatelecom.cn

(in charge of producing reports to the WG plenary on progress)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

N/A

# 9	Supporting Individual Members

Supporting IM name

Apple

BT

CableLabs

CATT

Charter Communications, Inc

China Telecom

Comcast

Ericsson

ETRI

InterDigital

Lenovo

LG Electronics

NEC

Nokia

SHARP

ZTE