SA WG2 Meeting #164	S2-2408136

# Maastricht, Netherland, August 19th – 23rd, 2024		(revision of S2-240)

Source: 	Nokia, MediaTek Inc., Deutsche Telekom

Title: 	Conclusions for DualSteer

Document for: 	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes conclusions for all the Key Issues related to DualSteer.

# 1	Discussion

This paper proposes conclusions on KI#1.1 through KI#1.4

Observation 1:

SA1 has defined requirement for two types DualSteer devices. One that does not support simultaneous transmission over two 3GPP access and "can be a single UE" and another that supports simultaneous transmission over two 3GPP access and "shall have 2 UEs". In SA2 we have not yet agreed on the term "DualSteer" and what does a "DualSteer device with 2 UEs" mean from SA2 pov. The SID assumes no RAN impact, however based on the definition of DualSteer device with 2 UEs, there are potential RAN impacts e.g:

-	If both the UEs of the DualSteer device are accessing same frequency band (e.g. FR1) there may be a need for coordination in RAN for UpLink transmission power control in the UE.

-	There is potential impact on RAN scheduler also to support simultaneous transmission, specifically if the same RAN is serving the two PLMNs of the 2 UEs or e.g. if Dynamic spectrum sharing is used and one UE is registered in LTE and the other in NR.

As per SA1 requirement all traffic of an application is always sent via one access (i.e. there is no splitting or duplication of traffic needed on two 3GPP access). As redundant steering mode and splitting are not in scope there is NO real scenario where simultaneous transmission of application traffic via both 3GPP access is required.

Proposal 1:

As the "DualSteer device with 2 UEs" cannot be supported without further studying the RAN impacts and considering that we have already exhausted the study TUs for the SID, it is proposed to support only DualSteer device with a single UE in Rel-19. It is proposed to conclude as below:

All applications traffic from/to the DualSteer device are always sent/received via a single 3GPP access. When switching is applied, all applications traffic of all PDU Sessions of the DualSteer device, that are allowed for DualSteer switching, are switched from the first 3GPP access to the second 3GPP access.

Observation 2:

If only DualSteer device with a single UE is supported, i.e. all application traffic from/to the DualSteer device are always sent/received via a single 3GPP access, this means only Active/Standby mode of operation can be supported.

Proposal 2:

It is proposed to conclude as below:

Only Active-Standby mode of operation is supported in Rel-19 for DualSteer Device

Observation 3:

For steering, the DualSteer device needs to determine which UE/SUPI is registered to active 3GPP access and which SUPI/UE is registered to the standby 3GPP access to be able to send Uplink traffic via the UE/SUPI registered to the active 3GPP access. All traffic of all application will be sent/received via PDU Sessions of a single UE/SUPI. Therefore, the steering rules are not required to be provided at PDU Session level. The Steering rules can be provided at UE policy level.

For switching the DualSteer device shall switch all the traffic from all the PDU Session(s) of a UE/SUPI in the active 3GPP access to the corresponding linked PDU Session(s) of the UE/SUPI in the standby 3GPP access. Again, the switching rules also does not need to be provided at PDU Session level. The switching rules can be provided at UE policy level.

Proposal 3:

It is proposed to conclude as below:

The Steering and switching rules are provided as a new UE policy to the DualSteer device.

Observation 4 (PDU Sessions from the two UE/SUPI and switching):

For switching traffic from the active UE/SUPI to the standby UE/SUPI, if the PDU Session(s) on the standby UE/SUPI is established after switching decision is made, it may cause potential switching issues:

-	First, as the UE/SUPI has to establish all the corresponding PDU Sessions, the time required for switching the traffic will be longer.

-	Secondly, during the PDU Session establishment through the standby UE/SUPI, there can be potential scenarios where the same SMF may not be possible to select e.g. due to overload scenarios etc. In such cases it may lead to further switching delay or even switching failure.

Instead, if the PDU Sessions are established from both the UE/SUPI at the beginning and they are already linked by the network (i.e. same SMF/UPF is selected, and same IP address is assigned for PDU Sessions from both the SUPIs), the switching experience will be faster and also avoids any corner scenarios where the switching may fail.

Since simultaneous transmission over two 3GPP access is not supported, the UP connections of PDU Session(s) from the standby UE/SUPI can be deactivated when the traffic is going via the PDU Session(s) from the active UE/SUPI. When switching is required, the DualSteer device can activate the UP connections of the PDU Session(s) from the standby UE/SUPI and move all the traffic from the active UE/SUPI to the standby UE/SUPI. The UP connections of the PDU Session(s) on the previously active UE/SUPI can then be deactivated.

Proposal 4:

It is proposed to conclude as below:

When an application traffic is detected, if DualSteer switching is required then the DualSteer device establishes two separate PDU Sessions through the two UE/SUPI. The two PDU Sessions are served by the same SMF, UPF, PCF and a single IP address are assigned for two linked PDU Sessions from the two UE/SUPI.

Based on the Steering rules, the application traffic is sent/received only through the 3GPP access of one SUPI i.e. the UP connection of the PDU Session is Active in only one SUPI.

Based on the switching rules, when the DualSteer device decides to switch all traffic from all PDU Sessions that are subject to switching, the second SUPI activates the User Plane connection for the corresponding PDU Session(s) using UE Triggered Service Request procedure and then the DualSteer devices moves all the applications traffic from the first SUPI to the second SUPI.

Proposal 5 (Registration of the two UE/SUPI):

It is proposed to conclude as below:

It is proposed that the DualSteer device registers the two UE/SUPI onto different PLMN and RAT combination. Any impact on PLMN selection for the registration of the second SUPI will be specified by CT1 based on further PLMN selection requirements identified by SA1.

Proposal 6 (Subscription Data changes):

It is proposed that there is a subscription level association between the two SUPIs of a DualSteer device and it is maintained in the UDM. The Access and Mobility subscription data in enhanced to contain:

-	an indication that the SUPI is part of DualSteer;

-	the linked SUPI information;

-	optionally DNN, S-NSSAI that are allowed for DualSteer switching;

Proposal 7 (Same SMF selection for PDU Sessions from the two UE/SUPI):

It is proposed that the SMF Selection subscription data is enhanced to contains information of the SMF(s) serving the PDU Session(s) of the linked SUPI and this information is used by the AMF to select the same SMF to serve the PDU Session(s) from the two UE/SUPI that require DualSteer switching.

Proposal 8 (Steering and Switching rules):

It is proposed to conclude as below:

The DualSteer steering and switching rules are provided at registration by the PCF as part of a new UE policy.

The steering rules contain the information indicating the active registered 3GPP access for routing the detected application traffic.

The switching rules contains conditions when traffic can be switched from the current active 3GPP access of one SUPI to the 3GPP access of the other SUPI. The switching conditions may contain below information:

-	When the radio conditions of the current active 3GPP access falls below a threshold, switch to the 3GPP access of the standby SUPI.

-	When the current active 3GPP access is not available, switch to the 3GPP access of the standby SUPI.

Based on the above proposals, the end-to-end call flow is depicted in the below figure:

# 2 Proposal

Based on the above observation it is proposed to update TR 23.700-54 as follows:

* * * * Start of Changes * * * *

## 8.1	Conclusions for DualSteer

### 8.1.x1	 Conclusions for KI#1.1

Below principle are agreed for KI#1.1:

1.	There is a subscription level association between the two SUPIs of a DualSteer device and it is maintained in the UDM.

2.	The Access and Mobility subscription data is enhanced to contain:

-	an indication that the SUPI is part of DualSteer;

-	the linked SUPI information;

-	optionally DNN, S-NSSAI that are allowed for DualSteer switching;

4.	The SMF Selection subscription data is enhanced to contains information of the H-SMF(s) serving the PDU Session(s) of the linked SUPI.

3.	The SM subscription is enhanced to contain the Linked SUPI information and for each DNN, S-NSSAI an indication whether DualSteer switching is allowed.

### 8.1.x2	 Conclusions for KI#1.2

Below principle are agreed for KI#1.2:

1.	DualSteer device registers the two SUPIs onto a different PLMN and RAT combination.

NOTE:	Any impact on PLMN selection for the registration of the second SUPI will be specified by CT1 based on further PLMN selection requirements to be specified by SA1.

2.	Each UE/SUPI includes in the Registration request an indication that the UE/SUPI is used with a DualSteer device.

3.	The UE indication of usage with DualSteer is forwarded by the AMF to the UDM, if the serving PLMN supports DualSteer.

4.	If UE indication of usage with DualSteer is received by the UDM and the SUPI has DualSteer subscription then the UDM also provides the new Access and Mobility Subscription data to the AMF.

5.	The UDM provides to the AMF, the reference to the H-SMF(s) serving the PDU Session(s) of the linked SUPI of a DualSteer device as part of the SMF selection subscription data of this SUPI.

6.	If the serving PLMN supports DualSteer feature then the AMF sends a DualSteer support indication to the UE in Registration Accept.

### 8.1.x3	 Conclusions for KI#1.3

Below principle are agreed for KI#1.3:

1.	All applications traffic from/to the DualSteer device are always sent/received via a single 3GPP access. When switching is applied, all applications traffic of all PDU Sessions of the DualSteer device, that are allowed for DualSteer switching, are switched from the first 3GPP access to the second 3GPP access.

2.	The PDU Sessions applicable for DualSteer switching are always anchored at the HPLMN SMF/UPF i.e. for Roaming scenarios PDU Sessions applicable for DualSteer switching are always Home Routed.

3.	When an application traffic is detected, if DualSteer switching is required then the DualSteer device establishes two separate PDU Sessions through the two SUPIs. The PDU Session Establishment request contains a DualSteer required indication. The PDU Session ID of the first PDU Session is also included in the corresponding PDU Session Establishment request from the second SUPI.

4.	If at the time of PDU Session establishment only one SUPI is registered, then the DualSteer device establishes PDU session with the registered SUPI. The DualSteer device establishes the linked PDU Session from the second SUPI whenever the second SUPI has successfully registered to a 3GPP access network.

5.	When the PDU Session Establishment request for the second SUPI is received by the AMF, the AMF uses the UE provided information (i.e. DualSteer required indication and the corresponding PDU Session ID) and the new SMF selection subscription data in clause 8.1.x1 to select the same H-SMF that is serving the corresponding linked PDU Session for the first SUPI.

6.	H-SMF uses the new SM Subscription data in clause 8.1.x1 to control whether DualSteer switching is allowed for a PDU Session and if switching is allowed, based on the PDU Session ID received in PDU Session establishment request from the second SUPI and the SM subscription data, the H-SMF selects the same PCF and UPF to serve the two linked PDU Sessions from the two SUPIs of the DualSteer device.

7.	The H-SMF/UPF allocates a single IP address for the two linked PDU Sessions from the two SUPIs.

8.	Whether the H-SMF maintains a single or two PFCP session(s) and a single or two SM Policy Association(s) respectively with the UPF and the PCF selected for serving the two linked PDU Sessions from the two SUPIs of a DualSteer device will be determined during normative specification work.

9.	Based on the Steering rules, the application traffic is sent/received only through the 3GPP access of one SUPI i.e. the UP connection of the PDU Session is Active in only one SUPI. .

10.	Based on the switching rules, when the DualSteer device decides to switch, all traffic from all PDU Sessions that are subject to switching, the second SUPI activates the User Plane connection for the corresponding PDU Session(s) using UE Triggered Service Request procedure and then the DualSteer devices moves all the applications traffic from the first SUPI to the second SUPI.

### 8.1.x4	 Conclusions for KI#1.4

Below principle are agreed for KI#1.4:

1.	DualSteer steering and switching rules are provided at registration by the PCF as part of a new UE policy.

2.	The steering rules contain the order of preference among the registered 3GPP access for routing the detected application traffic.

3.	The switching rules contains conditions when traffic can be switched from the current active 3GPP access of one SUPI to the 3GPP access of the other SUPI. The switching conditions may contain below information:

-	When the radio conditions of the current active 3GPP access falls below a threshold, switch to the 3GPP access of the standby SUPI.

-	When the current active 3GPP access is not available, switch to the 3GPP access of the standby SUPI.

* * * * End of Changes * * * *