3GPP TSG-SA2 Meeting #164	S2-2408161

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5539 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of UE-Satellite-UE feature |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of UE-Sat-UE feature has 5GS impacts. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE-Sat-UE architecure is described, Feature description added, mobility aspects are explained. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing UE-Sat-UE feature in 5GS |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.4.X	Support for UE-Satellite-UE communication

#### 5.4.X.1	General

This clause describes the support of UE-Satellite-UE communication using NR satellite access over 5GS.

UE-satellite-UE communication refers to the communication between UEs under the coverage of one or more serving satellites without the user plane traffic going through the ground network.

It is assumed that a link to the ground from the serving satellite to a ground gateway, is always available. The UE involved in IMS communication may be served by same or different satellite and being served by same or different SMF, UPF, PCF and/or IMS core. The Figure 5.4.X.2.1 & 5.4.X.2.2 depicts a UE-Satellite-UE architecture where 2 UEs involved in UE-Satellite-UE communication are being served by different satellite, RANs, SMFs, UPFs and IMS.

NOTE 1:	In this release, only IMS voice/video services are considered for UE-Satellite-UE communication.

NOTE 2:	In this release, only non-roaming support is considered. i.e. UEs involved in UE-Sat-UE communication can only be served in their respective H-PLMN.

#### 5.4.X.2	Architecture for UE-Satellite-UE communication

Figure 5.4.X.2-1: Architecture for UE-Satellite-UE communication via IMS capability on-board satellite

Figure 5.4.X.2-2: Architecture for UE-Satellite-UE communication without IMS capability on-board satellite

The IMS impacts for UE-Satellite-UE communication is specified in Annex X of TS 23.228 [15].

The following principles shall apply for the support of UE-Satellite-UE communication:

-	For the IMS-related PDU Session, UE shall get an IP address from a PSA UPF located on the ground. This is to make sure the IP address is not changed when there is a change of satellite.

-	UE shall register onto IMS (application level) with this IP address, as per clause 5.2 in TS 23.228 [58]. This is to make sure UE keep their IP address constant and doesn't need to reregister when there is a change of serving satellite.

-	Upon IMS application-level registration, the P-CSCF may subscribe to the PCF for UE connectivity via NR satellite access. P-CSCF shall get notified on the access network information such as satellite ID in which the UE is being currently served. This notification is also applicable for change of satellite.

-	An ISL IP network using ISL links is used for UE-Sat-UE user plane communication where IMS capability onboard satellite can route U-plane packets to other user's IMS capability on-board the satellite.

-	An ISL IP network using ISL links is used for UE-Sat-UE user plane communication where UPF (I-UPF) onboard satellite can route U-plane packets to other user's satellite's UPF (I-UPF). PtP tunnelling can be applicable for I-UPF to I-UPF packet forwarding where the media flows with inner source/destination IP address of both UE(s) are sent in an IP over IP tunnel where the outer source/destination IP addresses correspond to the I-UPF(s) serving each of the UE(s). It is assumed that inter I-UPF in satellite data traffic use a dedicated PtP tunnel over ISL IP network.

NOTE 1: It is assumed that the ISL(s) can be set up within the same satellite constellation or across different constellations depending on the satellite operator's deployments (SLA). The set of ISL(s) build up an IP routing network which is outside of the scope of 3GPP.

NOTE 2: In case of heterogenous deployment, i.e. involved satellites can have IMS capability and other without IMS capability, then the IMS voice traffic needs to fallback to without IMS capability mode or IMS voice routing via ground network.

-	During the Authorize QoS session and resource reservation in IMS described in clause 5.4.7 in TS 23.228 [], the PCF shall indicate to SMF a policy indicating the need for I-UPF insertion on-board the satellite.

-	The PCF shall report the success of QoS authorization/resource reservation and outer IP header information once SMF confirms the PDU resource allocation in UPF/I-UPF.

#### 5.4.X.3	Support for mobility in UE-Satellite-UE communication

This clause describes the principles applicable for change of serving satellite for UE involved in UE-Satellite-UE communication. In specific, this clause describes the impacts due to change of satellite during an ongoing UE-Satellite-UE communication.

The IMS impacts for support of mobility in UE-Satellite-UE communication is specified in Annex X of TS 23.228 [15].

Figure 5.4.X.3-1: Example of mobility scenario with IMS capability on-board

Figure 5.4.X.3-2: Example of mobility scenario with IMS capability on-board

In the above example in Figure 5.4.X.3.1 and 5.4.X.3.2, the originating side UE is changing the satellite due to movement of satellite. After the Handover procedure is complete, i.e. UE is moved from source RAN in source satellite to target RAN in target satellite.

If the source and target satellite have ISL connectivity the session continuity upon UL CL relocation can be applied. The uplink traffic destined to Source on-board IMS capability can go through Target (R)AN, Target UL CL and Source UL CL and then to Source on-board IMS capability or towards N6. The UL-CL UPF relocation is applied as per clause 4.3.5.7 in TS 23.502 [3]. This indirect tunnelling shall exist till the target on-board IMS capability is not removed from the user plane path and target UL CL configuration is not complete.

# * * * * End of change* * * *