SA WG2 Meeting #164	S2-2408166

19 Ausgust – 23 August, 2024, Jeju, Korea

Source:	Samsung

Title:	Update Conclusions for KI#1 for UIA_ARC

Document for:	Approval

Agenda Item:	19.8.1

Work Item / Release:	FS_UIA_ARC/ Rel-19

Abstract of the contribution: This paper proposes a Key Issue #1 conclusion update and just describes the differences with the NOTED S2-2407239

# 1.	 Discussion

This PCR proposes to update to KI#1 conclusions for UIA_ARC

- As per the use cases described in the TR 22.904 user identity feature can also be supported by the operator without supporting “multiple users” linked to a UE.

  - A single UE is linked to only a single user identifier, and the associated human user avails services according to the UE subscription (i.e. no user specific service settings in the UIP).

  - The UIP does not need to store any QoS specific settings and any of the network behaviour specified in the clause 8.1.2 (i.e. AMF providing user identifier to SMF, SMF providing it to PCF) is not used by the operator even though operator supports “user identifiers”

  - User verification results can be used by the operator/AF for various use cases, e.g. to verify whether the user is active at a particular UE (thereby also providing the trust that the user is authenticated and is in possession of the particular UE (or SIM Card)).

  - the 4G-5G interworking won’t have any impacts for this scenario

- Now even if the “multiple users” are used for a single UE, that UE can be assumed to be a shared UE which is not used for private purposes and any services for a “subscription” like calls/SMS are shared for all the users

  - Examples are a shared tablet of a family, a UE in the car, or even different service profiles used by the same human.

  - If the network operator wants to provide QoS differentiation/specific settings to these different users, then network may support the conclusions as described in clause 8.1.2

    - Otherwise, if operator does not want to support this, the information of which user is active at a subscription, or whether a user is active at a subscription can be used for the purposes as described for the single-user scenario.

  - For 5G to 4G handover, PDU session/QoS flows which are already established can be just transferred to the EPC, and any new PDN CONNECTION will be established based on the

    - The UE behaviour when it moves to EPC or even when UE is not even registered to the network is completely based on implementation (more specifically the UI settings of the UE)

      - E.g. UE can display to the user that UE is out of 5G coverage and cannot avail specific QoS settings;

The NOTED conclusions for the KI#1 (S2-2407239) are clarified based on the above discussion.

# 2. Proposals

It is proposed to adopt the following text in TR.23.700-32 v0.3.0.

*** Start of Changes ***

## 8.1	Key Issue #1: Identifying the Human User of a Subscription

### 8.1.1	Conclusion related to User Identifier and the User Identity Profile

The following bullets are conclusion principles for KI#1:

The User Identifier format is NAI.

-	The User Identifier can be operator assigned or third party assigned.

-	The User Identity Profile contains:

-	one User Identifier;

-	a list of linked subscription(s);

-	optionally, DNN/S-NSSAI combinations used for URSP Rules

-	optionally, QoS Information (e.g. default 5QI and ARP per DNN/S-NSSAI);

NOTE: 	Subscriber/subscription information will not be moved into a user profile and information from the user profile should not be used to override information in a subscription. For example, the slices and DNNs that are available to the UE do not change based on the user of the UE.

-	The UDM can store which User Identifier is active with a UE Subscription.

-	The User Identity Profile is stored in the UDR and the user identifier can be used as a data key.

-	The User Identity Profile can be managed by an operator. Managing a User Identity Profile includes creating, modifying, and deleting a User Identity Profile.

-	OAM procedures can also be used to manage the User Identity Profile.

-	NEF APIs can be used by an authorized AF (i.e. operator owned, or 3rd party owned) to manage the User Identity Profile.

-	The User Identifier is unique within PLMN and can be linked/unlinked with a 5GS subscription identified by a SUPI.

-	An authorized AF can use an NEF API to request that a user identifier be linked/unlinked with a subscription.

-	OAM procedures can be used to link/unlink a user identifier with a subscription.

-	For each user identifier, there is only one AF that is authorized to request that the user identifier be linked to a subscription.

-	For each subscription, there is only one AF that is authorized to request that a user identifier be linked to the subscription.

NOTE:	Where user credentials are stored can be decided by SA WG3.

-	The UE subscription data is extended to include:

-  	a list of linked User Identifier(s) (possibly only one user identifier may be linked to a subscription)

The user identifier is an optional feature in 5GS.

### 8.1.2	Conclusions Related to the User Identifier’s Influence on QoS

The following bullets are conclusion principles for KI#1 for the multi-user scenario and when the operator optionally wants to support QoS influence based on the active user identifier for a UE:

- When a user becomes active, the AMF can send the active user identifier to the SMF(s) that serve the UE’s PDU Session(s) and the SMF sends the user identifier to the PCF.

- During PDU Session Establishment, the AMF can send the active user identifier to the SMF and the SMF sends the user identifier to the PCF.

- PCC Rules may be adjusted/updated when a user becomes active with a subscription.

- PCC Rules may be updated by the PCF based on information received from the User Identity Profile.

- User Identities can be considered by the PCF when generating URSP Rules.

When the UE moves from 5G to EPC, UE’s existing PDU Sessions/QoS flows may be transferred to EPC using existing mechanisms. When UE creates a new PDU Session (PDN connection) while connected to EPC, parameters are provided based on the UE subscription data. When the UE returns to 5G, the UE may initiate the procedure to make the user become active again. Information from the User Profile is not sent to the EPC.

### 8.1.3	Conclusions Related to Activation of a User

The following bullets are conclusion principles for KI#1:

- NAS signalling support initiating the (re) authentication, authorization and activation procedure by sending/receiving the user identity information

NOTE 1:	The details of the Authentication mechanisms are in the remit of SA WG3.

A user identifier can only become active when the UE is registered in 5GS.

### 8.1.4	Conclusions Related to SMS over NAS

There will be no impacts to SMS over NAS in this release.

NOTE 1: 	Depending on input from SA WG1 or SA WG3, SMS over NAS may be disabled or enabled when a user becomes active or inactive.

*** End of Changes ***