3GPP TSG-WG SA2 Meeting #164	S2-2408178

# Maastricht, NL, 19th Aug – 23rd Aug, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.316 | CR | 2133 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of N3GPP device behind a 5G-RG |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | To add the support of Non-3GPP device connecting behind a 5G-RG as concluded in the TR 23.700-32 KI#4. This CR adds the support of identifable non-3GPP devices connecting behind a 5G-RG |  |  |  |  |  |  |  |  |
| Summary of change: |  | The support of identifiable Non-3GPP device connecting behind the UE are defined in TS 23.501, TS 23.502 and TS 23.503. This CR propose to add the difference to support the identifiable Non-3GPP device connecting behind a 5G-RG. The following new specifications are added: - definition of support of QoS differentiation for identifiable Non-3GP device in clause 4.5.3 - how to identify the Identifiable non-3GPP Devices Connecting behind a 5G-RG - Definition of the support of individual Identifiable non-3GPP devices behind a 5G-RG. 4. Definition of the usage of device identifier in the URSP rule. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Complete the UIA specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.x (new), 4.5.3 , 4.10e (new), 9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.5.3	Differentiated QoS for devices behind 5G-RG

When the 5G-RG does not support the identification of non-3GPP Device or does not apply QoS differentiation based on Non-3GPP identity, during PDU session establishment and PDU session modification, if the SMF provides to the 5G-RG QoS flow descriptions, the SMF may additionally signal Non-3GPP QoS Assistance Information (N3QAI) for each QoS flow to the 5G-RG. Based on the N3QAI together with QoS rule information, the 5G-RG may reserve resources in the non-3GPP network behind the 5G-RG (e.g. home LAN network). N3QAI consists of the following QoS information: QoS characteristics, GFBR/MFBR, Maximum Packet Loss Rate, ARP and Periodicity (if available at the SMF).

NOTE 1:	How 5G-RG uses the Non-3GPP QoS Assistance Information to enforce QoS in the non-3GPP network is outside the scope of 3GPP.

NOTE 2:	Transferring information like Periodicity to the 5G-RG is not meant to support TSC/TSN like flows but to support consumer real time applications like XR (extended Reality, etc.).

When the 5G-RG supports the QoS differentiation for Identifiable Non-3GPP Devices, the QoS differention defined in TS 23.501 [nn] clause 5.x.4 applies with 5G-RG taking the role of the UE.

# * * * * Second change * * * *

4.7.x	Identifiable Non-3GPP Device Identifier

The Identifiable Non-3GPP Device Indentifier (IN3DI) is used to classify in the 3GPP system an Identifiable Non-3GPP (IN3) device connecting behind a 5G-RG as specificed in TS 23.501 [nn].

# * * * * Third change * * * *

4.10e	Identifiable Non-3GPP devices accessing the 5GC from behind a 5G-RG

TS 23.501 [nn] clause 5.x applies to IN3 device connected behind a 5G-RG with the 5G-RG taking the role of the UE.

Unlike AUN3 devices, the IN3 devices is not authenticated by 5GC but is instead associated to the the subscription of the 5G-RG and uses the 5G-RG subscription to access the 5GC. The 5G-RG needs not support the NAS contextfor each IN3 device. Unlike NAUN3 device the IN3 device is uniquely identified by 5GC and associated to the 5G-RG subscription, therefore the 5GC may provide policy specific for each IN3 devices connected behind the same 5G-RG even if they are connected to the same LAN interface of the 5G-RG.

Figure 4.10d-1 shows an example of the overall architecture.

Figure 4.10e-1: Example scenario for identifiable Non-3GPP devices accessing 5GC from behind the same 5G-RG subscription

# * * * * Fourth change * * * *

9.5.2	UE access selection and PDU Session selection related policy information

9.5.2.1	5G-RG

This clause specifies the delta related to UE policy distribution defined in TS 23.503 [4] clause 6.1.2.2 and related to URSP defined in TS 23.503 [4] clause 6.6. for 5G-RG.

Only URSP policies are applicable for a 5G -RG.

NOTE:	For example, ANDSP policies, ProSe Layer-3 UE-to-Network Relay Offload Policy are not applicable to 5G-RG,

If the PCF provides a URSP policy to the 5G-RG, the PCF should only provide the following URSP policy information:

-	Rule Precedence.

-	Traffic descriptor/Application descriptors, IP descriptors, Domain descriptors, Non-IP descriptors, DNN, Connection Capabilities.

-	Route Selection Descriptor Precedence.

-	Route selection components/ SSC Mode Selection, Network Slice Selection, DNN Selection, PDU Session Type Selection.

-	Route Selection Validation Criteria/ Time Window.

The 5G-RG shall ignore any of the parameters and policies not listed above, if received from the 5GC.

The 5G-RG shall use the URSP policy as specified in TS 23.503 [4], for example for the association of application and PDU session, slices, etc.

The URSP indicates for the application of Auto-Configuration Server (ACS) which PDU session type, NSSAI and/or DNN is to be used. The 5G-RG establishes the connectivity to the management entity (e.g. ACS) via user plane connection on a PDU session according to the URSP.

UE Policy procedures defined in clause 6.1.2.2 of TS 23.503 [4] are applicable as follows:

-	Roaming is not applicable to W-5GAN access in this release of specification.

-	USRP rule enforcement reporting is not defined for 5G-RG.

In the case of an Authenticable Non-3GPP (AUN3) device behind a 5G-RG only URSP policies are applicable. Each AUN3 device is managed in 5G-RG as a separate device with its own NAS and states, therefore from point of view of 5GC, the AUN3 device is seen as a single device and the URSP received in the NAS dedicated to an AUN3 device shall apply to this AUN3 device only the 5G-RG shall apply URSP received within NAS for AUN3 device acting on behalf of AUN3 device. Therefore to support the case when AUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF for the SUPI associated with the AUN3 device.

UE Route Selection Policy information targeting an AUN3 device (i.e. sent to a 5G-RG in the NAS connection corresponding to an AUN3 device) follows the structure defined in clause 6.2.2 of TS 23.503 [4] with the following difference:

-	As an AUN3 can have only one PDU Session, its URSP shall contain a match all TD.

USRP rule enforcement reporting is not defined for AUN3 devices.

In order to support the case when NAUN3 devices may be connected via 5G-RG, specific URSP rules may be configured by the PCF on 5G-RG.

URSP rules for NAUN3 devices connected to 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4] and may contain any combination of the following traffic descriptors:

-	IP Descriptors: For IP traffic from NAUN3 devices connected to 5G-RG, IP descriptors are matched against header information contained in IP packets sent by NAUN3 devices; IP descriptors are only applicable for traffic from NAUN3 devices if network address translation (NAT) is performed for that traffic.

-	Non-IP descriptors: For Ethernet traffic from NAUN3 devices connected to 5G-RG, Non-IP descriptors are matched against header information contained in Ethernet frames sent by NAUN3 devices.

-	Connectivity Group ID: For traffic from a NAUN3 device connected to 5G-RG, Connectivity Group ID in the URSP rule is matched against the Connectivity Group ID that the NAUN3 device is associated with (see clause 4.10b).

URSP rules for IN3 devices accessing the 5GC behind a 5G-RG follow the structure defined in clause 6.6.2 of TS 23.503 [4]

# * * * * End of changes * * * *