3GPP SA WG2 Meeting #164	 S2-2408182

# Maastricht, NL, 19-23 August 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5545 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 23.501 enhancements to support UE-SAT-UE communication when the serving satellite changes |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Addition of features to support UE-satellite-UE communications for satellite communications when serving satellite changes based on the conclusions for KI#3 in TR 23.700-29. An AGW relocation procedure is needed for change of satellite to route the UE-satellite-UE traffic to the new AGW. The simultaneous UL/CL change due to satellite change can be coordinated with AGW relocation to avoid packet loss. To support UE-satellite-UE communications: - SMF needs to notify the changes of satellite to P-CSCF via PCF. - The serving satellite changes potentially result in UL CL UPF and L-PSA UPF and/or AGW relocation. Reuse the existing procedure Edge Relocation described in clause 6.3 of TS 23.548 can achieve AGW and/or L-PSA UPF relocation. The procedure can be initiated by P-CSCF or SMF, which can be fully applicable to all scenarios. - The relocation caused by serving satellite changes can be initiated by SMF, as described in clause 6.3.5 of TS 23.548, e.g. determining to relocate UL CL UPF and/or L-PSA UPF. This procedure supports low packet loss when AGW changes. - The relocation caused by serving satellite changes can be initiated by P-CSCF, as described in clause 6.3.7 of TS 23.548, e.g. determining to relocate AGW. - The relocation caused by AGW load balance or maintenance (serving satellite not change) can be initiated by P-CSCF, as described in clause 6.3.7 of TS 23.548. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause to describe the feature and general procedure to support serving satellite change. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Voice/video call will lost or high packet loss when AGW changes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.x.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change(all new) * * * *

5.4.x.y	Mobility support

When UE’s serving satellite changes (e.g. UE’s movement, or satellite’s movement), the changed satellite ID will be sent to the P-CSCF by 5GC, and then P-CSCF determines to continue or deactivate the UE-SAT-UE communication according to the procedure described in TS 23.228 [15].

Based on the different deployments e.g. UPF onboard or not onboard the UE’s serving satellite, AGW onboard or not onboard the UE’s serving satellite, the serving satellite changes can result:

-	the serving gNB changes and UE-SAT-UE communication can be continued. The Xn/N2-based handover procedure is performed as specified in clause 4.9.1 of TS 23.502 [3].

-	the serving gNB and UPF changes and UE-SAT-UE communication can be continued. To achieve this, the Edge Relocation procedure is performed as specified in clause 6.3 of TS 23.548 [130]. In this procedure, P-CSCF acts as the AF, and AGW on board satellite acts as the EAS. The procedures are initiated by SMF.

-	the serving gNB, UPF, and AGW changes. UE-SAT-UE communication can be continued or deactived. If the UE-SAT-UE communication is deactived, the SMF removes the UL CL UPF and local PSA UPF on-board satellite and the AGW is changed to ground. To achieve this, the edge Relocation procedure is performed as specified in clause 6.3 of TS 23.548 [130]. In this procedure, P-CSCF acts as the AF, and AGW on board satellite acts as the EAS. The procedures are initiated by P-CSCF or SMF.

# * * * * End of changes * * * *