SA WG2 Meeting #164	S2- 2408187

Maastricht, Netherlands, August 19 – 23, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1429 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS AS Service Enhancement to Support IMS Event Exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-13, the conclusion for KI#1 is agreed to enhance IMS AS service to support explicitly subscribe to IMS events. The notification of IMS events is concluded as following: 7. For the purpose of optimization and following 5GC exposure principles, any Notification related to a subscription is sent directly to the NF (e.g. NEF or AF) that triggered the subscription request using Nimsas_SessionEventControl Notify service operation. It is proposed to enhance IMS service to support subscription and notification of IMS events. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance IMS service to support subscription and notification of IMS events. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The subscription of the IMS events to IMS AS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AA.2.4	IMS AS Services

AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF, HSS |
|  | Notify | Subscribe/Notify | DCSF, NEF, AF |
|  | Unsubscribe | Subscribe/Notify | DCSF, HSS |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

AA.2.4.2	Nimsas_SessionEventControl

AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events.

-	Notifying IMS session control events of a specific IMS subscriber to NFs, e.g. DCSF, based on local configuration in the IMS AS.

IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification. The consumer is given the possibility to apply session media control and policies by invoking the Nimsas_MediaControl service before giving IMS AS a session event notification acknowledgement.

If the consumer does not apply media control/policies to a session event, IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

AA.2.4.2.2	Nimsas_SessionEventControl_Subscribe service operation

Service operation name: Nimsas_SessionEventControl_Subscribe

Description: This service operation enables NF consumers (e.g. DCSF, HSS) subscribe session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

Inputs, Required: NF type, Event type.

Event type is DC event or non-DC event.

Inputs, Optional: IMS Subscriber, Identity list of IMS subscribers.

IMS Subscriber is the subscriber requesting use of data channel media and its session event is subscribed.

Identity list of IMS subscribers is the identity list of IMS subscribers and the IMS Subscriber in the list is requesting use of data channel media.

These two inputs are optional and only one of them will be given in a service operation.

Outputs, Required: Result indication, when the subscription is accepted: Subscription Correlation ID.

Outputs, Optional: None.

AA.2.4.2.3	Nimsas_SessionEventControl_Notify service operation

Service operation name: Nimsas_SessionEventControl_Notify

Description: This service operation enables IMS AS to notify consumers of session events related to a specific served IMS subscriber requesting use of IMS data channel media.

Inputs, Required: Session ID, Event ID,

Session ID is the identity of the IMS session for which the event relates to.

Event ID is the event triggered within the IMS session.

Inputs, Optional: Calling ID, Called ID, Session case, Event initiator, Media info list.

Calling ID is the public identity of the calling IMS subscriber. Called ID is the public identity of the called IMS Subscriber. Session case indicates if this is an originating or terminating IMS session. Event initiator indicates initiator of the event, i.e. 'served IMS subscriber' vs 'remote IMS subscriber'. Media info list includes for each media in the list:

-	media ID: uniquely identifies this media item within the list. The identity is allocated by IMS AS,

-	media specification: This depends on media type including relevant media attributes of interest to the consumer. The media specification includes the following media description attributes:

-	Media Type: DC, Audio, or Video.

When the media type is "DC", the elements below are derived from the SDP received by the IMS AS in an SIP INVITE or a re-INVITE related to an IMS Data Channel and the corresponding DTLS connection.

-	Data Channel Mapping and Configuration Information: This attribute is applicable to Data Channel and includes relevant configuration Information, including the stream ID and application binding information of the Data Channel.

-	Maximum Message Size: This attribute defines the maximum size to be expected.

-	Data Channel Port: This attribute identifies the port for the Data Channel.

-	Security Setup: This attribute identifies the security set up of the DTLS connection.

-	Security Certificate Fingerprint: This attribute identifies the security certificate fingerprint.

-	Security Transport Identity: This attribute identifies transport layer identity.

Outputs, Required: Result indication.

Outputs, Optional: None.

The table below presents supported EventIDs and related parameters.

Table AA.2.4.2.2-1: List of events and Related Optional parameters

| EventID | Parameters |
| --- | --- |
| SessionEstablishmentRequestEvent | Calling ID, Called ID, Session case, Event initiator, Media info list |
| SessionEstablishmentProgressEvent | Media info list |
| SessionEstablishmentAlertingEvent | Media info list |
| SessionEstablishmentSuccessEvent | Media info list |
| SessionEstablishmentFailureEvent |  |
| MediaChangeRequestEvent. | Event initiator, Media info list |
| MediaChangeSuccessEvent | Media info list |
| MediaChangeFailureEvent | Media info list |
| SessionTerminationEvent | Session case |

AA.2.4.2.2	Nimsas_SessionEventControl_Unsubscribe service operation

Service operation name: Nimsas_SessionEventControl_Unsubscribe

Description: This service operation enables NF consumers (e.g. DCSF, HSS) unsubscribe session events related to a specific served IMS subscriber or a list of IMS subscribers requesting use of data channel media.

Inputs, Required: Subscription Correlation ID

Inputs, Optional: None

Outputs, Required: Result indication

Outputs, Optional: None.

END OF CHANGES