SA WG2 Meeting #164	S2- 2408190

Maastricht, Netherlands, August 19 – 23, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1431 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | IMS Capability Exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In TR 23.700-13, the conclusion for KI#2 is agreed to enhance IMS AS service to support IMS capability exposure as following: 2. The following capabilities need to be exposed by the IMS network: - establishing, updating, terminating of bootstrap data channels; - establishing, updating, terminating of (P2A, P2A2P, P2P) application data channels. 3. The Nnef and Nimsas services and/or service operations need to be enhanced to support the capabilities mentioned above. NOTE: The detailed impacts to the services and/or service operations are defined in normative phase. 4. In Rel-19, the only service provider is IMS AS. The service consumer needs to discover the IMS AS for a given user via HSS before sending a request to IMS AS. If the user is not registered yet, the request is rejected. It is proposed to enhance IMS AS service to support IMS capability exposure for Data Channel control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhance IMS AS service to support IMS capability exposure for Data Channel control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IMS capability exposure for Data Channel by IMS AS will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.2.4.1, AA.2.4.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AA.2.4.1	General

The following table shows the IMS AS Services and IMS AS Service Operations.

Table AA.2.4.1-1: NF services provided by the IMS AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nimsas_SessionEventControl | Subscribe | Subscribe/Notify | DCSF |
|  | Notify | Subscribe/Notify | DCSF |
| Nimsas_MediaControl | MediaInstruction | Request/Response | DCSF |
| Nimsas_DataChannelControl | Create | Request/Response | DCSF, NEF |
|  | Update | Request/Response | DCSF, NEF |
|  | Delete | Request/Response | DCSF, NEF |

NOTE:	In this release the IMS AS services is introduced to support Data Channel services in IMS further described in Annex AC.

Next CHANGE

AA.2.4.x	Nimsas_DataChannelControl Service

AA.2.4.x.1	General

Service description: This service enables the consumer e.g. DCSF, NEF to request IMS AS to create/update/delete data channels within an existing IMS session.

AA.2.4.x.2	Nimsas_DataChannelControl_Create service operation

Service operation name: Nimsas_DataChannelControl_Create

Description: The NF consumer requests IMS AS to create one or more IMS data channels to the identified IMS Session.

Inputs, Required: User Identity, Session ID, list of DCs, DC Media Specification.

User Identity is Private Identity and/or Public Identity of the user.

Session ID specifies the IMS session for which the service operation applies.

List of DCs is the data channels for which the service operation applies.

DC Media Specification is description of additional media specification information needed for Data Channel media stream.

Inputs, Optional: Binding information of data channel.

Binding information is the information used for controlling the Application DC setup.

Outputs, Required: Transaction Reference ID.

AA.2.4.x.3	Nimsas_DataChannelControl_Update service operation

Service operation name: Nimsas_DataChannelControl_Update

Description: The NF consumer requests IMS AS to update one or more IMS data channels to the identified IMS Session.

Inputs, Required: Transaction Reference ID, list of DCs, DC Media Specification.

Inputs, Optional: Binding information of data channel.

Outputs, Required: Operation result indication.

AA.2.4.x.3	Nimsas_DataChannelControl_Delete service operation

Service operation name: Nimsas_DataChannelControl_Delete

Description: The NF consumer requests IMS AS to delete one or more IMS data channels to the identified IMS Session.

Inputs, Required: Transaction Reference ID, list of DCs.

Inputs, Optional: None.

Outputs, Required: Operation result indication.

END OF CHANGES