SA WG2 Meeting #164	S2- 2408193

Maastricht, Netherlands, August 19 – 23, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1434 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for Standalone Bootstrap Data Channel Establishment |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion for KI of support of standalone IMS data channel sessions in TR 23.700-13, standalone bootstrap data channel should be supported. It is proposed to add procedure for standalone bootstrap data channel establishment. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for standalone bootstrap data channel establishment. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for standalone bootstrap data channel establishment is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.1.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.7.1.x	Procedure for Standalone Bootstrap Data Channel Establishment

Figure AC.7.1.x-1 depicts a signalling procedure for establishing a standalone bootstrap data channel. When a UE initiates establishment of a standalone bootstrap data channel, the UE includes a specific PSI used for standalone bootstrap DC only in the INVITE request as the Request-URI to terminate the session and data channel towards IMS AS and DCSF. The procedure for bootstrap data channel set up signalling as shown in Figure AC.7.1-1 of clause AC.7.1 is reused with some modifications.

Figure AC.7.1.x-1: Procedure for Standalone Bootstrap Data Channel Establishment

The steps in the procedure are as follows:

1-10.	Step 1-10 of Figure AC.7.1-1 in clause AC.7.1 for bootstrap data channel setup signalling procedure are performed with the following differences:

In step 1, the UE#1 sends the SIP INVITE request, the Request URI of the SIP INVITE message is a specific target URI used for standalone bootstrap DC only. The initial SDP contains only bootstrap data channel offers for UE#1.

In step 5, DCSF determines that UE#1 requests to establish a standalone bootstrap DC based on the value of the Called ID and Media Info List, and creates only the originating side media information.

In step 6, DCSF instructs the IMS AS how to set up bootstrap data channel with MF for originating side only.

In step 8, IMS AS instructs MF to allocate required data channel media resources for originating side only. IMS AS request creation of only one Media Termination which representing the local bootstrap media to be terminated.

11.	IMS AS terminates the IMS session based on the specific Request URI and sends 18X response with the SDP answer for bootstrap data channel through S-CSCF and P-CSCF to UE#1.

12.	IMS AS sends 200 OK to the UE #1, and the IMS session for standalone bootstrap data channel is established.

13.	The standalone bootstrap data channels have been established between originating MF or MRF and UE#1. The UE#1 sends application request messages to MF or MRF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel with its data channel capabilities. The MF or MRF replaces the root URL with the replacement URL received in steps 8 and forwards the message to received media point of DCSF. The DCSF provides the application list and proper data channel applications further to UE#1 based on their data channel capabilities and their choices through MF or MRF.

14-15.	If the established bootstrap DC is not used for a period of time, UE#1 sends SIP BYE to release the bootstrap DC.

END OF CHANGES