SA WG2 Meeting #164	S2- 2408194

Maastricht, Netherlands, August 19 – 23, 2024                  	(revision of S2-24xxxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1435 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for Transition Between IMS Avatar and Audio/video Communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on conclusion for KI of support of IMS Avatar communication in TR 23.700-13, transition between IMS Avatar and audio/video communication should be supported. It is proposed to add procedure for transition between IMS Avatar and audio/video communication. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add procedure for transition between IMS Avatar and audio/video communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Procedure for transition between IMS Avatar and audio/video communication is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

AC.7.x	Procedure for Transition Between IMS Avatar and Audio/video Communication

Transition between IMS Avatar and audio/video communication is based on the IMS Avatar communication architecture as specified in clasue AC.7.y. The transition procedure includes procedure for transition from audio/video communication to IMS Avatar communication in clause AC.7.x.1 and procedure for transition from IMS Avatar communication to audio/video communication in clause AC.7.x.2.

AC.7.X.1 Procedure for Transition from Audio/video to IMS Avatar Communication

Figure AC.7.X.1-1 depicts a signalling procedure for transition from audio/video to IMS Avatar communication.

Figure AC.7.x.1-1: Procedure for Transition from Audio/video to IMS Avatar Communication

The steps in the procedure are as follows:

0.	UE A has audio/video communication with UE B.

1.	UE A exchanges audio/video media with UE B through MF/MRF or directly over RTP.

2.	UE A decides to switch the communication from audio/video to Avatar communication mode.

3.	UE A interacts with IMS Core and UE B to negotiate the Avatar media and establish application data channel.

4.	DCSF instructs MF/MRF to fetch Avatar representation from Base Avatar Repository (BAR).

Avatar representation is specific for the user of UE A; it can be pre-configured in BAR or uploaded by the user.

5.	UE A captures sensing data of the user e.g. facial expressions data, body motion data.

6.	UE A sends Avatar animation data (i.e. sensing data) to MF/MRF through application data channel.

7.	MF/MRF renders Avatar representation received at step 4 based on the Avatar animation data received.

8.	MF/MRF sends rendered Avatar representation to UE B through application data channel.

9.	UE B displays rendered Avatar representation to user.

10.	UE B also sends its Avatar representation to UE A through MF/MRF; and the video between two UEs is stopped.

AC.7.X.2 Procedure for Transition from IMS Avatar to Audio/video Communication

Figure AC.7.X.2-1 depicts a signalling procedure for transition from IMS Avatar to audio/video communication.

Figure AC.7.x.2-1: Procedure for Transition from IMS Avatar to Audio/video Communication

The steps in the procedure are as follows:

0.	UE A has IMS Avatar communication with UE B.

1.	UE A exchanges Avatar media (i.e. Avatar representation) with UE B transferred by MF/MRF through application data channel.

2.	UE A decides to switch the communication from IMS Avatar to audio/video mode.

3.	UE A negotiates with IMS Core and UE B for exchanging video media between UE A and UE B.

4.	UE A exchanges video media with UE B through MF/MRF or directly over RTP.

The video media in step. 3 and 4 is the real video of users and is captured by the camera of UEs, rather than Avatar media.

5.	UE A negotiates with IMS Core and UE B to stop exchanging Avatar media and may release application data channel.

END OF CHANGES