SA WG2 Meeting #164	S2- 2408201

Maastricht, NL, August 19–23, 2024	(revision of S2-24xxxxx)

Source:	ZTE

Title:	KI#1, 2, 3, Sol#22, Clarification on AF Based Solution over Control Plane

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: Update Solution #22 to clarify AF based solution over control plane.

1. Discussion

The paper is proposed to clarify AF base solution over control plane.

For control plane, both Non IP Data Delivery (NIDD) through NEF and 5G CIoT Control Plane Optimisation through UPF are supported in Solution #22.

2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

6.22.1.2	Protocol Stack

The Figure 6.22.1.2-1 illustrates the protocol stack for AF based solution over user plane:

Figure 6.22.1.2-1: Protocol Stack for AF Based Solution (over user plane)

Within the protocol stack:

-	UE AIoT layer: between AF and UE reader. AF provides Ambient IoT operation commands to Intermeidate UEs via UE AIoT layer.

-	Command layer: The command layer protocol between AIoT devices and AF, including, e.g. read, write, execute, etc.

-	Uu AS layer: On top of the existing Uu AS layer, radio resource information request from UE reader to NG-RAN.

It is assumed that the end-to-end protection is implemented between AF and AIoT devices.

NOTE 1:	The UE AIOT layer and Command layer are assumed to be defined by SA WG2 (Stage 2 aspects) and CT WG1 (Stage 3 aspects) but can be determined during conclusion phase.

NOTE 2:	The Details of the protection between AIoT device and AF and the protection between AIoT device and UE reader are assumed to be addressed by SA WG3.

The UE AIOT layer messages between AF and UE reader can be transferred via PDU session user plane. AF delivers downlink messages towards Intermediate UE and receives uplink messages from Intermediate UE via UPF.

The protocol stack for AF Based Solution over control plane is similar, with the involvement of CN control plane NFs (AMF, SMF, NEF). Over control plane, UE AIOT layer is over the NAS layer in UE reader. AF communicates with UE reader via control plane, by utilizing the Control Plane CIoT 5GS Optimisation through UPF as specified in clause 5.31.4 of TS 23.501 [4] and clause 4.24 of TS 23.502 [5] or the Non IP Data Delivery (NIDD) through NEF as specified in clause 5.31.5 of TS 23.501 [4] and clause 4.25 of TS 23.502 [5]. The downlink messages are sent by AF, and delivered through UPF or NEF, SMF, AMF, NG-RAN towards Intermediate UE. The uplink messages are sent by the Intermediate UE, and delivered through NG-RAN, AMF, SMF, UPF or NEF towards AF.

* * * * End of Changes * * * *