SA WG2 Meeting #164	S2- 2408202

Maastricht, NL, August 19–23, 2024	(revision of S2-24xxxxx)

Source:	ZTE

Title:	Evaluation and Conclusion for Topology 2 AIoT Architecture

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: Add evaluation and conclusion for Topology 2 AIoT architecture.

1. Discussion

Based on the solutions in TR 23.700-13, there are two categories of solutions for Topology 2 AIoT architecture: control plane based and user plane based. The paper lists these two categories of solutions for Topology 2 AIoT architecture and proposed protocol stacks, and further evaluate the advantages and disadvantages of these two categories of solutions. Based on the evaluation, the paper proposes to take CP based solution as the baseline for the normative work for Topology 2 AIoT architecture.

2. Proposal

It is proposed to capture the following changes into TR 23.700-13.

* * * * Start of Changes * * * *

7	Overall Evaluation

7.X	Topology 2 Architecture Evaluation

For Topology 2, the following solutions provide CP architecture and protocol stack:

Solution #21:

Solution #30:

Solution #33:

Some solutions use CP architecture while without providing protocol stack, such as: Solution #18, Solution #23, Solution #27, and Solution #31.

The following solutions provide UP architecture and protocol stack:

Solution #22:

Solution #32:

Solution #34 uses UP architecture while without providing protocol stack.

Solution #4 provides both CP and UP protocol stack for Topology2:

UP based solutions can reuse legacy deployed network to fulfil AIoT functions. It requires Reader UE to establish user plane connection to transfer AIoT data between UE and AF; while the transportation of AIoT data is not always needed since it’s a short period operation for AIoT devices. In addition, for UP based solution AIoT traffic in user plane will bypass the RAN, so the RAN will not recognize the AIoT operations to optimize the radio resource used by AIoT service.

For topology 1, CP based architecture and protocol stack will be used. To align with topology 1, it will be better to endorse similar mechanism for topology 2. Furthermore, the RAN may have more accurate information about the association between Reader UE and AIoT devices in CP based architecture. To support AIoT operations (inventory and command), CP-based solution is more feasible to make the RAN have the ability to control the service operations.

It proposed to endorse CP based solution for topology 2 architecture. CP based solution includes RRC involved (Sol#33) and NAS involved (Sol#21, 30); it needs further discussion on which one will be the way forward.

* * * * Next Change * * * *

8	Conclusions

8.X	Interim Conclusion for Topology 2 Architecture

It proposed to endorse CP based solution for topology 2 architecture and proceed normative work based on it.

NOTE 1: CP based solution includes RRC involved and NAS involved; it needs further discussion on which one will be the way forward.

Enhanced AMF or a new AIoTMF function is used to handle service operations for AIoT devices.

A UE needs to be authorized as an AIoT Reader before interacting with AIoT devices.

The protocol stack and interaction between AIoT device and UE Reader will be defined by RAN WGs.

NOTE 2:	The security aspects for Ambient IoT requires coordination with SA WG3.

* * * * End of Changes * * * *