SA WG2 Meeting #164 	S2-2408229

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1156 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedure of sample alignment when NWDAF as VFL Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in Conclusions for KI#2: 5GC Support for Vertical Federated Learning, the NWDAF acting as VFL Server should obtain the samples from the VFL clients, and generates the intersection of samples. P#2.3.1: For NWDAF acting as VFL Server, NWDAF triggers sample alignment, may query NWDAFs or AFs acting as VFL clients to query availability of samples, and generates the intersection of samples. P#2.3.2 In case of VFL between the NWDAF as VFL server and AF as VFL client, the NEF may be involved in sample alignment with mapping of information (e.g. internal versus external information), then NWDAF as VFL server will determine the final list of participants supporting the samples for VFL training. Following the above conclusion, this CR proposes to add a new clause to introduce the procedure for how the NWDAF acting as VFL Server obtain the samples from the VFL clients, and generates the intersection of samples. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Base on the conclusion, this CR proposes to add a new clause to introduce the procedure for how the NWDAF acting as VFL Server obtain the samples from the VFL clients, and generates the intersection of samples. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature of sample alignment when the NWDAF acts as VFL Server is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text)* * *

6.2X.Y	Procedure for sample alignment when NWDAF acts as the VFL Server

Figure 6.2X.Y-1: Sample alignment procedure when NWDAF acts as the VFL server

1a. [Conditional] The NWDAF as VFL server starts samples alignment procedure with AF(s) as VFL client using Naf_VFLSampleAlign_Request service with Analytics ID, allocated ML Model ID, VFL Training Preparation Information, and optional analytics filter information, then it will obtain sample ID(s) for the VFL task from the AF(s) as VFL Client. If the AF is an untrusted AF, the NWDAF as VFL server sends sample alignment request via NEF.

The VFL Training Preparation Information may comprises:

-	Indication of VFL capability type to be performed by the NF receiving the VFL Training Preparation Indication;

-	feature spaces information selected by NWDAF as VFL server.

Editor’s Note: Whether and how to transports feature space information is FFS.

Editor’s Note: How the NEF assists the sample alignment process, and which NEF service should be invoked are FFS.

1b. [Conditional] The NWDAF as VFL server starts samples alignment procedure with other NWDAF(s) as VFL client using Nnwdaf_VFLSampleAlign_Request service with Analytics ID, allocated ML Model ID, VFL Training Preparation Information, and optional analytics filter information, then it will obtain sample ID(s) for the VFL task from other NWDAF(s) as VFL client.

2a. [Conditional] The AF(s) as VFL client returns the selected sample ID(s) to the NWDAF as VFL server by using Naf_VFLSampleAlign_Request Response service. If the AF is an untrusted AF, the AF returns the selected sample ID(s) via NEF. Besides, the NEF may be involved to translates the list of samples, e.g. GPSIs or External-Group-Id into SUPIs and Internal-Group-Id.

2b.	[Conditional] The NWDAF as VFL client returns the selected sample ID(s) to the NWDAF as VFL server by using Nnwdaf_VFLSampleAlign_Request service.

3.	The NWDAF as VFL server generates final samples based on the intersection of samples received samples in step 2a and/or 2b.

4a.	[Conditional] The NWDAF as VFL server sends the final samples to each AF(s) as VFL client by using the Naf_VFLSampleNotification_request service. If the AF is an untrusted AF, the NWDAF as VFL server sends the final samples to the AF(s) via NEF.

4b.	[Conditional] The NWDAF as VFL server sends the final samples to each NWDAF(s) as VFL client by using the Nnwdaf_VFLSampleNotification_request service.

* * * End of Changes * * *