SA WG2 Meeting #164 	S2- 2408231

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1158 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedure of VFL Inference when NWDAF as VFL Server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in Conclusions for KI#2: 5GC Support for Vertical Federated Learning, the NWDAF acting as VFL Server could trigger VFL inference and determine the VFL clients and VFL task correlation ID after receiving the analytics info request from a NF consumer. P#2.5.1: NWDAF acting as VFL server scenario, the NF consumer of analytics will obtain the required output based on the VFL inference process coordinated by VFL server, which is generated via the VFL inference process between VFL sever and corresponding AFs or NWDAF acting as VFL client(s): - For NWDAF acting as VFL server, NWDAF triggers the VFL inference phase after receiving the subscription or request for analytics and delivers the analytics to the NF consumer. P#2.5.3: Before performing the VFL inference, the NWDAF acting as VFL server determines corresponding AF(s) and/or NWDAF(s) as VFL client(s) for the inference process based on the same identifier used in the VFL training process. Related details will be specified in the normative phase. Following the above conclusion, this CR proposes to add a new clause to introduce the procedure for VFL inference when the NWDAF acts as VFL Server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Base on the conclusion, this CR proposes to add a new clause to introduce the procedure for VFL inference when the NWDAF acts as VFL Server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | New feature of VFL Inference when the NWDAF acts as VFL Server is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (all new text)* * *

6.2X.Y	Procedure for VFL Inference when NWDAF acts as the VFL Server

Figure 6.2X.Y-1: VFL Inference procedure when NWDAF acts as the VFL server

1.	To obtain analytics, the NF consumer sends analytics request to NWDAF as VFL server by using Nnwdaf_AnalyticsSubscription_Subscribe service or Nnwdaf_AnalyticsInfo_Request service with analytics ID, Target of Analytics Reporting and analytics filter information.

2.	For the analytics request, the NWDAF as the VFL server decides to perform VFL inference. If VFL training for the analytics ID has not been performed, then it triggers VFL training process as defined in clause 6.2x.x. If VFL training for the analytics ID has been finished, then it determines the ML Model ID used in previous VFL training process for the analytics ID, and corresponding VFL clients (i.e. NWDAF(s) and/or AF(s)) based on the ML Model ID, which participated in the VFL training process.

3a.	[Conditional] The NWDAF as VFL server sends VFL inference request to corresponding AF(s) as VFL client by using Naf_VFLInference_Request or Naf_VFLInference_Subscribe service, including a flag that indicates the VFL Client(s) to perform VFL inference, the ML Model ID, analytics ID and analytics filter information. If the AF is an untrusted AF, the VFL inference request is sent via NEF.

Editor’s Note: How the NEF assists the VFL inference process, and which NEF service should be invoked are FFS.

3b.	[Conditional] The NWDAF as VFL server sends VFL inference request to corresponding NWDAF(s) as VFL client by using Nnwdaf_VFLInference_Request or Nnwdaf_VFLInference_Subscribe service, including a flag that indicates the VFL Client(s) to perform VFL inference, the ML Model ID, analytics ID and analytics filter information.

4.	Each VFL Client generates VFL inference intermediate result for the target UE based on its local input data and the local model corresponding to the ML Model ID trained in previous VFL training.

5a. [Conditional]	Each AF as VFL client sends the VFL inference intermediate result for the target UE to the NWDAF as the VFL server by using Naf_VFLInference_Request Response or Naf_VFLInference_Notify service. If the AF is an untrusted AF, it sends the intermediate results to the NWDAF as the VFL server via NEF.

5b.	[Conditional] Each NWDAF as VFL client sends the VFL inference intermediated result for the target UE to the NWDAF as the VFL server by using Nnwdaf_VFLInference_Request Response or Nnwdaf_VFLInference_notify service.

6.	For the target UE, the NWDAF as the VFL server aggregates the VFL inference intermediate result from each VFL Client(s) and generates the analytics result.

7.	The NWDAF sends analytics info response to the NF consumer with the generated analytics result.

* * * End of Changes * * *