SA WG2 Meeting SA2#164	S2-2408236

19 - 23 August, 2024, Maastricht, Netherlands	(revision of)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3804 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Architecture and functional description for S&F |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR23.700-29 has concluded the S&F operation with MME split architecture and full CN onboard. Corresponding conclusions should be capture in TS 23.401. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the functional descriptions for Support of Store and Forward Satellite operation based on conslusions. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | S&F is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.x (New clause), 3.1, Annex X(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

MME Pool Area: An MME Pool Area is defined as an area within which a UE may be served without need to change the serving MME. An MME Pool Area is served by one or more MMEs ("pool of MMEs") in parallel. MME Pool Areas are a collection of complete Tracking Areas. MME Pool Areas may overlap each other.

Serving GW Service Area: A Serving GW Service Area is defined as an area within which a UE may be served without need to change the Serving GW. A Serving GW Service Area is served by one or more Serving GWs in parallel. Serving GW Service Areas are a collection of complete Tracking Areas. Serving GW Service Areas may overlap each other.

PDN Connection: The association between a PDN represented by an APN and a UE, represented by one IPv4 address and/or one IPv6 prefix (for IP PDN Type) or by the UE Identity (for Non-IP and Ethernet PDN Types).

Default Bearer: The EPS bearer which is first established for a new PDN connection and remains established throughout the lifetime of the PDN connection.

Default APN: A Default APN is defined as the APN which is marked as default in the subscription data and used during the Attach procedure and the UE requested PDN connectivity procedure when no APN is provided by the UE.

eCall Only Mode: A UE configuration option that allows the UE to attach at EPS and register in IMS to perform only eCall Over IMS, and an IMS call to a non-emergency MSISDN or URI for test and/or terminal reconfiguration services. For a short period following either such call, an incoming call (e.g. callback from a PSAP or HPLMN operator) or other incoming session (e.g. for USIM reconfiguration) is possible. At other times when the UE is configured in this mode, the UE is required to refrain from any signalling to a network. Use of eCall Only Mode is configured in the USIM for the UE.

PDN Connection to the SCEF: The association between a UE, represented by the UE Identity, and a PDN represented by an APN to external packet data network via SCEF to allow transfer of Non-IP data. It includes establishment and persistence of T6 connection between MME and SCEF (see TS 29.128 [79]).

Emergency attached UE: A UE which only has bearer(s) related to emergency bearer service.

NOTE 1:	The above term is equivalent to the term "attached for emergency bearer services" as specified in TS 24.301 [46].

LIPA PDN connection: a PDN Connection for local access (e.g. for IP or Ethernet access) for a UE connected to a HeNB.

en-gNB: As defined in TS 37.340 [85].

SIPTO at local network PDN connection: a PDN connection for SIPTO at local network for a UE connected to a (H)eNB.

Correlation ID: For a LIPA PDN connection, Correlation ID is a parameter that enables direct user plane path between the HeNB and L-GW.

SIPTO Correlation ID: For a SIPTO at local network PDN connection, SIPTO Correlation ID is a parameter that enables direct user plane path between the (H)eNB and L-GW when they are collocated.

Local Home Network: A set of (H)eNBs and L-GWs in the standalone GW architecture, where the (H)eNBs have IP connectivity for SIPTO at the Local Network via all the L-GWs.

Local Home Network ID: An identifier that uniquely identifies a Local Home Network within a PLMN.

Presence Reporting Area: An area defined within 3GPP Packet Domain for the purposes of reporting of UE presence within that area due to policy control and/or charging reasons. In the case of E-UTRAN, a Presence Reporting Area may consist in a set of neighbor or non-neighbor Tracking Areas, or eNodeBs and/or cells. There are two types of Presence Reporting Areas: "UE-dedicated Presence Reporting Areas" and "Core Network pre-configured Presence Reporting Areas" that apply to an MME pool.

RAN user plane congestion: RAN user plane congestion occurs when the demand for RAN resources exceeds the available RAN capacity to deliver the user data for a prolonged period of time.

NOTE 2:	Short-duration traffic bursts is a normal condition at any traffic load level, and is not considered to be RAN user plane congestion. Likewise, a high-level of utilization of RAN resources (based on operator configuration) is considered a normal mode of operation and might not be RAN user plane congestion.

IOPS-capable eNodeB: an eNodeB that has the capability of IOPS mode operation, which provides local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs via a Local EPC when the eNodeB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNodeBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNodeBs in IOPS mode of operation use, instead of the Macro EPC, in order to support public safety services.

Macro EPC: the EPC which serves an eNodeB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNodeB(s)), local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs in the absence of normal EPS

Multi-USIM UE: a UE with multiple USIMs, capable of maintaining a separate registration state with a PLMN for each USIM at least over 3GPP Access and supporting one or more of the features described in clause 4.3.33.

IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

Cellular IoT: Cellular network supporting low complexity and low throughput devices for a network of Things. Cellular IoT supports IP, Ethernet and Non-IP traffic. Unless otherwise stated in this specification, Cellular IoT and all functionality applicable to Cellular IoT also apply to satellite access.

Narrowband-IoT: a 3GPP Radio Access Technology that forms part of Cellular IoT. It allows access to network services via E-UTRA with a channel bandwidth limited to 180 kHz (corresponding to one PRB). Unless otherwise indicated in a clause, Narrowband-IoT is a subset of E-UTRAN. Unless otherwise stated in this specification, Narrowband-IoT also includes satellite access.

LTE-M: a 3GPP RAT type Identifier used in the Core Network only, which is a sub-type E-UTRAN RAT type, and defined to identify in the Core Network the E-UTRAN when used by a UE indicating Category M in its UE radio capability. Unless otherwise stated in this specification, LTE-M also includes satellite access.

WB-E-UTRAN: in the RAN, WB-E-UTRAN is the part of E-UTRAN that excludes NB-IoT. In the Core Network, the WB-E-UTRAN also excludes LTE-M. Unless otherwise stated in this specification, WB-E-UTRAN also includes satellite access.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

For the purposes of the present document, the following terms and definitions given in TS 23.167 [81] apply:

eCall Over IMS: See TS 23.167 [81].

RLOS attached UE: A UE is attached only for accessing Restricted Local Operator Services (see TS 23.221 [27]).

IAB-donor: For the purposes of this specification, this is a NR Secondary RAN node is further described in TS 37.340 [85] that supports Integrated access and backhaul (IAB) feature and provides connection to the core network to IAB-nodes. It supports the CU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

IAB-node: A relay node that supports wireless in-band and out-of-band relaying of NR access traffic via NR Uu backhaul links. It supports the UE function and the DU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

Feeder link: as defined in TS 36.300 [6].

Service link: as defined in TS 36.300 [6]

S&F Satellite operation: S&F (Store and Forward) Satellite operation is an operation mode of a E-UTRAN system with satellite-access where the E-UTRAN system can provide some level of service (by storing and forwarding the data) when satellite can not provide simulatenous service link connection and the feeder link connection.

Second Changes

4.13.x	Support of Store and Forward Satellite operation

4.13.x.1	General

Store and forward satellite operations are for delay-tolerant communication service under satellite-access, where the network provide service (in storing and forwarding the data) when satellite connectivity is intermittently/temporarily unavailable, e.g. to provide communication service for UEs under satellite coverage without a simultaneous active feeder link connection to the ground segment.

The examples of deployment architetures to support Store and Forward Satellite Operation are illustrated in informative Annex X.

A UE determines that the network is operating in Store and Forward Satellite mode of operation based on the SIB of indicating Store and Forward Satellite Operation as described in TS 36.300 [5]

NOTE 1:	The trigger for the eNB to broadcast support of S&F operation is based on the decision of RAN WGs. From system perspective the expectation is that if the network does not support S&F operation and the feeder link is not available, then eNB switches off and does not broadcast any signal.

When UE initiates Attach or TAU procedure, it indicates support for S&F operation to the MME. If UE doesn’t indicate support of S&F operation capability, MME shall reject the UE request with suitable cause.

As part of an Attach, TAU, Control Plane Service Request or Service Request for Store and Forward operation, if no UE context for the UE exists, attach request is not integrity protected, check of the integrity is failed, or the HSS on-board satellite do not have UE specific authentication vectors or subscription information, the onboard MME rejects corresponding request with a S&F Wait Timer, or a S&F Monitoring List, or both for future signalling and data exchange. The onboard MME functionality or other functionality on the satellite payload may request information (e.g. the authentication, security and UE context information) to support the UE access request, once the feeder link becomes available.

In case of SGW and/or PGW being on ground, as part of an Attach procedure with a PDN connection needing to be established or as part of UE requested PDN connectivity procedure, the onboard MME rejects corresponding request with a S&F Wait Timer, or a S&F Monitoring List, or both for future signalling and data exchange. Once getting the ground connection, the onboard MME initiates the PDN connection establishment with SGW and PGW on ground.

NOTE X:	How the MME determines S&F Wait Time and/or the S&F Monitoring List is up to network implementation.

As part of an Attach, TAU, Control Plane Service Request, Service Request or UE requested PDN connectivity procedure, onboard MME may provide a S&F Wait Timer, or a S&F Monitoring List, or both for future signalling and data exchange in the corresponding accept message.

The S&F Wait Timer is used for UE to determine when to re-initiate a procedure or send MO data.

The S&F Monitoring List is used for UE to determine which satellite can be used for re-initiating a procedure or sending MO data.

To support verification of UE location as described in clause 4.13.4, the E-SMLC is deployed on the satellite.

In case of HSS being on ground, MME may indicate to HSS a "Request Time", allowing the HSS to check that no other (e.g. terrestrial) MME has sent an Update Location Request after the "Request Time" for the UE, and fetches the authentication vector and other details from HSS following current Authentication and security procedures. The MME may trigger Update location with the HSS and Update location ACK is received by MME. i.e. all the subscription details are retrieved by MME-ground. The Update Location Request includes an indication that this location update is provisional i.e. the HSS must not consider the UE as registered until it receives the final Update Location Request.

The MME may expose to the SCS/AS that a UE is registered in S&F Mode and a Downlink S&F estimated Delivery Time. The exposure to the SCS/AS is described in clause 5.6.1.4 of TS 23.682 [74].

Third Changes

Annex X (informative):

Annex X.1	Architecture for Satellite support Store and Forward in MME split

Annex X.1.1	Non-roaming architecture

Figure Annex X.1.1-1: Non-roaming architecture for Satellite support Store and Forward in MME split

Figure Annex X.1.1-1 depicts the EPS architecture in Non-roaming supporting Store and Forward of satellite under MME split case. The MME functionality is split into the two parts of MME-onboard and MME-ground. The MME-onboard is the MME part which is onboard the satellite and the MME-ground is the MME part which is on the ground network. The interface and interaction between the two parts of MME are implemented internally which is out of scope of 3GPP.

Annex X.1.2	Roaming architecture

Figure Annex X.1.2-1: Roaming architecture for Satellite support S&F in MME split. Home routed traffic

Figure Annex X.1.2-2: Roaming architecture for Satellite support S&F in MME split. Local breakout

The figures Annex X.1.2-1 and Annex X.1.2-2 represent the EPS architecture in roaming with home routed and local breakout supporting Store and Forward of satellite under MME split case respectively.

Annex X.2	Architecture for Satellite support Store and Forward with whole CN onboard

Figure Annex X.2-1: Architecture for Satellite support Store and Forward in MME split

Figure Annex X.2-1 depicts the EPS architecture for satellite supporting store and forward with whole CN onboard. The whole CN including eNB, MME, SGW, PGW, HSS, E-SMLC, SMSC etc are on board each satellite. Proxies are deployed on the satellite and the ground for application traffic, including support of MT traffic, MO traffic, SMS, etc. The implementation of the proxies and the interface between them is out of 3GPP scope.

End of Changes