3GPP TSG-WG SA2 Meeting #164	S2-2408250

Maastricht, Aug 19 – 23, 2024	(revision of S2-24xxxxx)

Source:	Nokia

Title:	Conclusions for KI#3

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract: initial conclusions for KI#3 are proposed.

# 1. Introduction/Discussion

This contribution is prepared to initially capture some conclusions for KI#3.

# 2. Text Proposal

It is proposed to approve the following changes in TR 23.700-13 v0.4.0.

FIRST CHANGE

# 8	Conclusions

## 8.3	Conclusions for key issue 3

It is proposed that the service principles below be considered for the normative work:

Editor's note:	Additional conclusions are FFS.

- Common for topology 1 and 2:

- Topology 1:

- Topology 2:

- The UE reader(s) is selected by the new NF responsible for AIoT service handling (e.g., AIoTF) based on the location information provided by the AF, UE authorization information, UE capability, UE related analytics, UE expected behaviours.

Editor's note: 	Whether the BS selects the UE reader(s) indicated by the CN as they are, or selects some or none of them based on the radio conditions, will be further discussed with RAN WG3.

- The new NF responsible for AIoT service handling (e.g., AIoTF) creates and maintains a binding table that indicates the association information between the AIoT devices and their UE reader(s) in order to avoid performing the UE selection procedure each time the 5GC receives the AIoT request for the same AIoT devices.

- The UE reader(s) forwards the data read from the AIoT devices to the AIoTF, over either CP or UP path. The UL data forwarding method is determined by the new NF responsible for AIoT service handling (e.g., AIoTF), considering the UE’s capability and the application requirements (e.g., aggregated data volume, latency, communication frequency)

- The data read from the AIoT devices can be aggregated by the UE reader(s) and/or the new NF responsible for AIoT service handling (e.g., AIoTF) before responding to the consumer AF.

- The selected UE(s) is configured with the necessary information for communicating with the AIoT devices and the CN, including the UE role, UP data forwarding method, aggregation policy, target reporting destination.

END OF CHANGES