3GPP TSG-WG SA2 Meeting #164	S2-2408251

Maastricht, Netherlands, 19-23 August 2024	(revision of S2-24xxxxx)

Source:	Apple

Title:	Conclusions for Key Issue #1

Document for:	Approval

Agenda Item:	19.14.1

Work Item / Release:	FS_AmbientIoT / Rel-19

Abstract of the contribution: This paper proposes some conclusions for Key Issue #1

# 1. Introduction

This contribution proposes some conclusions for Key Issues #1 in TR 23.700-13 based on Solution #33 for different types of Ambient IoT devices.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-13.

# * * * * Start of changes * * * *

## 8.1	Conclusions for Key Issue #1: Architecture support of Ambient IoT Devices

For Key Issue #1, the following is concluded.

1) The capabilities of AIoT devices vary quite significantly in terms of their baseband capabilities, storage, form factor, cost, complexity etc. As such, AIoT devices are classified into two device types:

a) Type A AIoT devices:

-	These AIoT devices may not need to address privacy requirements and may also use a fixed permanent device identifier that it uses for all communication. These AIoT devices are suitable for Topology 2 wherein the AIoT device communicates through an intermediate UE Reader.

-	These AIoT devices do not perform the 5GC Registration procedure with the network and the device does not authenticate with the network, though the network may perform certain checks to validate the AIoT device subscription and service operation based on the AIoT Device ID. The AIoT device has no NAS or RRC states as defined in 5GS. The AIoT device can be paged.

-	There is no PDU session created for session management and data transfer is done using NAS messages. The device communicates with the network directly by sending data (Ethernet, Unstructured frames) using its AIoT Device ID.

b) Type B AIoT devices:

-	These AIoT devices may use a temporary identifier to address privacy concerns. These AIoT devices are suitable for Topology 1 wherein the AIoT device communicates directly with the Base station (Reader).

-	These AIoT devices register with the network based on a network trigger (or a trigger from an external AF) and the network authenticates the device and assigns the device temporary identifiers. The AIoT device maintains some notion of NAS states (registered, de-registered, etc.) and can be paged.

-	The AIoT devices performs data transfer using NAS messages. The communication between the device and network is secure (may be encrypted and integrity protected).

2) The 5GC has a new network function AIoTCF (Ambient IoT Controller Function) to support different AIoT service operations (command, inventory, etc. The AIoTCF may be integrated with AMF as well. The AIoTCF exposes the Naiotcf interface and in case the AIoTCF is integrated with AMF, the Namf interface can be enhanced.

3) The AIoT NGAP protocol is the interface between the Reader and AIoTCF and can be a lightweight version of NGAP protocol tailored for AIoT or a new different protocol as well, which will be in scope of RAN3.

NOTE: There is an optional control protocol (AIoT C) between AIoT device and the Reader and will be defined by RAN.

# * * * * End of changes * * * *