3GPP TSG-SA WG2 Meeting #16z4 	S2-2408256

# Maastricht, NL, August 19 – 23, 2024	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1160 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical Federated Learning if AF is VFL Server in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on several principles listed in conclusion part in clause 8.2 of TR 23.700-84, the proposal specifies the procedure of Vertical Federated Learning among NWDAF(s) and/or AF in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the general procedure of Vertical Federated Learning among NWDAF(s) and/or AF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for Vertical Federated Learning will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE(All new text)

6.X	Vertical Federated Learning

6.X.1	Description

This clause specifies how NWDAF and AF can leverage Vertical Federated Learning technique.

6.X.2	Procedure

6.X.2.1	Preparation procedure of Vertical Federated Learning if AF is VFL Server

Figure 6.x.2.1-1 VFL Preparation if AF is VFL Server

1-3. VFL Server AF discovers NWDAF as VFL Client NF from NRF and/or UDM by invoking the Nnrf_NFDiscovery_Request and/or Nudm_UECM_Get service operation. The following criteria might be used: VFL capability Type (i.e., VFL client), data source NF, ML model interoperability ID, and data sample information (e.g., UE ID). If untrusted AF is involved, the NEF translates the candidate UE ID (e.g., from GPSI to SUPI).

NOTE: This VFL procedure assumes one NWDAF is involved in the VFL procedure.

4. VFL Server AF sends a request to the VFL Client NWDAF to query the availability of samples. The request includes the candidate sample IDs (e.g., list of DNAI, NSSAI, SUPI, GPSI) and VFL correlation IDs. If untrusted AF is involved, the NEF translates the candidate sample ID (e.g., from GPSI to SUPI).

5. VFL Client NWDAF checks the availability of the data.

NOTE: If VFL Client NWDAF determines to use another VFL model training procedure, the VFL Client NWDAF can initiate another VFL model training or inference procedure.

6. VFL Client NWDAF responds to the availability of the data. If untrusted AF is involved, the NEF translates candidate data sample information (e.g., SUPI into GPSI).

7. VFL Server AF demines the final list of data samples that will be used for the VFL process.

6.X.2.2	Vertical Federated Model Training if AF is VFL Server

Figure 6.x.2.2-1. VFL model training if AF is VFL Server

After VFL preparation procedure as in clause 6.X.2.1, the VFL Serve AF initiate the VFL model training procedure.

1. To start VFL training, VFL Server AF sends a request to the VFL Client NWDAF. The request includes an initial ML model that will be involved in the VFL training, sample information (e.g., a list of UE IDs), and the VFL correlation ID.

2. Each VFL Client NWDAF and/or AF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

3. The VFL Client NWDAF computes the intermediate results for its local ML models involved in the VFL training and reports the intermediate results to the VFL Server AF.

4. The VFL Server AF aggregates intermediate results from VFL Client NWDAF, trains a local model, and computes intermediate results based on its local ML model. VFL Server AF may compute different intermediate training information (e.g., gradient information, loss information) to update its own local model and the models of VFL clients during the VFL training process after processing the received intermediate results (that may include convergence reports).

5-6. VFL Server AF sends the intermediate results to VFL Client NWDAF. The VFL server/client(s) update their local ML model based on the received information.

NOTE: Steps 2-6 should be repeated until the training termination condition (e.g., the maximum number of iterations or the loss function result is lower than a threshold) is reached.

7. The VFL server AF determines when the VFL training process terminates, and then it informs the VFL Client NWDAF that the training ends.

6.X.2.3	Vertical Federated Inference if AF is VFL Server

Figure 6.x.2.3-1 VFL model inference if AF is VFL Server

1-2. AF triggers the VFL inference phase based on the internal logic.

NOTE: Step 2 can be triggered after the AF receives a data exposure request from 5GC.

3. The VFL Server AF interacts with the VFL Client NWDAF and selects the data sample information (e.g., GPSI) via the VFL Client discovery and sample alignment procedure in clause 6.X.2.1.

4. The VFL Server AF sends a VFL inference request to NWDAF (if AF is untrusted, NEF is inserted). The request includes data sample information and VFL correlation ID. The NEF translates the data sample information and selects VFL Client NWDAF. Then, the NEF sends a VFL inference request to the NWDAF.

NOTE: If VFL Client NWDAF determines to use another VFL model training procedure, the VFL Client NWDAF can initiate another VFL model training or inference procedure.

5. Each VFL Client NWDAF or AF collects its local data by using the current mechanism in clause 6.2 of TS 23.288 [5].

6. The VFL Client NWDAF computes the intermediate results for their local ML models involved in the VFL training and provides reports with the intermediate results to the VFL Server AF.

7-8. VFL server AF aggregates intermediate results from VFL client NWDAF and generates final results. The results can be consumed in AF and can be provided to 5GC in clause 6.2 of TS 23.288 [5].

END of CHANGES