3GPP TSG-SA WG2 Meeting #16z4 	S2-2408258

# Maastricht, NL, August 19 – 23, 2024	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1162 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Vertical Federated Learning if NWDAF is VFL Server in TS 23.288 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KDDI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on several principles listed in conclusion part in clause 8.2 of TR 23.700-84, the proposal specifies the procedure of Vertical Federated Learning among NWDAF(s) and/or AF in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define the general procedure of Vertical Federated Learning among NWDAF(s) and/or AF. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure for Vertical Federated Learning will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(All new text) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE(All new text)

6.X	Vertical Federated Learning

6.X.1	Description

This clause specifies how NWDAF and AF can leverage Vertical Federated Learning technique.

6.X.2	Procedure

6.X.2.1	Preparation procedure of Vertical Federated Learning if NWDAF is VFL Server

Figure 6.x.2.1-1 VFL if NWDAF is VFL Server

NWDAF as VFL Server NWDAF or VFL Client NWDAF registers to NRF with its NF profile, which includes NWDAF NF Type (see clause 5.2.7.2.2 of TS 23.502 [3]), Analytics ID(s), Address information of NWDAF, Service Area, VFL capability type information (i.e., VFL server or VFL client).

AF as VFL Server AF or VFL Client AF registers to NRF with its NF profile, which includes VFL capability type information (i.e. VFL server or VFL client). If AF is untrusted AF, NEF registers in its NEF profile.

1. The NWDAF as VFL Server determines, based on internal logic and the operator's policy, to use VFL to provide a particular analytics ID.

2-3. VFL Server NWDAF discovers and selects other NWDAF(s) and/or AF(s) as candidates of VFL Client NF(s) from NRF by invoking the Nnrf_NFDiscovery_Request service operation. VFL Server NWDAF may also interact with UDM by invoking Nudm_UECM service operation for NWDAF discovery. The following criteria might be used: VFL capability Type (i.e., VFL client) for NWDAF and AF, Analytic ID, ML model interoperability, and data sample information (e.g., UE ID) for NWDAF.

4. VFL Server NWDAF sends a request to the candidate VFL Client NF(s) (i.e., NWDAF(s) and/or AF(s)) to check the availability of the data sample. The request includes the candidate data sample information that will be used in the VFL procedure (e.g., list of DNAI, NSSAI, SUPI, and GPSI) and a VFL correlation ID that identifies the VFL process. If untrusted AF is involved, the NEF translates candidate data sample information (e.g., SUPI into GPSI).

5. VFL Client NF(s) check the availability of the candidate data sample indicated in Step 4.

6. The VFL Client NF(s) send the availability of each candidate in the data sample to the VFL Server NWDAF.

7. VFL Server NWDAF demines the final list of data samples that will be used for the VFL procedure and VFL Client NF(s) that will be involved in it.

6.X.2.2	Vertical Federated Model Training if NWDAF is VFL Server

Figure 6.x.2.2-1 VFL model training if NWDAF is VFL Server

After VFL discovery and sample alignment procedure as in clause 6.X.2.1, the VFL Serve initiate the VFL model training procedure.

1. To start VFL training, VFL Server NWDAF sends a request to the selected VFL Client NF (s). The request includes the initial ML model that will be involved in the VFL training, data sample information (e.g., list of UE ID(s)), and VFL correlation ID.

2. Each VFL Client NWDAF or AF collects its local data using the current mechanism in clause 6.2 of TS 23.288 [5]. The VFL Server NWDAF may collect label data from an NF, including the AF.

3. The VFL Client NF(s) compute the intermediate results for their local ML models involved in the VFL training and provide reports with the intermediate results to the VFL Server NWDAF.

4. The VFL Server NWDAF aggregates intermediate results from VFL Client NF (s), trains a local model and computes intermediate results based on its local ML model. The VFL server may compute different intermediate training information (e.g., gradient information and loss information) to update its own local model and the models of VFL clients during the VFL training process after processing the received intermediate results (that may include convergence reports).

5. The VFL Server NWDAF sends the intermediate results to VFL Client NF(s).

6. The VFL server/client(s) update their local ML model based on the received information.

NOTE: Steps 2-6 should be repeated until the training termination condition (e.g., the maximum number of iterations or the loss function result is lower than a threshold) is reached.

7. The VFL Server NWDAF determines when the VFL training process terminates and informs the VFL Clients that the training ends.

6.X.2.3	Vertical Federated Inference if NWDAF is VFL Server

Figure 6.x.2.3-1 VFL inference if NWDAF is VFL Server

After VFL model training procedure as in clause 6.X.2.1 and 6.X.2.2, the VFL Serve NWDAF initiate the VFL inference procedure.

1-2. The VFL Server NWDAF triggers the VFL inference phase after receiving the subscription or request for analytics and delivers the analytics to the NF consumer. The VFL Server NWDAF selects VFL Client NF (s) and corresponding VFL correlation ID.

3. VFL Server NWDAF discoveries VFL client NFs and triggers data sample alignment and determines the final data sample as in clause 6.X.2.1.

4. To start VFL inference, VFL Server NWDAF sends a request to the selected VFL Client NF (s). The request includes data sample information (e.g., list of UE ID(s)) and VFL correlation ID that identifies the VFL process.

5. Each VFL Client NF (s) collects its local data using the current mechanism in clause 6.2 of TS 23.288 [5].

6. VFL Clients compute the intermediate results for their local ML models related to VFL correlation ID and provide reports with the intermediate results to the VFL Server NWDAF.

7. The VFL server aggregates intermediate results from VFL client(s), generates the final VFL inference result, and provides the analytics to the consumer NF, taking the final result into account.

END of CHANGES