SA WG2 Meeting #S2-164	S2-2408261

19 - 23 August, Maastricht, Netherlands	(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5551 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MWAB Configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | According to the FS_VMR2 conclusion (clause 8.2 of 23.700-06.), this CR proposes to add the MWAB Configuration to 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding MWAB Configuration clause. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MWAB Configuration is missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X.Y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.X.Y		Configuration of the MWAB

In order for an MWAB to operate as a Mobile gNB with Wireless Access Backhauling, it receives configuration from the OAM system of the MWAB owner PLMN as specified in TS 38.300 [27] and TS 38.401 [42]. The MWAB-UE establishes a secure and trusted connection to the OAM server only if it is authorized to operate as MWAB-UE in the serving PLMN.

In addition, the MWAB-UE is assumed to be configured with preferred PLMN lists and forbidden PLMNs by the HPLMN to perform PLMN selection as specified in TS 23.122 [17]. The MWAB-UE may be configured with parameters (i.e. DNN/S-NSSAI) which can be used for the Backhaul PDU session(s).

-	The MWAB can be (pre-)configured with UE policy or provisioned using existing UE Policy mechanism as defined in TS 23.503 [45] including the OAM access PDU session parameters for the authorized PLMNs.

The MWAB-gNB is locally configured with OAM server address (e.g. IP address, FQDN) of MWAB owner PLMN and credential to establish the secure and trusted connection. The OAM may configure the MWAB-gNB with AMF address(es), TAC/cell ID based on location of the MWAB and parameters needed for the MWAB-gNB, etc.

NOTE 1: 	The parameters needed for the MWAB-gNB is defined by RAN WG.

END OF CHANGES