3GPP TSG-SA WG2 Meeting #164	S2-2408267

# Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0554 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Data collection for AI/ML based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, OPPO,ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R19 FS_AIML_CN has concluded KI#1 for LCS supporting Direct AI/ML based positioning in TR 23.700-84. As for data collection, the LMF is enhanced to perform data collection based on the request from NWDAF or the internal trigger according to following conclusion principles. “Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. NOTE 1: When receiving the request for a UE location, the LMF selects an appropriate method to determine the UE location and may select AI/ML positioning as method. Principle #2: LMF is enhanced to perform model training for AI/ML based Positioning, the trigger for data collection and for model training in LMF is up to implementation. ” This contribution aims to provide procedure detail to achieve data collection for training an ML model for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add one procedure of data collection for AI/ML based positioning triggered by NWDAF - Add one procedure of data collection for AI/ML based positioning triggered by LMF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Data collection for AI/ML based positioning is not supported in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x(new), 6.x.1(new), 6.x.2(new), 6.x.3(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * * (All new text)

6.x	Procedure for AI/ML based positioning data collection

6.x.1	General

This clause specifies procedures for the data collection by LMF to train the ML model internally or in NWDAF containing MTLF for AI//ML based positioning. The data collection can be triggered by the LMF or the NWDAF containing MTLF.

Subclause 6.x.2 describes the procedure of LMF data collection for AI/ML based positioning based on NWDAF containing MTLF request.

Subclause 6.x.3 describes the procedure of LMF data collection for AI/ML based positioning based on the internal trigger.

6.x.2	LMF Data collection for AI/ML based positioning triggered by NWDAF containing MTLF

The figure 6.x.2-1 shows how LMF performs data collection for AI/ML based positioning triggered by NWDAF containing MTLF. The NWDAF containing MTLF triggers this LMF data collection for ML model training for AI/ML based positioning.

Figure 6.x.2-1: Procedure for AI/ML based positioning data collection triggered by NWDAF

0.	Based on the ML model provision subscription/request from the service consumer (e.g., LMF) or internal trigger, the NWDAF may determine to initiate the data collection to train the ML model for AI/ML based positioning.

1.	The NWDAF invokes the Ngmlc_Location_AIPosData_Subscribe service operation to the GMLC to collect data within the area of interest. The service operation includes the NWDAF ID, Notification Target Address (+Notification Correlation ID) pointing to the NWDAF itself, the area of interest, an indication that indicates the data is collected to train ML model for AI/ML based positioning, data source type (e.g. RAN, UE, PRU) and time window for data collection.

2.	The GMLC determines AMF based on the area of interest received in step 1. The GMLC invokes Namf_Location_AIPosData_Subscribe service operation to the AMF including the NWDAF ID, Notification Target Address (+Notification Correlation ID) and all other information received in step 1.

3.	The AMF selects one or more LMF(s) considering the area of interest, LMF Service Area, the LMF capabilities supporting AI positioning and data collection as defined in clause 5.1. The AMF may either query the NRF or based on AMF local configuration to discover and select a proper LMF.

4.	After receiving the request in step 2, the AMF determines the UEs for data collection considering the area of interest, LCS privacy, user consent, UE AI positioning capability and local configuration. The AMF also allocates the LCS correlation identifiers for each selected UE.

The AMF invokes the Nlmf_Location_AIPosData_Subscribe service operation to the LMF(s) selected in step 3 to initiate data collection. The service operation includes an indication that the data is collected for training ML model, a list of LCS correlation identifiers, the Notification Target Address (+Notification Correlation ID), NWDAF ID and all other information received in step 2.

5A.	If the requested data is not available at the LMF or ADRF, the LMF collects data for AI/ML based positioning from UE, PRU or NG-RAN node using LPP protocol and/or NRPPa protocol, based on the information received in step 6 and local policy.

Editor’s Note: The sources of data, specified data types and protocol used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs.

5B.		After data collection from UE, PRU or NR RAN, the LMF may store the data in the ADRF, using the Nadrf_DataManagement_StorageRequest service operation as defined in TS 23.288[37] .

6.	The LMF invokes the Nlmf_Location_AIPosData_Notify service operation towards the NWDAF based on Notification Target Address or NWDAF ID received in step 4. The service operation includes either the requested data, or the ADRF ID plus DataSetTag or Storage Transaction ID if the LMF stores the requested positioning data in step 5B.

When the ADRF ID plus DataSetTag or Storage Transaction ID are received, the NWDAF could obtain the data by invoking Nadrf_DataManagement_RetrievalRequest as defined in TS 23.288[37].

6.x.3	LMF Data collection for AI/ML based positioning based on internal trigger

The figure 6.x.3-1 shows how LMF performs data collection to train an ML model for AI/ML based positioning based on the internal trigger.

Figure 6.x.3-1: Procedure for AI/ML based positioning data collection triggered by LMF

1.	The LMF determines to collect data to locally train an ML model for AI/ML based positioning. The LMF determines the area of interest where the ML model is applied.

2.	The LMF invokes the Namf_Location_AIPosData_Subscribe service operation to the AMF including the area of interest, an indication that indicates the data is collected to train ML model for AI/ML based positioning, and data source type (e.g. RAN, UE, PRU), Notification Target Address (+Notification Correlation ID) pointing to the LMF itself or the LMF ID. The AMF determines the UEs for data collection considering the area of interest, UE AI positioning capability, LCS privacy, user consent and local configuration, and allocates the LCS correlation identifiers for each selected UE.

3.	The AMF invokes the Nlmf_Location_AIPosData_Notify service operation to LMF. The service operation includes an indication that the data is collected for training ML model, a list of LCS correlation identifiers, and optionally, serving cell identifiers used by LMF to determine the NG-RAN node(s) for data collection.

4.	The LMF collects data from UE, PRU or NG-RAN node via LPP protocol and/or NRPPa protocol by using the LCS correlation identifiers received in step 7.

Editor’s Note: The sources of data, specified data types and protocol used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs.

# * * * * End of changes * * * *