3GPP TSG-SA WG2 Meeting #164	S2-2408268

# Maastricht, Netherlands, 19 August – 23 August, 2024	(revision of S2-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1163 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF collects data from LMF in order to train ML model for AI positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-07-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | R19 FS_AIML_CN has concluded KI#1 for LCS supporting Direct AI/ML based positioning in TR 23.700-84. As for data collection, when the NWDAF receives the AIML positioning model training request from the consumer (e.g., LMF), the NDWAF can trigger the LMF to collect data in order to train an ML model for AI positioning. The interaction between the LMF and NWDAF needs to be specified. Currently, clause 6.2.12 of TS 23.288 captures the interaction between the NNWDAF and LCS system to collect location information, e.g., UE location, verlocity, positioning method and so on. It is supported the NWDAF utilizes Ngmlc service to collect the location information from LMF. Thus, the same interaction (i.e., via GMLC services) is proposed for data collection for training AIML positioning model. |  |  |  |  |  |  |  |  |
| Summary of change: |  | NWDAF collects positioning data from LMF via GMLC services in order to train ML model for AI positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NWDAF triggers the data collection for training AIML positioning model is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.12.1, 6.2.12.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.2.12	Data Collection using LCS

6.2.12.1	General

The NWDAF may collect location information for a target UE or a group of target UEs using LCS. The collected location related information can include:

‐	Location estimate of the UE in geographical coordinates and/or local coordinates expressed as a shape as defined in TS 23.032 [34] or local coordinate reference system;

-	Time stamp of location estimate;

-	Velocity of the UE as defined in TS 23.032 [34];

-	Information about the positioning method used to obtain the location estimate of the UE;

-	Indication of area event, when UE enters, is within or leaves the Geographical area;

-	Indication of motion event when UE moves by more than some predefined straight line distance from a previous location.

NOTE:	The location information that can be retrieved is defined within the location service response in clause 5.5 of TS 23.273 [39].

NWDAF shall use Ngmlc service as defined in TS 23.273 [39] to collect the location information using LCS. Only Mobile Terminated Location Request (MT-LR) is supported, including both Immediate Location Request and Deferred Location Request.

NWDAF may determine to query LCS system instead of AMF to obtain UE's location information based on the following attributes as received from NWDAF consumer:

-	Analytics ID (e.g. UE Mobility, QoS Sustainability, Relative Proximity, Movement Behaviour);

-	Preferred granularity of location information.

The NWDAF may collect measurement data from LMF to train an ML model for AI/ML based positioning. NWDAF utilizes Ngmlc services as defined in TS 23.273 [39] to collect data.

Editor’s Note: What data and which protocol used for model training, inference and model performance monitoring for AI/ML based positioning will be decided by RAN WGs, and SA WG2 will align with RAN WGs.

# * * * * Next change * * * *

6.2.12.2	Procedure for data collection using LCS

The interactions between NWDAF and LCS for data collection are illustrated in Figure 6.2.12.2-1. The data collected depends on the use cases. This figure is an abstraction of how NWDAF collects location information or measurement data for training AI positioning model using LCS. The actual procedures that NWDAF may use are as follows:

-	For a target UE, both 5GC-MT-LR procedure for the commercial location service as specified in clause 6.1.2 and deferred 5GC-MT-LR procedure as specified in clause 6.3 of TS 23.273 [39] can be utilized;

-	For a group of target UEs, bulk operation of LCS Service Request Targeting to Multiple UEs as specified in clause 6.8 of TS 23.273 [39] can be utilized;

-	For the area of interest, procedure for AI/ML based positioning data collection as specified in clause 6.x of TS 23.273 [39] can be utilized.

Figure 6.2.12.2-1: Data collection using LCS

1.	NWDAF requests the location information from GMLC about a target UE (that may be identified by a SUPI) or a group of target UEs (identified by a group ID) using Ngmlc_Location_ProvideLocation service.

The NWDAF requests the measurement data for training AI positioning model from GMLC about any UEs within an area of interest using Ngmlc_Location_AIPosData_Subscribe service.

2.	GMLC interacts within LCS, i.e. with AMF/LMF as described in TS 23.273 [39], to obtain the UE's location information or the measurement data for training AI positioning model. If privacy verification is required, GLMC will interact with UE via AMF before sending the location information to NWDAF.

3.1	If it is Immediate Location Request, GMLC sends the location service response including the location information for the target UE (or a group of target UEs) within a short time period as specified in clause 4.1a.4 of TS 23.273 [39] to the NWDAF.

3.2	If it is Deferred Location Request, GMLC sends the location service response including the indication of event occurrence and location information if requested for the target UE (or group of target UEs) at some future time (or times) as specified in clause 4.1a.5 of TS 23.273 [39] to the NWDAF.

3.3	If it used for collecting the measurement data for training AI positioning model, the GMLC sends the location service response including the requested measurement data.

# * * * * End of changes * * * *