SA WG2 Meeting #164 	S2-2408269

# 19 - 23 August 2024, Maastricht, Netherlands

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0555 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New procedure for LMF to retrieve ML Model from MTLF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, OPPO, China Telecom, ZTE, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As agreed in Conclusions for KI#1: Enhancements to LCS to support Direct AI/ML based Positioning of TR 23.700-84, the LMF could discovers a NWDAF containing MTLF and request an ML Model for AI/ML based positioning. Principle #1: The LMF is enhanced to perform location calculation based on ML model. The interaction between LMF and MTLF is only for model provisioning and possibly, based on coordination with RAN WGs, for data collection for training. - Principle #3.1: To retrieve a model to perform location calculation, the LMF discovers a suitable MTLF via NRF. Following the above conclusion, this CR proposes to add a new clause to introduce the procedure for how the LMF get the ML Model for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a new clause to introduce the procedure for how the LMF get the ML Model for AI/ML based positioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The procedure of how LMF get the ML Model for AI/ML based positioning is missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes (All new text) * * *

6.X	ML Model Retrieving by LMF

6.X.1	General

This clause specifies procedures for LMF to subscribe or request an ML Model from NWDAF containing MTLF.

LMF subscribes or requests an ML Model for AI/ML based positioning from NWDAF by using Nnwdaf_MLModelProvision services or Nnwdaf_MLModelInfo services, as described in 6.2A.1 and 6.2A.3 of TS 23.288 [37]. The LMF may use the retrieved ML Model information to perform inference and determine UE location information when the positioning method is AI/ML based positioning.

6.X.2	Procedure for LMF retrieving ML Model from NWDAF containing MTLF

The procedure in Figure 6.X.2-1 can be used by an LMF to retrieve an ML Model for AI/ML based positioning from NWDAF containing MTLF.

Figure 6.X.2-1: Procedure for ML Model retrieve from the NWDAF containing MTLF

0.	If LMF decides to use AI/ML based positioning to derive UE location, the LMF may determine to retrieve an ML model for AI/ML based positioning from NWDAF containing MTLF.

1.	To discover an NWDAF containing MTLF supporting ML Model training for AI/ML based positioning, the LMF queries the NRF by invoking the Nnrf_NFDiscovery_Request service operation (Indication of supporting ML Model training for AI/ML based positioning, Positioning case (Uplink positioning or Downlink positioning), ML Model filter information, NF consumer information (e.g. vendor ID of LMF)).

2.	The NRF notifies the LMF with one or more NWDAF containing MTLF instances.

3.	The LMF retrieves an ML model for AI/ML based positioning from the NWDAF containing MTLF by invoking Nnwdaf_MLModelProvision_Subscribe (or Nnwdaf_ MLModelInfo_Request) including AI/ML based positioning indication, ML Model filter information, optional Positioning case and other information as defined in TS 23.288 [37].

4.	When the trained ML model for AI/ML based positioning is available, the NWDAF containing MTLF sends the trained ML model information and optional Positioning case to the LMF by invoking Nnwdaf_MLModelProvision_Notify (or Nnwdaf_ MLModelInfo_Request Response) as defined in TS 23.288 [37].

Dependent on the different positioning case (e.g. Uplink positioning or Downlink positioning), the LMF will collect different measurement data from UE or RAN to perform model inference to obtain UE location.

* * * End of Changes * * *