3GPP TSG-WG SA2 Meeting #164 	S2-2408279

Aug. 19 – 23, 2024, Maastricht, NL	(revision of S2-240xxxx)

Source:	ZTE

Title:	KI#3: Conclusion update

Document for:	Approval

Agenda Item:	19.9.1

Work Item / Release:	FS_eEDGE_5GC_Ph3 / Rel-19

Abstract: This paper proposes conclusion update for KI#3.

1. Text Proposal

It is proposed to capture the following changes in TR 23.700-49.

# * * * * First change * * * *

8.3	Conclusion for KI#3

Interim conclusion:

-	The traffic routing rule between local DN and central DN should be controlled by SMF, which can be influenced by AF.

-	The requirement of establishing connectivity for traffic routing between local DN and central DN should be provided by AF.

-	QoS should be supported for the traffic routed between local DN and central DN.

-	Charging should be supported for the traffic routed between local DN and central DN, any charging issue should be verified with SA WG5.

-	GTP-U tunnels between the L-PSA UPF and PSA UPF are established for traffic routing between local DN and central DN.

-	For the traffic associated with UE(e.g. using UE IP address as source address or target address in the IP header), the GTP-U tunnel is established per PDU Session and may be established via UL CL UPF as described in Solution#17.

-	For the traffic not associated with the UE(e.g. not using the UE IP address in the IP header), the GTP-U tunnel is established per AF request as described in solution#21.

# * * * * End of changes * * * *