3GPP TSG-WG SA2 Meeting #164	S2-2408289

# Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5561 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Control Plane and User Plane Protocol stacks involving the MWAB node |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Lenovo, Huawei, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | VMR_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | It is agreed in TR 23.700-60’s conclusion that, - MWAB-gNB N2 interface with UE-AMF is provided over the IP connectivity offered by the PDU session of MWAB-UE. - MWAB-gNB N3 interface with UE-UPF is over the IP connectivity provided by the PDU session of MWAB-UE. However, the Control Plane and User Plane Protocol stacks involving the MWAB node are missing in TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the Control Plane Protocol stacks for MWAB-gNB N2 interface with UE-AMF and User Plane Protocol Stacks for MWAB-gNB N3 interface with UE-UPF into TS 23.501 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Control Plane and User Plane Protocol stacks involving the MWAB node can not be supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.x.y(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.x.y	Protocol Stacks for MWAB node

5.x.y.1 Control Plane Protocol Stacks to support N2 interface for MWAB node

Figure 5.x.y.1-1: Control Plane Protocol Stacks to support the N2 interface

Figure 5.x.y.1-1 illustrates the Control Plane Protocol Stacks to support the N2 interface. A PDU session between MWAB-UE and MWAB-UPF is established as a backhaul link to support the overlayed N2 interface. The N2 interface terminates at MWAB-gNB and UE-AMF respectively, which includes NGAP and SCTP/IP layer protocols between them.

5.x.y.2 User Plane Protocol Stacks to support N3 interface for MWAB node

Figure 5.x.y.2-1: User Plane Protocol Stacks to support the N3 interface

Figure 5.x.y.2-1 illustrates the User Plane Protocol Stacks to support the N3 interface. A PDU session between MWAB-UE and MWAB-UPF is established as a backhaul link to support the overlayed N3 interface. The N3 interface terminates at MWAB-gNB and UE-UPF, which includes GTP-U and UDP/IP layer protocols between them. A PDU session between UE and UE-UPF is realized by 1) the Uu between UE and MWAB-gNB, and 2) the N3 between MWAB-gNB and UE-UPF on top of the backhauling PDU session between MWAB-UE and MWAB-UPF.

# * * * End of changes * * * *