3GPP TSG-SA2 Meeting #164	S2-2408292

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0469 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | High level functionality and features and protocol stacks for 5G ProSe Multi-hop UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper covers the required updates and additions for the high level functionality and features and protocol stacks to support 5G ProSe Multi-hop Relay based on the agreed conclusions of the TR 23.700-03. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §5.2.3A: Add high level functionality description for 5G ProSe Multi-hop UE-to-Network Relay Discovery §5.4.5: Add high level functionality description for 5G ProSe Multi-hop Layer-3 UE-to-Network Relay §5.4.6: Add high level functionality description for 5G ProSe Multi-hop Intermediate UE-to-Network Relay §6.1.1.7.1A: Add control plane control plane stack for Multi-hop Layer-3 Relay with N3IWF §6.1.2.3.1A:Add user plane control plane stack for Multi-hop Layer-3 Relay with N3IWF |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | High level functionality and features and protocol stacks for supporting 5G ProSe Multi-hop Relay are not described |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.3A (new), 5.4.5 (new), 5.4.6 (new), 6.1.1.7.1A (new), 6.1.2.3.1A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

### 5.2.3A	5G ProSe Multi-hop UE-to-Network Relay Discovery

For 5G ProSe Multi-hop UE-to-Network Relay discovery, both Model A and Model B discovery are supported:

-	Model A uses a single discovery protocol message (Announcement) which is relayed by the Intermediate Relays. The Remote UE selects the Intermediate Relay based on the announcement message sent by the Intermediate Relay.

-	Model B uses two discovery protocol messages (Solicitation and Response) which are relayed by the Intermediate Relays. The Remote UE selects both the UE-to-Network Relay and the path to reach the UE-to-Network Relay.

For Relay Discovery Additional Information, only Model A discovery is used.

The 5G ProSe Direct Discovery message is extended with an indication that multi-hop relay is supported, along with the hop count and the maximum number of hops. The 5G ProSe Direct Discovery message may be dropped if the maximum number of hops is reached.

The procedures for 5G ProSe Multi-hop UE-to-Network Relay discovery are defined in clause 6.3.2.3A.

* * * * Next Change * * * *

### 5.4.5	5G ProSe Multi-hop Layer-3 UE-to-Network Relay

The 5G ProSe Multi-hop Layer-3 UE-to-Network Relay shall provide generic function that can relay any IP, Ethernet or Unstructured traffic for 5G ProSe Layer-3 Remote UEs transmitted via one or more Intermediate UE-to-Network Relay(s) towards the 5GS. PDU Session type towards 5GC selection is performed as described in clause 5.4.1.1.

* * * * Next Change * * * *

### 5.4.6	5G ProSe Intermediate UE-to-Network Relay

The 5G ProSe Intermediate UE-to-Network Relay shall provide generic function that can relay any type of traffic for 5G ProSe Layer-3 Remote UEs towards the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay. The end-to-end path between the 5G ProSe Layer-3 Remote UE and the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay may include more than one 5G ProSe Intermediate UE-to-Network Relay.

* * * * Next Change * * * *

##### 6.1.1.7.1A	5G ProSe Multi-hop Layer-3 UE-to-Network Relay

When N3IWF is supported by the 5G ProSe Multi-hop Layer-3 UE-to-Network Relay, the following control plane protocol stack apply.

Legend:

-	NAS, EAP-5G and IKEv2 between the Remote UE and the N3IWF are defined in clause 8.2.4 of TS 23.501 [4].

Figure 6.1.1.7.1A-1: Control plane protocol stacks between 5G ProSe Layer-3 Remote UE and N3IWF over 5G ProSe Layer-3 Intermediate Relay and Layer-3 UE-to-Network Relay before the signalling IPSec SA is established

Legend:

-	NAS, TCP and IPsec between the Remote UE and the N3IWF are defined in TS 23.501 [4] clause 8.2.4.

Figure 6.1.1.7.1A-2: Control plane protocol stacks between 5G ProSe Layer-3 Remote UE and N3IWF over 5G ProSe Layer-3 Intermediate Relay and Layer-3 UE-to-Network Relay after the signalling IPSec SA is established

* * * * Next Change * * * *

##### 6.1.2.3.1A	5G ProSe Multi-hop Layer-3 UE-to-Network Relay

Legend:

-	IPSec, Inner IP and GRE between the UE and the N3IWF are defined in TS 23.501 [4] clause 8.3.2.

Figure 6.1.2.3.1A-1 User plane protocol stacks between 5G ProSe Layer-3 Remote UE and N3IWF over 5G ProSe Layer-3 Intermediate Relay and Layer-3 UE-to-Network Relay after the signalling IPSec SA is established

* * * * End of Changes * * * *