3GPP TSG-SA2 Meeting #164	S2-2408294

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0471 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5G ProSe Communication via 5G ProSe Multi-hop Layer-3 UE-to-Network Relay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This paper includes the procedure for 5G ProSe Communication via 5G ProSe Multi-hop Layer-3 UE-to-Network Relay to support 5G ProSe Multi-hop Relay based on the agreed conclusions of the TR 23.700-03. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the procedure for 5G ProSe Communication via 5G ProSe Multi-hop Layer-3 UE-to-Network Relay |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5G ProSe Communication via 5G ProSe Multi-hop Layer-3 UE-to-Network Relay is not supported |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes * * * *

### 6.5.1A	5G ProSe Communication via multi-hop Layer-3 UE-to-Network Relay

#### 6.5.1A.1	5G ProSe Communication via multi-hop Layer-3 UE-to-Network Relay without N3IWF support

Figure 6.5.1A-1 depicts the procedure for 5G ProSe Communication via multi-hop Layer-3 UE-to-Network Relay without N3IWF support.

Figure 6.5.1A.1-1: 5G ProSe Communication via multi-hop 5G ProSe Layer-3 UE-to-Network Relay without N3IWF

1.	Service authorization and parameter provisioning are performed for the 5G ProSe Layer-3 UE-to-Network Relay (step 1a), the 5G ProSe Intermediate Relay(s) (step 1b) and the 5G ProSe Layer-3 Remote UE (step 1c).

2.	Similar to step 2 in clause 6.5.1.1 of TS 23.304 [4], the 5G ProSe Layer-3 UE-to-Network Relay may establish a PDU Session for relaying.

3.	The 5G ProSe Layer-3 Remote UE performs the (re-)discovery of a 5G ProSe Layer-3 UE-to-Network Relay and multi-hop path as described in clause 6.3.2.3A. As part of the discovery procedure the Layer-3 Remote UE learns about the connectivity service the Layer-3 UE-to-Network Relay provides. For Model B Discovery, the Layer-3 Remote UE obtains the User Info ID of Layer-3 UE-to-Network Relay and an ordered list of User Info IDs of Intermediate Relays during the discovery procedure. If no multi-hop path is discovered, the 5G ProSe Communication cannot be established and the other steps are skipped.

4a.	Once the 5G ProSe UE-to-Network Relay and the multi-hop path have been selected by the Remote UE, the Remote UE sends a multi-hop Communication Request to the 5G ProSe UE-to-Network Relay.

For Model B discovery, the Request includes information about the selected path (i.e. ordered list of User Info IDs of 5G ProSe Intermediate Relay(s)), optionally a path ID and (optional) the requested PC5 QoS information. The path ID may refer to the list of User Info IDs of Intermediate Relay in the DCR message.

Each Intermediate Relay forwards the received multi-hop Communication Request to the next Intermediate Relay or the UE-to-Network Relay based on the selected path information. The Communication Request from the Intermediate Relay may include per-hop QoS information or cumulative QoS information (e.g. cumulative delay between Remote UE up to the Intermediate Relay).

The 5G ProSe UE-to-Network Relay responds with a multi-hop Communication Accept message which includes the optional path ID for Model B discovery. The path ID is generated and managed by the Remote UE and is associated with the UE-to-Network Relay and/or referred to the reversed list of User Info IDs of Intermediate Relay in the DCA message.

It is assumed an Intermediate Relay sends the DCR message to the next Intermediate Relay only if the security association is established with the previous Intermediate Relay.

5.	For IP PDU Session Type and IP traffic over PC5 reference point, IPv6 prefix or IPv4 address (including NAT case) may be allocated for the 5G ProSe Layer-3 Remote UE by the UE-to-Network Relay to Intermediate Relay.

-	Option1: the UE-to-Network Relay assigns IP address for Remote UE and all the intermediate Relays in the path, e.g. based on the User Info ID list included in the DCR message, assign for each node.

-	Option2: the UE-to-Network Relay and Intermediate Relay assigns IP address to the next hop.

6.	The complete UE-to-Network multi-hop path is established and 5G ProSe Direct Communication is established between the 5G ProSe-enabled UEs for each hop. If there is no PDU Session associated with the Relay Service Code or a new PDU Session for relaying is needed, the 5G ProSe Layer-3 UE-to-Network Relay may establish a new PDU Session for relaying.

7.	Steps 5 to 7 as described in clause 6.5.1.1 of TS 23.304 [4] with the following update:

-	When the UE-to-Network Relay determines the QoS parameters for PC5 QoS flows, it considers the requested PC5 QoS info from the Remote UE and the cumulative delay between UE-to-Network relay and Remote UE via the Intermediate Relays.

-	Each Intermediate Relay determines per hop QoS depending on its position in the path.

The IP address of Remote UE is allocated by the UE-to-Network Relay and the mechanism defined in clause 5.5.1.3 of TS 23.304 [4] is used and the IPv4 DHCP messages/IPv6 Router messages are routed via the Intermediate Relays.

If the PDU Session for relaying is released by the UE-to-Network Relay or the network as described in clause 4.3.4 of TS 23.502, the UE-to-Network Relay and 5G ProSe Intermediate Relay(s) should initiate the release of the layer-2 links associated with the released PDU Session using the procedure defined in clause 6.4.3.3 of TS 23.304 [4].

The PDU Session(s) used for relaying should be released as described in clause 4.3.4 of TS 23.502 [12] (e.g. by 5G ProSe Layer-3 UE-to-Network Relay), if the service authorization for acting as a 5G ProSe Layer-3 UE-to-Network Relay in the serving PLMN is revoked.

The 5G ProSe Layer-3 UE-to-Network Relay may send the Remote UE Report message when the 5G ProSe Layer-3 Remote UE or any 5G ProSe Intermediate Relay disconnects from the multi-hop 5G ProSe Layer-3 UE-to-Network Relay chain, to inform the SMF that the 5G ProSe Layer-3 Remote UE(s) have left.

When the Remote UE needs to switch to a new Intermediate Relay (e.g. due to link failure), the Remote UE re-selects a new path terminated at the same UE-to-Network Relay and allocates a new path ID. The new path ID is sent to the UE-to-Network Relay and the older path is released.

NOTE:	The security aspects of multi-hop UE-to-Network path establishment will be addressed by SA WG3.

After 5G ProSe Indirect Relay Discovery, when a 5G ProSe Layer-3 Remote UE is using discovered 5G ProSe Intermediate Relay UE(s) and 5G ProSe Layer-3 UE-to-Network Relay without involving N3IWF, the PCF based provisioning and update of 5G ProSe Policy/parameters to the 5G ProSe Layer-3 Remote UE are not supported

#### 6.5.1A.2	5G ProSe Communication via multi-hop Layer-3 UE-to-Network Relay with N3IWF support

Intermediate Relay UE can connect to N3IWF according to clause 6.5.1.2 of TS 23.304 [4] and provides the relay function to Remote UE.

Figure 6.5.1A.2-1: 5G ProSe Communication via multi-hop 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support

1.	The 5G ProSe Layer-3 UE-to-Network Relay, the Remote UE and the 5G ProSe Layer-3 Intermediate Relay UE perform service authorization and parameter provisioning.

Supporting of the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF is preconfigured or provisioned to the Intermediate Relay by the ProSe Policy.

2.	The 5G ProSe Layer-3 Remote UE performs the (re-)discovery of a 5G ProSe Layer-3 UE-to-Network Relay and multi-hop path as described in clause 6.3.2.3A using the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF.

3.	A 5G ProSe Layer-3 UE-to-Network Relay and 5G ProSe Layer-3 Remote UE follow the procedures described in steps 4-5 in clause 6.5.1A.1 for communication establishment using the RSC configured for making the 5G ProSe Layer-3 Remote UE access to 5GC via N3IWF.

4.	IP address and prefix allocation.

5.	The 5G ProSe Layer-3 Remote UE that connects to a 5G ProSe Layer-3 UE-to-Network Relay with N3IWF support selects an N3IWF and determines the N3IWF IP address.

6.	The 5G ProSe Layer-3 Remote UE establishes a signalling IPsec tunnel using IKE procedures with a N3IWF via Intermediate Relays and UE-to-Network Relay performs NAS Registration.

7.	Based on Additional QoS Information received from the N3IWF, the 5G ProSe Layer-3 Remote UE determines whether it is necessary to request for QoS session modification for the dedicated QoS Flows toward the 5G ProSe Layer-3 UE-to-Network Relay, taking the number of hops into account.

After the procedure, both 5G ProSe Indirect Relay Communication and 5G ProSe Direction Communication between remote UE, Intermediate Relay UE(s) and UE to Network Relay UE are enabled. UE(s) can communicate each other directly or indirectly for the purpose of maintenance like heartbeat, etc.

For N3IWF of 5G ProSe Layer-3 Remote UE case, IKE keep alive(s) between the 5G ProSe Layer-3 Remote UE and the N3IWF are used for detecting possible path failure. The 5G ProSe Layer-3 Remote UE may change 5G ProSe Intermediate Relay UE(s) and 5G ProSe Layer-3 UE-to-Network Relay(s) while maintain the session with the N3IWF when the 5G ProSe Layer-3 Remote UE and the N3IWF support MOBIKE. This is negotiated between the 5G ProSe Layer-3 Remote UE and the N3IWF as specified in clause 4.12.2.2 of TS 23.502 [12]).

When IKE keep alive(s) are used:

1.	5G ProSe Layer-3 Remote UE needs to keep the PC5 connection with 5G ProSe Intermediate Relay UE(s)

2.	5G ProSe Intermediate Relay UE(s) keep the intermediate PC5 connections between 5G ProSe Layer-3 Remote UE and 5G ProSe Layer-3 UE-to-Network Relay

3.	5G ProSe Layer-3 UE-to-Network Relay keeps the PDU Session.

QoS differentiation can be provided on per-IPsec Child Security Association basis. When accessing 5GS via 5G ProSe Intermediate Relay UE(s) and 5G ProSe Layer-3 UE-to-Network Relay with N3IWF, the 5G ProSe Layer-3 Remote UE can request for PDU Session establishment or handover an existing PDU session to the N3IWF using UE requested PDU Session Establishment procedure defined in clause 4.12.5 of TS 23.502 [12].

* * * * End of Changes * * * *