3GPP TSG-WG SA2 Meeting #164	S2-2408316

# Maastricht, NL, 19th Aug – 23rd Aug, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1167 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF Discovery for VFL. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As described in conclusion for KI#2: 5GC support for Vertical Federated Learning, 5GC shall support VFL entities registration and discovery. P#2.2.1 The NWDAF as VFL client shall register to NRF with NF profile including VFL capability information (VFL capability type (i.e. VFL Clients) ). For an untrusted AF as the VFL client, the NEF registers based on configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 of TS 23.288 [5] and includes as part of the information about the AF an VFL capability information (VFL capability type (i.e. VFL Clients) ). P#2.2.2 The NWDAF as VFL server will select NWDAF(s) as candidate VFL client(s) and /or AF(s) (via NEF profile for untrusted AF) from NRF for VFL training process. P#2.2.3 The AF (via NEF in case of untrusted AF) as VFL server will select candidate NWDAF(s) as VFL client(s) from NRF for VFL training process. NOTE 1: Whether and how Vendor specific feature information in P#2.2.1 is included will be determined in the normative phase. Considering the untrusted AF is come from 3rd party, the vendor information of untrusted AF is complex and there are multi-vendors that the untrusted AF used. The vendor of the AF does not make sense as the application service provider could be different. It is proposed to use Vertical Federal learning Group Identifier which represents the Vertical Federal learning capability group. Each VFL entites can support multiple VFL Group ID(s). |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add VFL group ID in the procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The FL server could not select the VFL entities which can collaborate in VFL tasks. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2X(new), 6.2X.1(new), 6.2X.2(new), 6.2X.2.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes * * * *

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML Model Information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF containing AnLF using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

-	If the analytics are related to UE(s) and if NWDAF instances indicate weights for TAIs in their NF profile (see clause 6.3.13 of TS 23.501 [2]), the NWDAF service consumer may use the weights for TAIs to decide which NWDAF to select.

-	If the NWDAF service consumer needs to discover an NWDAF containing an AnLF with analytics accuracy checking capability, the consumer may query NRF providing also the analytics accuracy checking capability in the discovery request.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types or to collect data from particular NWDAF Serving Area, the consumer may query NRF providing the NF Set IDs or NF types or Area of Interest in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources or NWDAF Serving Area information as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF containing MTLF via NRF:

-	When one or more trained ML Models are available for one or more Analytics ID(s) the NWDAF containing MTLF shall include the Analytics ID(s) that is(are) supported per service in the registration towards NRF. The NWDAF containing MTLF may wait to register in NRF the above services until at least one trained model is available. The NWDAF containing MTLF may provide to the NRF a list of Analytics IDs corresponding to the trained ML Models and possibly the ML Model Filter Information for the trained ML Model per Analytics ID(s), if available. In this Release of the specification, only the S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information for the trained ML Model per Analytics ID(s) may be registered into the NRF during the NWDAF containing MTLF registration. If the NWDAF containing MTLF supports ML Model interoperability, the NWDAF containing MTLF includes, in the registration to the NRF, an ML Model Interoperability indicator for each Analytics ID.

-	The ML Model Interoperability indicator comprises a list of NWDAF providers (vendors) that are allowed to retrieve ML Models from this NWDAF containing MTLF. It also indicates that the NWDAF containing MTLF supports the interoperable ML Models requested by the NWDAFs from the vendors in the list.

NOTE 4:	The S-NSSAI(s) and Area(s) of Interest from the ML Model Filter Information are within the indicated S-NSSAI and NWDAF Serving Area information in the NF profile of the NWDAF containing MTLF, respectively.

-	During the discovery of NWDAF containing MTLF, a consumer (e.g. an NWDAF containing AnLF, an NWDAF containing MTLF as FL server or FL client) may include in the request the target NF type (i.e. NWDAF), the Analytics ID(s), the S-NSSAI(s), Area(s) of Interest of the Trained ML Model required, ML Model Interoperability indicator and NF consumer information. The NRF returns one or more candidate instances of NWDAF containing MTLF to the NF consumer and each candidate instance of NWDAF containing MTLF includes the Analytics ID(s), possibly the ML Model Filter Information for the available trained ML Models and ML Model Interoperability indicator, if available.

NOTE 5:	NF consumer information such as Vendor ID is defined in stage 3.

-	If the NWDAF service consumer needs to discover an NWDAF containing an MTLF with ML Model accuracy checking capability, the consumer may query NRF also providing the ML Model accuracy checking capability in the discovery request.

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. FL server) and may include Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include FL capability type (i.e. FL client) and may include Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 6:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as FL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as FL client and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

In order to discover an VFL entites (e.g, NWDAF, Trusted AF, Untrusted AF with Verticle Federated Learning (VFL) capability) via NRF:

-	An NWDAF supporting VFL capability shall also include the indication that indicate the NWDAF contain the   AnLF and MTLF during the registration in NRF. If the NWDAF supports Verticle Federated Learning (VFL) Group Identifier, the NWDAF includes, in the registration to the NRF, list of Verticle Federated Learning (VFL) Group ID(s).

-	Verticle Federated Learning (VFL) Group Identifier identifies the Verticle Federated Learning (VFL) Capability Group. Each VFL entites can support multiple VFL Group ID(s). Entities sharing the same VFL Group ID can collaborate in VFL tasks. These IDs could be pre-configured by the OAM, and each VFL entites shall register its supported VFL Group ID(s) in the NRF.

-	An NWDAF supporting VFL as a client shall additionally include VFL capability type (i.e., VFL client) as VFL capability information during the registration in NRF

NOTE 6a:	An NWDAF containing MTLF may indicate to support both VFL server and VFL client in the FL capability for specific Analytics ID.

-	During the discovery of NWDAF containing MTLF as VFL client, a consumer (e.g., a VFL server) may include in the request VFL capability type as VFL client. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

Editor's note:	Whether VFL capability type as VFL server are also needed to be registered in NRF is FFS.

NOTE 7:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF in this Release.A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

# * * * * Next changes * * * *

6.2X	Verticle Federated Learning among Multiple NWDAFs and AF(s)

6.2X.1	General

This clause specifcs how NWDAF(s) and AF(s) can leverage Verticle Federated technique to train ML models.

6.2X.2	Procedures

6.2X.2.1 	Registration and Discovery procedure for Verticle Rederated Learning

NWDAF

(VFL server)

NRF

NEF

Untrusted AF

(VFL Client)

NWDAF/Trusted AF

(VFL Client)

1c. Nnef_AFManagement

_AFRegister

(AF Profile)

1d. Nnrf_NFManagement

_NFRegister/update

Request (NEF profile)

1b. Nnrf_NFManagement_NFRegister

2. store VFL members profiles

1a. Nnrf_NFManagement_NFRegister Request

3a. Nnrf_NFManagement_NFRegister Response

3b. Nnrf_NFManagement_NFRegister response

3d. Nnrf_NFManagement_NFRegister

Response

3e. Nnef_AFManagement

_AFRegister

Response

4. Nnrf_NFDiscovery_Request

5. Authorize NF Service Discovery

6. Nnrf_NFDiscovery_Request Response

Step1 to 4 are VFL entities registration procedure.

1a.	VFL Server (i.e, NWDAF containing MTLF and AnLF, Trusted AF) registers to NRF with its NF profile, which   includes Verticle Federated Learning (VFL) Group Identifier(s), Time interval supporting VFL as described in clause 5.2.

1b.	VFL Client (i.e, NWDAF containing MTLF and AnLF, Trusted AF) registers to NRF with its NF profile, which includes VFL capability type information (i.e., VFL Client), Verticle Federated Learning (VFL) Group Identifier(s), Time interval supporting VFL as described in clause 5.2.

1c.	Untrusted AF(s) with VFL capability registers to NRF via NEF NF profile. Untrusted AF registers its AF information to NEF, which includes VFL capability type information (i.e., VFL Server, VFL Client), Verticle Federated Learning (VFL) Group Identifier(s), Time interval supporting VFL as described in clause 5.2.

1d.	NEF registers to NRF with its NF profile, which includes the AF information received in step 1c.

2. NRF stores the NF profile of VFL members.

3a-3d.	The NRF acknowledge NF Registration is accepted via Nnrf_NFManagement_NFRegister response.

3e. The NEF acknowledge AF Registration is accepted via Nnef_AFManagement_AFRegister response.

Steps 4 to 6 are the VFL entities Discovery procedure.

4.	When the VFL Server determines Vertical Federal Learning is needed, the VFL Server discovers the VFL entities from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The message may include Analytic ID(s), NF type(s), VFL Group Identifier(s), VFL capability Type (i.e, VFL client), Time Period of Interest.

5.	The NRF authorizes the NF discovery request and determines a set of VFL entities. The following criteria might be used: Analytic ID, NF type(s), VFL capability Type (i.e., VFL client), VFL Group Identifier(s) of VFL entities.

6.	The NRF returns discovered VFL entities (e.g, NWDAF(s), Trusted AF(s) as VFL clients). If the VFL entities contain the Untrusted AF(s), the NRF may also return the NEF which contains Untrusted AF(s) with VFL capability Type.