3GPP TSG-WG SA2 Meeting #164	S2-2408326

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	CATT

Title:	Initial conclusions for DualSteer KI#1.1

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract: Initial conclusions for DualSteer KI#1.1.

- Introduction

This paper provides an initial conclusion for KI#1.1.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54 1.0.0.

# * * * * First change * * * *

8.1	Conclusions for DualSteer

8.1.1	Conclusion for KI#1.1: Subscription aspects to support DualSteer

The following principles are proposed to be considered in normative work:

-	UDM/UDR enables to associate the two SUPIs applying to DualSteer service in the user subscription data for each SUPI.

-	DualSteer Authorization shall be included in the user subscription data for each SUPI to indicate whether the SUPI is authorized to use the DualSteer service.

-	The dedicated Access Type/RAT restriction for the DualSteer service shall be included in the AM subscription data of each SUPI.

NOTE:	The RAT selection may need coordination with SA1/CT1.

-	Which SUPIs can be identified as first/primary SUPI in the user subscription data is up to the operator’s polices.

# * * * * End of changes * * * *