3GPP TSG-WG SA2 Meeting #164	S2-2408328

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	CATT

Title:	Update solution#1.7 for subscription update for DualSteer Service

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract: This document proposes to update solution#1.7 for updating the subscription for DualSteer Service.

1. Discussion

This contribution proposes to update solution#1.7 to indicate how the UDM/PCF updating the subscription data for DualSteer Service when one of SUPI’s subscription data updates.

According to the SA1 TS 22.261[2], the SUPI1 and SUPI2 belonging to one DualSteer Device may share some same subscription data. If the shared same subscription data is changed for one of SUPIs, the other SUPI should also be triggered to update the changed shared subscription data.

When the AM and/or SM subscription data is shared and updated for one of SUPIs, the UDM will trigger the related subscription update procedure (e.g. UCU) to inform the serving AMF(s)/SMF(s) the subscription data for both of SUPI1 and SUPI2 have been update.

When the policy subscription data (e.g., UE policy, SM policy) is shared and update for one of SUPIs, the PCF will inform the serving AMF(s)/SMF(s) to trigger the related policy notification procedure to inform both of SUPI1 and SUPI2 the policy subscription have been update.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-54 V0.3.0.

* * * * Start of Changes * * * *

6.1.7	Solution #1.7: The UE subscription data for DualSteer in the UDM/UDR

6.1.7.1	Description for provisioning subscription data for DualSteer

This solution addresses the Key Issue #1.1: Subscription aspects to support DualSteer.

This solution shows how the UDM/UDR identifies and associates the two subscriptions/SUPIs for the DualSteer Device in the UE subscription data.

This solution is based on the following assumptions:

-	The DualSteer Device has two UEs which identified by SUPI1 and SUPI2 separately.

The following table 6.1.7.1-1 shows the example of new added “DualSteer Subscription data” in the UDM/UDR for the DualSteer Device.

Table 6.1.7.1-1 Example of DualSteer Subscription data

| DualSteer Subscription data | List of SUPIs | DualSteer Authorization | Associated SUPI/IMSI | Access type/RAT restriction |
| --- | --- | --- | --- | --- |
|  | SUPI 1 | True | SUPI 2 |  |
|  | SUPI 2 | True | SUPI 1 |  |

The above parameters in the table 6.1.7.1-1 are described as follows:

-	List of SUPIs: List of identifiers for the UE which can be identified by SUPI/IMSI.

-	DualSteer Authorization: Indicate whether the UE is authorized to use the DualSteer service.

-	Associated SUPI/IMSI: Indicate the associated UE identifier sharing a DualSteer device, which can be identified by SUPI/IMSI.

-	Access Type/RAT restriction: Indicate which Access Type/RAT cannot be accessed by the DualSteer device, only if UE acts as part of DualSteer Device and is authorized to use the DualSteer function. Otherwise the existing "RAT restriction" is used.

After provisioning the DualSteer Subscription data in the UE subscription data, the UDM in the 5GS can identify and associate the two subscriptions/SUPIs for the DualSteer Device if it has two single UEs.

NOTE:	This table only includes access and mobility related subscription data.

6.1.7.2	Description for updating subscription data for DualSteer

According to the SA1 requirements in TS 22.261[2], the SUPI1 and SUPI2 applying to DualSteer service may share some same subscription data. If the shared same subscription data is changed for one of SUPIs, the other SUPI should also be triggered to update the changed shared subscription data to keep the consistency.

When the AM and/or SM subscription data is shared and updated for one of SUPIs, the UDM will trigger the related subscription update procedure (e.g. UCU) to inform the new subscription data to the serving AMF(s)/SMF(s) for both of SUPI1 and SUPI2.

When the policy subscription data (e.g., UE policy, SM policy) is shared and updated for one of SUPIs, the PCF will inform the serving AMF(s)/SMF(s) to trigger the related policy notification procedure to update the new policy subscription for the SUPI1 and SUPI2.

6.1.7.3	Procedures

The procedure of AMF/SMF retrieving the UE subscription data from the UDM as defined in clause 4.2.2.2 and clause of 4.3.2.2 of TS 23.502 [4] can be reused for clause 6.1.7.1.

The procedure of UE Configuration Update for access and mobility management as defined in clause 4.2.4.2 of TS 23.502[4] can be reused when the shared AM subscription data is updated for DualSteer Service as described in clause 6.1.7.2. The enhancement is UDM needs to trigger the UCU procedure for SUPI1 and SUPI2 respectively when identified they apply to DualSteer Service and one of SUPI’s shared AM subscription data has been updated.

The procedure of network requested PDU Session Modification as defined in clause 4.3.3.2 of TS 23.502[4] can be reused when the shared SM subscription data is updated for DualSteer Service as described in clause 6.1.7.2. The enhancement is UDM needs to trigger the Nudm_SDM_Notification request for SUPI1 and SUPI2 respectively when identified they apply to DualSteer Service and one of SUPI’s shared SM subscription data has been updated.

The procedure of UE Configuration Update procedure for transparent UE Policy delivery as defined in clause 4.2.4.3 of TS 23.502[4] can be reused when the shared UE policy is updated for DualSteer Service as described in clause 6.1.7.2. The enhancement is PCF needs to trigger the UCU procedure for SUPI1 and SUPI2 respectively when identified they apply to DualSteer Service and one of SUPI’s shared UE policy has been updated.

The procedure of network requested PDU Session Modification as defined in clause 4.2.4.3 of TS 23.502[4] can be reused when the shared SM policy is updated for DualSteer Service as described in clause 6.1.7.2. The enhancement is PCF needs to trigger the SM Policy Association Modification procedure for SUPI1 and SUPI2 respectively when identified they apply to DualSteer Service and one of SUPI’s shared SM policy has been updated.

6.1.7.4	Impacts on services, entities and interfaces

The solution impacts the following network functions:

UDM/UDR:

-	Provision DualSteer Subscription data to identify and associate the two subscriptions/SUPIs for DualSteer Device.

-	Trigger the related subscription update procedure to inform the new subscription data to the serving AMF(s)/SMF(s) for both of SUPI1 and SUPI2.

UE:

-	The USIM profile should also be configured to align with the DualSteer Subscription data in the UDM/UDR.

AMF:

-	Control the UE access type/RAT based on the subscribed data if it is acts as part of DualSteer Device and is authorized to use the DualSteer service.

PCF:

-	Trigger the related policy notification procedure to update the new policy subscription for the SUPI1 and SUPI2.

* * * * End of Changes * * * *