3GPP TSG-WG SA2 Meeting #164	S2-2408336

# Maastricht, Netherlands, Aug 19 – 23, 2024		(revision of S2-240xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5565 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UPF event exposure during UPF relocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, SK Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UPEAS_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As concluded in clause 8.2 of TR 23.700-63: - NEF/NWDAF can use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for data collection. - For periodic reporting type of the UPF event exposure service with direct notifications when the UPF changes, the source UPF may report to the consumer any data collected between the time instance of the last notification to the consumer and the time instance of the UPF change. - When the PDU session is released, if the UPF event subscription is via the SMF, the SMF may send a notification of subscription termination. Based on the above conclusions for KI#2, the UPF event exposure can be enhanced as follows: 1. UPF event consumer discovers PSA UPF by using UE IP address and subscribes directly to the PSA UPF: The NWDAF may need to collect User Data Usage Measures (e.g., number of packet transmission/ retransmission) for Service Experience analytics based on analytics subscription/request from AF/NEF, and the Target of Analytics Reporting may contain a single UE, as described in clause 6.4 of TS 23.288. Therefore, NF consumer (e.g. NWDAF) may subscribe to the UPF Event Exposure service directly for data collected for a single UE, if the subscription is for User Data Usage Measures or User Data Usage Trends and the NF consumer can obtain the IP address of the UE, the NF consumer can use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for the data collection. 2. Source UPF sends the collected data to the consumer when UPF changes: The source UPF may have collected some data but not sent to the consumer yet when the UPF changes, and the consumer can decide how to handle this data, e.g., discard the collected data or send the collected data to the consumer. For example, when the consumer subscribes to the UPF event via SMF, the consumer can include a relocation reporting indication, which indicates how to handle the data collected by the source UPF when UPF relocation occurs, and when the UPF changes, the SMF can include in the unsubscription request a last notification indication which indicates how the collected data should be handled when receiving the last notification indication. The source UPF sends the collected data to the UPF event consumer if the last notification indication from SMF indicates that the source UPF should send the collected data to the UPF event consumer. 3. Send a notification with a subscription termination indication to the UPF event consumer when UPF changes: The AMF can inform the consumer NF that the subscription is terminated at the AMF, and the AMF can also indicate the reason for the subscription termination, as specified in TS 29.518. For the UPF event exposure, the subscription to UPF event may also be terminated, e.g., due to UPF relocation. However, currently, the SMF/UPF cannot indicate the subscription termination to the consumer. Therefore, the SMF/UPF event notification can be enhanced to include a subscription termination indication to indicate that the subscription to the UPF event has been terminated, and may also include a cause code indicating the reason for subscription termination, e.g., UPF relocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description that the UPF event consumer subscribes directly to the PSA UPF. Add the description that source UPF sends the collected data to the consumer and SMF/UPF sends a notification with a subscription termination indication and an optional cause code to the UPF event consumer when UPF changes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The enhancements on UPF event exposure during UPF relocation are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.8.2.17	Data exposure via Service Based interface

The UPF may expose information by means of UPF Event Exposure service as described in TS 23.502 [3] clause 5.2.26.2, via a service-based interface directly. The NF consumers, which may receive UPF event notifications, are AF/NEF, TSNAF/TSCTSF and NWDAF/DCCF/MFAF.

When the UPF supports the data exposure via the service based interface, it may register its NF profile to the NRF including the UPF Event Exposure services and the related Event ID(s).

For data collection from UPF (see clause 4.15.4.5 of TS 23.502 [3]), NF consumers can do the subscription to the UPF either directly or indirectly via SMF. An NF consumer may subscribe to the UPF Event Exposure service directly for data collected for "any UE", e.g. to collect user data usage information for NWDAF NF Load analytics (see clause 6.5 of TS 23.288 [86]) and if the subscription is not including any of the following parameters: AoI, BSSID/SSID and DNAI. An NF consumer may subscribe to the UPF Event Exposure service directly for data collected for a single UE, if the subscription is for User Data Usage Measures or User Data Usage Trends and the NF consumer can obtain the IP address of the UE, and the subscription is not including any of the following parameters: AoI, BSSID/SSID and DNAI, e.g., NWDAF collects user data usage information for Service Experience analytics based on analytics subscription/request from AF/NEF (see clause 6.4 of TS 23.288 [86]), the NF consumer can use UE IP address to discover the PSA UPF and subscribe directly to PSA UPF for the data collection. Otherwise, the NF consumer shall subscribe indirectly via SMF.

When the UPF changes for the scenario of UPF relocation, if the UPF event subscription for a UE is via the SMF, the SMF will send a notification with a subscription termination indication and an optional cause code to the UPF event consumer; if the UPF event subscription for a UE is directly sent to the UPF, the UPF will send a notification with a subscription termination indication and an optional cause code to the UPF event consumer. In addition, the source UPF may report to the consumer any data that has been collected but not sent to the consumer yet when the UPF changes (see clause 4.15.4.5 of TS 23.502 [3]).

To alleviate the load of UPF due to frequent event notification for data collection related events, the event subscription may include Reporting suggestion information. The Reporting suggestion information includes Report urgency and Reporting window information. Reporting urgency information represents whether this event report can be delay tolerant, i.e. the event report can be delayed. If the Reporting urgency information indicates "delay tolerant", the Reporting window is also provided, which defines the last valid reporting time, and UPF shall report the event before that time. If the Reporting suggestion information allows this, the UPF can concatenate several event reports (of the same event) to the same notification endpoint into one notification message.

The UPF may also expose UE information by means of the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in TS 23.502 [3] clause 5.2.26.3. An UPF which is deployed with NAPT (Network Address Port Translation) functionality may support to provide the 5GC UE IP address to NEF based on NEF request containing public IP address and port number using the Nupf_GetUEPrivateIPaddrAndIdentifiers service as described in clause 4.15.10 of TS 23.502 [3] for AF specific UE ID retrieval.

# * * * * End of changes * * * *