3GPP TSG-WG SA2 Meeting #164	S2-2408342

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	End-to-end solution for DualSteer

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_DualSteer / Rel-19

Abstract: An end-to-end solution is proposed for DualSteer to reach consensus on the conclusion.

# 1. Introduction/Discussion

This paper provides an End-to-End solution for the DualSteer. Compared with the proposed solution in the TR from our side, the differences are summarized below:

-	Not mandatory to use Primary SUPI and Secondary SUPI. This is the possible option. If there is no Primary/Secondary SUPI concept, each 3GPP access will be identified by a path identifier provided by UDM.

-	Add a new policy about when to establish the associated PDU session of the source PDU session for DualSteer traffic switching. If the policy allows the DualSteer Device to pre-establish the associated PDU session for DualSteer traffic switching in advance, the SMF shall deactivate the newly established PDU session. UE sends service request to perform the DualSteer traffic switching and SMF activates the UP of the target PDU session and release/deactivate the source PDU session based on operator policy.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54.

# * * * * First change (all new text below) * * * *

### 6.1.X	Solution #1.X: E2E solution to support DualSteer

#### 6.1.X.1	Solution Assumption

This solution supports the following assumptions:

##### 6.1.X.1.1	Assumption for Subscription and Registration

Assumption 1: It is not necessary to translate the service requirement regarding the "one subscription profile" into a new table in the UDM. A pointer to the other record based on SUPI is likely to be sufficient. The creation of a separate table needs to be well justified.

Assumption 2: One SUPI can be marked as Primary SUPI and the other SUPI is marked as Secondary SUPI, if operator would like to provide asymmetrical policies to each SUPI and have a differentiated handling according to policies. Alternatively, two SUPIs can be identified by a path identifier respectively in order to use the path identifier in the policies for DualSteer to indicate the corresponding SUPI if needed. The SUPI role or path identifier is independent with the order of registration of the two SUPIs.

Assumption 3: The UDM record corresponding to each SUPI contains a reference to the other SUPI for the 5GS to be able to correlate the two SUPIs of the DualSteer device and properly enforce the policies.

Assumption 4a: The UE(s) of the DualSteer device need to be aware of their own capabilities (e.g. whether they are single Dual-USIM DualSteer UE of DualSteer device or two separate Single-USIM DualSteer UEs of DualSteer device).

Assumption 4b: Based on observation 3, the UE(s) of the DualSteer device need to be aware whether a given USIM belongs to a DualSteer device subscription, and what is the linked USIM for that subscription, including the case for which the other USIM is under control of another UE of the same DualSteer device (i.e. the UE needs to know which other UE of the DualSteer device controls the other USIM corresponding to this USIM's subscription).

Assumption 4c: Based on observation 2. The UE(s) of the DualSteer device may need to know whether a given SUPI of a DualSteer device subscription is the primary or secondary SUPI, or need to know the path identifier of the given SUPI.

Assumption 4d: Based on the SID, the UE(s) of the DualSteer device need to be able to assess that the HPLMN supports DualSteer. Likely, this can be understood that if the UE has been informed that the subscription of the SUPI is linked to a DualSteer subscription, the HPLMN supports the DualSteer functionality.

Assumption 5a: The HPLMN needs to understand whether the UE(s) registering with a SUPI of a DualSteer subscription supports the DualSteer feature and adjust its behaviour accordingly. This may include the fact that the UE supports a single Dual-USIM DualSteer UE of DualSteer device or can be part of a two separate Single-USIM DualSteer Ues of DualSteer device.

Assumption 5b: The HPLMN needs to be able to understand whether a SUPI is the primary or secondary SUPI of a DualSteer subscription or the HPLMN needs to be able to derive the path identifier of a SUPI, and apply the network policies accordingly.

Assumption 6a: If specific handling is required in the serving network for DualSteer, the serving network needs to know whether the subscription of a UE registering to its network belongs to a DualSteer subscription.

Assumption 6b: If specific handling is required in the serving network for DualSteer, the serving network needs to know whether the UE registering with a SUPI linked to a DualSteer subscription supports DualSteer.

Assumption 6c: If specific handling is required in the serving network for DualSteer, and based on the SID, a UE of a DualSteer device registering to a serving network (other than the HPLMN – covered by assumption 4d) needs to know whether the serving network supports DualSteer.

Assumption 6d: If specific handling is required in the serving network for DualSteer, the HPLMN needs to know whether the serving network (other than itself) of a DualSteer device supports the DualSteer feature.

Assumption 7a: If a UE part of a DualSteer device is not configured with the knowledge whether the SUPI it wants to use is part of a DualSteer subscription, then that UE shall not initiate any DualSteer specific registration mechanism. The UE shall however make the network aware that it is supporting DualSteer, in order to enable DualSteer specific handling.

Assumption 7b: If a HPLMN supporting DualSteer receives a registration request from a UE for a SUPI belonging to a DualSteer subscription, and the UE is indicating being part of a DualSteer device, but the UE is not behaving accordingly, the HPLMN needs to inform the UE of the DualSteer specific aspects of the subscription.

Assumption 7c: The UE part of a DualSteer device indicates its capabilities to the network, including: (1) that it supports the DualSteer feature; (2) whether it supports being a single Dual-USIM DualSteer UE of DualSteer possibly, if it is part of a separate Single-USIM DualSteer Ues of DualSteer device, whether it acts (or can act) as the primary UE or secondary UE.device; (3) whether it supports being part of a separate Single-USIM DualSteer Ues of DualSteer device.

Assumption 7d: The UE part of a DualSteer device needs to behave as a "normal UE" if the serving network does not support DualSteer.

Assumption 7e: The DualSteer device can only use single SUPI if one of the serving networks does not support DualSteer.

Assumption 8: The serving network is expected to support DualSteer in order to enable DualSteer functionality in the DualSteer device. If the serving network does not support DualSteer, then the UE cannot enable DualSteer functionality while connected to that network.

##### 6.1.X.1.2	Assumption for Policy

Assumption 1: The policy provisioned to a UE/SUPI of DualSteer Device is a complete policy, which is independent with the registration status of the other UE/SUPI of the DualSteer Device. In other words, there is no need for the DualSteer Device to consolidate the policies received by the two UEs/SUPIs.

##### 6.1.X.1.3	Assumption for Session Management

Assumption 1: DualSteer traffic switching is applicable to SSC mode 1. For other SSC modes, the DualSteer traffic switching feature is not necessary to be applied.

Based on Assumption 1, SMF will reuse the same IP address for target PDU Session of target UE/SUPI for DualSteer traffic switching. The N4 session can be reused and correlated with target PDU Session of target UE by replacing the context of source PDU session with the context of target PDU session. SSC mode 1 is assigned to the PDU Session.

Assumption 2: The two associated PDU Sessions (source PDU Session of source UE and target PDU Session of target UE) have same parameters (DNN and S-NSSAI, SSC mode, etc).

Based on Assumption 2, URSP should be complementary for the two UEs within the DualSteer device, so that the same service can be mapped to same parameters during the switching.

Assumption 3: The same SMF/H-SMF manages the both PDU Sessions (before and after the switching) from the two UEs of the DualSteer device.

Based on Assumption 3, SMF selection needs to be enhanced to enable the AMF to select the same SMF/H-SMF. The SMF is aware of which two PDU Sessions (i.e., source PDU session and target PDU session) are related for the switching.

Assumption 4: PDU Session level switch (i.e., DualSteer traffic switching feature) is performed. All services transmitted via the PDU Session with the same IP address share the same DualSteer traffic switching policy and therefore they are switched from the source PDU session of source UE/SUPI to the target PDU session of the target UE/SUPI together.

Assumption 5: Not all services are required/allowed to use DualSteer traffic switching feature. In other words, DualSteer traffic switching policy applies to per service level.

Based on Assumption 4 and Assumption 5, the services with different policies for DualSteer switching can be grouped into different PDU Sessions, although they are associated with the same characteristics (DNN, S-NSSAI, SSC mode, etc).

URSP can be enhanced to include a new component "DualSteer traffic switching actions" within Route Selection Descriptors. If there is "DualSteer traffic switching actions" in the Route Selection Descriptors, it means those data services are allowed to be switched via DualSteer traffic switching mechanism. Moreover, when there is new service, the URSP is evaluated to determine whether an existing PDU session can be reused for the new services or a new PDU session shall be established. In this way, all traffics transmitted in the same PDU session share the same policy for the DualSteer traffic switching. When the UE of DualSteer Device requests to establish the PDU Session, it sets Request Type to "DualSteer Request for session switching".

If the "DualSteer traffic switching actions" does not exist in the Route Selection Descriptors, it means that this service is not intended to use DualSteer traffic switching feature. Therefore, this new service is transmitted via a PDU session without DualSteer traffic switching feature.

Assumption 6: Whether the DualSteer switching feature can be performed also depends on the network capability (e.g., AMF and SMF needs to support the switch) and DualSteer device’s SM subscription/policy.

Based on Assumption 6, the network needs to authorize whether the PDU Session with the Request Type of “DualSteer Request for session switching” can be switched, and needs to notify the decision to the DualSteer device. For example, "DualSteer allowed indication" is provided to the UE of the DualSteer device indicating that the PDU Session can be switched between UEs of the DualSteer device based on the policies provided to the DualSteer device.

Assumption 7: An IP address can only be used by one PDU Session for service transmission.

Based on Assumption 7, after the switching, SMF triggers to release/deactivate the source PDU Session.

Assumption 8: A PDU Session that is marked as DualSteer allowed between UE1 and UE2 of the DualSteer device, can be further handed over from the 3GPP access to the non-3GPP access within one UE using the existing mechanism specified in clause 4.9.2.2 of TS 23.502. However, based on SA1 requirements, it would need to be handed over back to the 3GPP access of that UE before being switched to the 3GPP access of the other UE based on the policies provided by the network.

#### 6.1.X.2	Solution General Principles

##### 6.1.X.2.1	General principles for Subscription and Registration

Based on assumptions for subscription and registration aspects, this solution proposes to introduce the following parameters as part of the subscription in the UDM:

-	Associated SUPI: indicates the associated SUPI for the DualSteer subscription;

-	possibly additional information as necessary, based on policies, such as whether the DualSteer simultaneous transmission is allowed or not.

-	Optionally, path Identification: information used to identify the SUPI (access) in the policy for DualSteer (e.g. Primary/Secondary or Path ID).

Based on these observations, this solution proposes to introduce the following UE capabilities to be provided as part of the registration:

-	DualSteer support: single Dual-USIM DualSteer UE of DualSteer device; separate Single-USIM DualSteer UE of DualSteer device.

Based on these assumptions, this solution proposes to introduce a new parameter as part of the registration request, that acknowledges the UE knowledge of the following aspects (1) that the UE has the proper knowledge of being part of a DualSteer configuration (incl. whether the SUPI associated to such UE is the primary or secondary SUPI or the path identifier corresponding to the SUPI), (2) that the UE knows that the serving network supports DualSteer (since otherwise it cannot provide this additional information so as to not negatively impact the serving PLMN), and (3) that the UE knows the other SUPI associated with this SUPI:

-	DualSteer Pairing information: The DualSteer Pairing information is used by the HPLMN to determine whether the two UEs/SUPIs are from the same DualSteer Device.

For example, the information is allocated by the HPLMN to the first UE/SUPI of the DualSteer Device registering to a network first to activate the DualSteer. This information will be transferred from the first UE/SUPI to the second UE/SUPI of the DualSteer Device. When the second UE/SUPI of the DualSteer Device registers to the network, it shall provide the DualSteer Pairing information. If the same information is received by the HPLMN, the HPLMN can determine that the two UEs/SUPIs are from the same DualSteer Device. In addition, this DualSteer Pairing information is also used by the DualSteer Device to determine whether the two SUPIs/SUPIs are associated to support DualSteer. When the HPLMN accepts the registration of the second UE/SUPI of the DualSteer Device, it can also provide the same DualSteer Pairing information to the UE/SUPI of the DualSteer Device. Based on the DualSteer Pairing information received by the two UEs/SUPIs of the DualSteer Device, the DualSteer Device can determine that whether the two SUPIs are associated. Alternatively, the DualSteer Pairing information provided by the network to the DualSteer Device can include the associated SUPI, which enables the DualSteer Device to be aware of the associated SUPI before attempting the Registration procedure.

Based on these assumptions, this solution proposes a serving network supporting DualSteer to provide the following information to a UE part of a DualSteer Device that has not indicated the DualSteer Pairing information:

-	information that the SUPI being used is part of a DualSteer subscription, and that it is the primary SUPI or the secondary SUPI of that DualSteer subscription, or the path identifier of the SUPI;

-	DualSteer Pairing information that is provided by the HPLMN to the UE of the DualSteer device, which will be used by the HPLMN to determine whether the two UEs/SUPIs are from the same DualSteer device and that allows the DualSteer device to correlate the other SUPI to use for the DualSteer feature;

-	the HPLMN policies related to the use of the DualSteer feature with this SUPI;

-	the fact that this serving network supports the DualSteer feature.

Based on these assumptions, the following changes to procedures are expected:

-	If the UE has indicated its UE capabilities related to DualSteer, but has not provided DualSteer Pairing information:

-	The AMF of the serving network supporting DualSteer retrieves the subscription data associated to the SUPI from the UDM, indicating the UE capabilities;

-	The UDM, based on the received UE capabilities and the subscription data for the SUPI, provides the DualSteer role of the SUPI or the path identifier of the SUPI, DualSteer Pairing information to the AMF, and/or information related to potential restrictions to registering with this SUPI in this serving network (e.g. it might reject the current registration request for the secondary SUPI if the requirements for allowing the secondary registration are not met at this stage);

-	The AMF provides the relevant information to the UE and proceeds to the registration according to the relevant HPLMN policies.

-	If the UE has indicated its UE capabilities related to DualSteer and has provided DualSteer Pairing information:

-	In addition to the above, the AMF of the serving network provides the DualSteer Pairing information to the UDM;

-	In addition to the above, the UDM correlates the DualSteer Pairing information received from this UE and the one provided to another UE with the DualSteer Pairing information of the SUPI, and determines whether to accept or reject the registration;

-	If the UDM accept the registration, the UDM shall provide the same DualSteer Pairing information to the UE via the serving AMF so that the UE can determine that the two SUPIs are associated to support DualSteer.

-	If the UE has not indicated UE capabilities or the serving network does not support DualSteer:

-	The UE follows the regular registration procedure as described in TS 23.502 [4];

-	The UDM, when receiving the request for subscription data for the SUPI, identifies that DualSteer-related information from the AMF is missing for a SUPI part of a DualSteer subscription, and may allow the registration to proceed or rejects the registration request from the AMF according to configuration. If the serving network is known to support the DualSteer feature, a specific rejection cause code can be provided to prevent further attempts.

Unless DualSteer is enabled at registration, DualSteer related functionality is disabled in the DualSteer device, and only the primary UE can be used.

NOTE:	In the case of a single Dual-USIM DualSteer UE of DualSteer device, the procedures are unchanged, except that it is the same UE that triggers the primary SUPI registration and the secondary SUPI registration. However, the policies and restrictions provided to the serving network and to the UE may be different.

##### 6.1.X.2.2	General principles for Policy Enhancement

Some of the policies provided to the UE of DualSteer Device might apply only to single Dual-USIM DualSteer UE case or two separate Single-USIM DualSteer UEs case.

According to SA1 requirements, it is assumed that there is no restriction regarding registration for the first SUPI attempting the registration, and HPLMN policies regarding registration only apply to the second SUPI attempting the registration.

In addition, the network should be provided with enough information to be able enforce these policies.

Therefore, considering the above aspects and device capabilities, the general principles are listed below:

Policy for registration of Second SUPI:

Policy for registration of Second SUPI is used to guide the UE of a DualSteer Device to perform the Registration Procedure with the Second SUPI in some certain conditions for different purposes (e.g. for backup) based on operator policy.

The DualSteer device can be provided with a combination of the following policies related to the registration of the Secondary SUPI:

-	No restriction (the second SUPI can always attempt registration).

-	Only when the first SUPI is not registered.

-	Only when the first 3GPP access is below a certain SINR/RSRP/RSSI threshold (similar to the mechanism defined in clause 23.6 in TS 36.300 [24]).

-	Location dependent (only attempt to register when in a specific area e.g. TAI(s), countries or PLMN(s)).

NOTE 1:	This policy is provided by HPLMN to guide the DualSteer Device when to trigger the Second SUPI to perform the registration. PLMN/RAT selection is per existing mechanism. The policy does not have any impact on PLMN/RAT selection.

Policy for DualSteer traffic steering:

The policy for DualSteer traffic steering is used by the UE of DualSteer Device to determine the 3GPP access to be used to transmit the new matching traffic. Once the 3GPP access has been determined, the traffic will be transmitted via that 3GPP access and the policy of DualSteer traffic steering for such matching traffic will not be considered. Whether the ongoing traffic will be moved to the other 3GPP access will be determined by the policy for DualSteer traffic switching.

Considering the capability of the DualSteer Device in single Dual-USIM DualSteer UE case and the DualSteer Device in two separate Single-USIM DualSteer UEs case is different for traffic steering, some policies provisioned to the DualSteer Device cannot be provided to the DualSteer Device with single Dual-USIM DualSteer UE case.

Therefore, following are the policies for DualSteer traffic steering which is applicable to individual new matching traffic:

-	Steer to existing activated access (either the first 3GPP access or the second 3GPP access). If neither SUPI is active, trigger to activate UP on preferred 3GPP access.

-	Steer to the first 3GPP access regardless of other traffic (applicable to two separate Single-USIM DualSteer UEs only).

-	Steer to the second 3GPP access regardless of other traffic (applicable to two separate Single-USIM DualSteer UEs only).

-	Steer to the 3GPP access whose PLMN and RAT combination is mostly matched to the preferred PLMN and RAT combination.

NOTE 2a:	The first 3GPP access and the second 3GPP access can be marked as Primary 3GPP access and Secondary 3GPP access respectively. Alternatively, each 3GPP access can be marked with a path identifier provided by UDM respectively.

NOTE 2b:	Additional optional policies can be further considered to improve the flexibility of DualSteer.

When the PCF generates the policy for DualSteer traffic steering, the PCF shall determine whether simultaneous transmission is allowed or not based on operator policy and the DualSteer Device capability (whether simultaneous transmission is supported or not) if provided by the DualSteer Device. If the PCF determines that simultaneous transmission is not allowed, the PCF shall ensure that the DualSteer traffic steering policy for all services are coordinated so that all services are transferred over the same single 3GPP access.

Since the DualSteer Device shall select the 3GPP access first and then determine whether to establish a new PDU session over the selected 3GPP access, the DualSteer traffic steering policy should be provided via UE policy. Either the existing UE policy can be enhanced or new UE policy can be introduced to include the DualSteer traffic steering policies. Following are the two possible options, which can be further discussed during conclusion or normative phase.

Option 1: A new UE policy can be introduced to specify policy for DualSteer traffic steering. In this case, the UE Configuration Update procedure as specified in clause 4.2.4 of TS 23.502 [4] is re-used to provision this policy to UE. Following is the example of the new UE policy.

Table 6.1.15.1.2-1: Access Selection Rule

| Information name | Description | Category | PCF permitted to modify in a UE context | Scope |
| --- | --- | --- | --- | --- |
| Rule Precedence | Determines the order the ASP rule is enforced in the UE. | Mandatory | Yes | UE context |
| Traffic descriptor | This part defines the traffic descriptors for the policy |  |  |  |
| Application identifiers | Application identifier(s) | Optional | Yes | UE context |
| IP descriptors | IP 3 tuple(s) (destination IP address or Ipv6 network prefix, destination port number, protocol ID of the protocol above IP) | Optional | Yes | UE context |
| Non-IP descriptors | Descriptor(s) for non-IP traffic | Optional | Yes | UE context |
| …… |  |  |  |  |
| 3GPP access for DualSteer traffic steering | One single value of following 3GPP access: 1. Existing activated 3GPP access, otherwise the first 3GPP access. 2. Prefer the first 3GPP access. 3. Prefer the second 3GPP access. 4. List of ordered PLMN and/or RAT, indicating to select the 3GPP access whose PLMN and RAT are mostly matched the list of ordered PLMN and/or RAT. | Mandatory | Yes | UE context |

Option 2: The URSP rule can be enhanced to include the policy for DualSteer traffic steering. It is assumed that such URSP rules supporting DualSteer traffic steering can be provisioned to the DualSteer Device via either Primary 3GPP access or Secondary 3GPP access. The UE of the DualSteer Device can evaluate the URSP to determine whether the new matching traffic is transmitted via current 3GPP access or the other 3GPP access. If the new matching traffic is determined to be transmitted via the other 3GPP access, the traffic will be transferred across UEs within the DualSteer Device (in two separate Single-USIM DualSteer Ues case) based on implementation. Following is the example of the enhanced URSP rules.

Table 6.1.15.1.2-2: Route Selection Descriptor

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory | Yes | UE context |
| Route selection components | This part defines the route selection components | Mandatory |  |  |
| … | … | … | … | … |
| 3GPP access for DualSteer traffic steering | One single value of following 3GPP access: 1. Existing activated 3GPP access, otherwise Primary 3GPP access. 2. Prefer Primary 3GPP access. 3. Prefer Secondary 3GPP access. 4. List of ordered PLMN and/or RAT, indicating to select the 3GPP access whose PLMN and RAT are mostly matched the list of ordered PLMN and/or RAT. | Mandatory | Yes | UE context |

The different values of 3GPP access for DualSteer traffic steering are used to support different steering polices:

-	Value 1 - Existing activated 3GPP access, otherwise Primary 3GPP access: steering the new service to the existing activated 3GPP access, if no activated 3GPP access, use the first 3GPP access to transmit the new service.

-	Value 2 - Prefer the first 3GPP access: try to steering the new service to the first 3GPP access. Otherwise, use the second 3GPP access to transmit the new service.

-	Value 3 - Prefer the second 3GPP access: try to steering the new service to the second 3GPP access. Otherwise, use the first 3GPP access to transmit the new service.

-	Value 4 - List of ordered PLMN and/or RAT: select the 3GPP access network which match the PLMN and/or RAT in the list of ordered.

The value of the 3GPP access included in the URSP is determined based on the DualSteer subscription and DualSteer Device capability. For example, if the DualSteer Device is in single Dual-USIM DualSteer UE case, the second and third value would not be included in the URSP rule.

The UE Configuration Update procedure as specified in clause 4.2.4 of TS 23.502 [4] is re-used to provision this policy to UE.

Policies for DualSteer traffic switching:

The policies for DualSteer traffic switching are used by the UE of DualSteer Device to determine following aspects:

1)	Whether the traffic of partial services can be switched to the other 3GPP access or the traffic of all services shall be moved so that only single 3GPP access will be used to transmit all traffic at one time.

This policy is used by the DualSteer Device to determine a general principle for the DualSteer traffic switching which considers the DualSteer Device capability and operator policy. For example, the policy that the traffic of some services only can be switched is only provided to the DualSteer Device in two separate Single-USIM DualSteer UEs case.

2)	Whether and when the matching traffic can be switched to the other 3GPP access.

This policy is used by the UE of DualSteer Device to determine DualSteer traffic switching actions (i.e. whether and when the matching traffic should be moved to the other 3GPP access) based on the policy and principle mentioned in 1).

This policy will further help the UE of DualSteer Device to group the traffic with the same DualSteer traffic switching actions into the same PDU Session so that the traffic with the same DualSteer traffic switching action can be switched together to the other 3GPP access by switching the PDU Session from the source 3GPP access to the target 3GPP access. In this case, service level switching can be achieved.

Therefore, based on the above consideration, the policies for DualSteer traffic switching are:

1.	DualSteer simultaneous transmission policy for DualSteer Device:

-	no restriction (only be applicable to two separate Single-USIM DualSteer UEs case);

-	always use single 3GPP access for all traffic (i.e. non-simultaneous transmission).

DualSteer simultaneous transmission policy for DualSteer Device is a new information of UE policy. The UE Configuration Update procedure as specified in clause 4.2.4 of TS 23.502 [4] is re-used to provision this policy to the DualSteer Device.

2.	DualSteer traffic switching policy for the matching traffic:

-	based on DualSteer simultaneous transmission policy, switch to the first 3GPP access whenever it becomes available;

-	based on DualSteer simultaneous transmission policy, switch to the second 3GPP access whenever it becomes available;

-	based on DualSteer simultaneous transmission policy, switch to the other 3GPP access if the current 3GPP access status meets the condition of DualSteer traffic switching;

-	based on DualSteer simultaneous transmission policy, switch to the other 3GPP access whenever necessary.

NOTE 3:	Policy for DualSteer traffic switching can be set properly to optimize the performance of DualSteer traffic switching.

Either the existing UE policy can be enhanced or new UE policy can be introduced to include the DualSteer traffic steering policies. Following are the two possible options, which can be further discussed during conclusion or normative phase.

Option 1: enhancing URSP to include the DualSteer traffic switching policies

For the DualSteer traffic switching policies used by the UE of DualSteer Device for the matching traffic, the URSP rule can be enhanced to include such policy. It is assumed that such URSP rules supporting DualSteer traffic switching can be provisioned to the DualSteer Device via either the first 3GPP access or the second 3GPP access or both of them. Each UE can evaluate the URSP to determine whether the matching traffic can be moved to the other 3GPP access via DualSteer traffic switching with the same condition.

The policy for DualSteer traffic switching is stored in the PDU Session context similarly to DNN, S-NSSAI, SSC mode, etc. When new traffic is established, the rule for reusing an existing PDU Session (see clause 6.6.2 of TS 23.503 [5]) will also take in account the policy for DualSteer traffic switching, i.e. it will only reuse an existing PDU Session if that PDU Session has the same policy for DualSteer traffic switching as the matching URSP rule for the new traffic. Otherwise, a new PDU Session will be established for the new traffic. This allows to switch all traffic of a given PDU Session simultaneously as they share the same trigger conditions for switching. If the parameter of "DualSteer traffic switching actions" is not present in the URSP, it means that the DualSteer traffic switching mechanism is not enabled for this service.

Table 6.1.15.1.2-4: Route Selection Descriptor

| Information name | Description | Category | PCF permitted to modify in URSP | Scope |
| --- | --- | --- | --- | --- |
| Route Selection Descriptor Precedence | Determines the order in which the Route Selection Descriptors are to be applied. | Mandatory | Yes | UE context |
| Route selection components | This part defines the route selection components. | Mandatory |  |  |
| … | … | … | … | … |
| DualSteer traffic switching actions | One single value of following actions: 1. switch to Primary 3GPP access whenever it becomes available. 2. switch to Secondary 3GPP access whenever it becomes available. 3. switch to the other 3GPP access if the current access status meets the condition of DualSteer traffic switching. 4. switch to the other 3GPP access whenever necessary. | Optional | Yes | UE context |

The different values of DualSteer traffic switching actions are used to support different switching triggers:

-	Value 1 - Switch to Primary 3GPP access whenever it becomes available: switch the services of the PDU session to the Primary 3GPP access whenever it becomes available.

-	Value 2 - Switch to Secondary 3GPP access whenever it becomes available: switch the services of the PDU session to the Secondary 3GPP access whenever it becomes available.

-	Value 3 - Switch to the other 3GPP access if the current access status meets the condition of DualSteer traffic switching: when the status of current 3GPP access meets the condition of DualSteer traffic switching, switch the services of the PDU session to the other 3GPP access.

-	Value 4 - Switch to the other 3GPP access whenever necessary: some services need not have specific conditions to trigger the switching. However, in some cases such as when the DualSteer Device is in single UE case, some services are triggered to be switched to the other 3GPP access network. The leftover services are also triggered to switched to the other 3GPP network. In this case, the DualSteer traffic switching actions corresponding to these services can use the action.

Option 2: introduce new UE policies to provide the DualSteer traffic switching policies

A new UE policy can be introduced for the DualSteer traffic switching policies. In this case, when there is a new coming service, apart from evaluating the URSP rules, the UE of the DualSteer Device shall also consider the DualSteer traffic switching policies to determine whether a new PDU session should be established in order to transmit all the services with the same PDU session parameters and the same switching conditions into the same PDU session.

When the conditions of the 3GPP accesses change (e.g. one 3GPP access becomes available/registered, one 3GPP access becomes unavailable), the value of the DualSteer traffic switching action associated to the PDU Sessions of the DualSteer device are evaluated, and taking in account the DualSteer simultaneous transmission policy for the DualSteer device, none, some, or all PDU Sessions are moved from one 3GPP access to another.

When the PCF generates the policy for DualSteer traffic switching, the PCF shall determine whether simultaneous transmission is allowed or not based on operator policy and the DualSteer Device capability (whether simultaneous transmission is allowed or not) if provided by the DualSteer Device. If simultaneous transmission is not allowed, the PCF shall set the DualSteer simultaneous transmission policy for DualSteer Device to "always use single 3GPP access for all traffic" and shall ensure that the DualSteer traffic switching policy for all services shall be coordinated so that all services are transferred over the same single 3GPP access. When the DualSteer traffic switching policy is not present:

-	for SSC mode 1, the traffic will never be moved to the other 3GPP access via DualSteer traffic switching mechanism;

-	for SSC mode 2 or 3, the traffic can be moved to the other 3GPP access by disconnecting the current UP and establishing the new UP over the other 3GPP access.

Policies for when to establish the associated PDU session for DualSteer traffic switching:

For services that can be switched via DualSteer traffic switching mechanism, the HPLMN can further provide a policy to instruct the UE of DualSteer Device when to establish the target PDU session for DualSteer traffic switching. Following are the possible options:

-	establish the associated PDU session for DualSteer traffic switching when switching condition is met (i.e., when performing DualSteer traffic switching); or

-	establish the associated PDU session for DualSteer traffic switching in advance if possible (i.e., before performing DualSteer traffic switching).

NOTE:	If the DualSteer Device is allowed to pre-establish the PDU session for DualSteer traffic switching, the SMF would not activate the UP of the target PDU session until the switching condition is met. After switching, the SMF will either release or deactivate the source PDU session based on operator policy.

##### 6.1.X.2.3	General principles for Session Management

The PDU Session handling for DualSteer Device can be divided into two parts. One is the PDU Session Establishment for source UE of the DualSteer device (UE1), and the other one is the procedure of PDU Session Switching from the source UE to the target UE of the DualSteer device (UE2).

Principle for PDU Session Establishment

The DualSteer traffic steering policies are evaluated in the DualSteer Device to decide which 3GPP access (or which UE/SUPI) is used to transmit the new service (existing activated access, the first 3GPP access, the second 3GPP access), with the assumption that the DualSteer Device knows the registration status of two UEs/SUPIs. The details of DualSteer traffic steering policies are specified in clause 2.2.2.

For more flexible switching handling the PDU Session with same parameters (DNN and S-NSSAI, SSC mode, etc) can be further divided into two categories, i.e., PDU sessions with DualSteer traffic switching feature and the PDU Session without DualSteer traffic switching feature.

Once the 3GPP access has been determined (i.e., the UE/SUPI has been selected), the URSP rule of the selected UE/SUPI is evaluated to determine whether to reuse an established PDU Session or to establish a new PDU Session based on the DualSteer traffic switching actions and other existing information elements (e.g., DNN, S-NSSAI) in the RSD. If the "DualSteer traffic switching actions" is present in the Route Selection Descriptors, it means that this new service can be switched via DualSteer traffic switching mechanism. Therefore, this new service shall be transmitted via a PDU session which is allowed to use DualSteer traffic switching mechanism with the corresponding "DualSteer traffic switching actions".

If the "DualSteer traffic switching actions" does not exist in the Route Selection Descriptors, it means that this service does not use DualSteer traffic switching feature. Therefore, this new service is transmitted via PDU session without DualSteer traffic switching feature.

If only one UE/SUPI in the DualSteer device is registered, PDU Sessions are only established by the registered UE/SUPI. The DualSteer traffic switching can only be performed after the other UE/SUPI in the DualSteer device has registered to the network.

Principle for PDU Session Switching

DualSteer simultaneous transmission policy for DualSteer Device:

-	no restriction (only be applicable to two separate Single-USIM DualSteer UEs case)

-	always use single 3GPP access for all traffic (i.e., non-simultaneous data transmission)

When the triggers for DualSteer traffic switching are met, the DualSteer traffic switching policy is evaluated in the DualSteer Device for each PDU session to determine how to handle other PDU sessions whose triggers for DualSteer traffic switching are not met.

If "no restriction" is present, each PDU session supporting the DualSteer traffic switching feature is switched based on the DualSteer traffic switching actions associated to the PDU session.

If "always use single 3GPP access for all traffic", only one 3GPP access can be used for data transmission. Therefore, all PDU sessions are moved or re-established according to the policy, or none is moved.

DualSteer Device determines to perform DualSteer traffic switching based on the DualSteer traffic switching policies specified in clause 2.2.2 and the current conditions of the DualSteer device (e.g., the first/the second 3GPP access is available or not, current/other 3GPP access status).

There are two options considering how AMF selects the same SMF, and how the SMF correlates the two PDU sessions.

-	In Option#1, the association is based on PDU Session ID#1 and SUPI of UE1.

-	In Option#2, the association is based on Session correlation ID allocated by SMF.

Enforcement of policy for non-simultaneous transmission

If the HPLMN determines that the simultaneous transmission is not allowed based on operator policy (e.g. due to subscription or DualSteer Device capability), the HPLMN shall provision policy within the network to ensure that only one 3GPP access is used by the DualSteer Device for data transmission. In this solution, the UDM is enhanced to enforce such policy as follow:

-	If the simultaneous transmission is not allowed, when there is a new PDU Session established by the target UE/SUPI over the target 3GPP access, the UDM shall instruct the SMF(s) to deactivate/release the PDU Session(s) established by the source UE/SUPI (if any).

#### 6.1.X.3	Procedures for DualSteer

##### 6.1.X.3.1	Registration Procedure

The registration procedure for DualSteer Device can be divided into two parts. One is the registration procedure for the primary SUPI as specified in clause 6.1.8.2.2, while the other is the registration procedure for the secondary SUPI as specified in clause 6.1.8.2.3.

NOTE:	The procedures are described for the DualSteer Device with two separate Single-USIM DualSteer Ues. In case of the DualSteer Device with single Dual-USIM DualSteer UE, the same procedures apply with UE1 and UE2 being the same.

###### 6.1.X.3.1.1	Registration procedure for the first SUPI

Figure 2.3.1.1-1: Registration procedure for Primary SUPI

1.	The UE uses the first SUPI to perform Registration procedure. UE sends to AMF 1 via RAN 1 a Registration Request including a DualSteer Capability. The DualSteer Capability further indicates whether the UE is part of a single Dual-USIM DualSteer UE of DualSteer device or of a two separate Single-USIM DualSteer Ues of DualSteer device. If the DualSteer device has previously obtained the DualSteer Pairing information from this serving PLMN, the UE will also provide the DualSteer Pairing information in the Registration Request.

2.	AMF 1 triggers the authentication and security procedure.

3.	After the UE with SUPI 1 is successfully authenticated, AMF sends Nudm_UECM_Registration Request to the UDM to register itself as the serving AMF. If the serving AMF supports DualSteer (i.e. the serving network supports DualSteer), the AMF also includes a DualSteer Capability in the request indicating that the UE is registered for DualSteer.

4.	When receiving the DualSteer Capability from the serving AMF, UDM is aware of that the serving PLMN supports the DualSteer and further checks whether this SUPI is associated with the DualSteer subscription. The UDM checks whether this SUPI is the primary SUPI for this DualSteer Device.

In addition, the UDM provides DualSteer Pairing information. The DualSteer Pairing information is an identifier used by the HPLMN to determine whether the two Ues are from the same DualSteer device. The DualSteer Pairing information is provided to the UE of the DualSteer Device to identify the SUPI pair to support the DualSteer functionality.

5.	If this SUPI is associated with DualSteer subscription, the UDM provides the result of DualSteer authorization (e.g. DualSteer Authorized), the DualSteer Pairing information to the serving AMF. Alternatively, the DualSteer Pairing information can include the associated SUPI, which enables the DualSteer Device to be aware of the associated SUPI before attempting the Registration procedure of another UE/SUPI.

Editor's note:	Whether there are potential issues of sending associated SUPI to the UE is FFS and shall coordinate with SA WG3.

6.	When receiving the DualSteer Authorized result from the UDM, the serving AMF will request to get the DualSteer subscription data from the UDM via Nudm_SDM_Get Request including a DualSteer Indication.

7.	The UDM retrieves the DualSteer subscription based on the indication from the serving AMF. The UDM sends the DualSteer subscription to the serving AMF via Nudm_SDM_Get Response. The UDM indicates that this is primary SUPI associated with the DualSteer subscription.

8.	The serving AMF sends Registration Accept message to the UE of the first SUPI with DualSteer Authorized Indication, the first SUPI indication, and the DualSteer Pairing information received from the UDM.

9.	After receiving the Registration Accept message, the DualSteer Device can determine whether the network (both the serving PLMN and the HPLMN) supports DualSteer and whether the DualSteer Device can behave to support DualSteer. In other words, if there is DualSteer Authorized Indication in the Registration Accept message, the DualSteer Device determines that the network supports the DualSteer and it is allowed to act as DualSteer Device.

After the Registration procedure, policies for DualSteer are provisioned to the UE of the first SUPI. The DualSteer Device may further trigger to perform Registration with the secondary SUPI based on the policy from the HPLMN. Additionally, the UE1 of the DualSteer Device informs the UE2 of DualSteer Device its registration status. The DualSteer Pairing information shall be transferred to the UE of the DualSteer device using the second SUPI to perform registration. After the registration procedure, the HPLMN can determine that the two UEs are from the same DualSteer device based on the DualSteer Pairing information and the DualSteer Device can correlate the two SUPIs to support the DualSteer functionality based on the DualSteer Pairing information as specified in clause 6.1.8.2.3.

Once registered on the first 3GPP access, if the second 3GPP access is already registered and there are ongoing PDU Sessions on the second 3GPP access, the DualSteer device may verify whether traffic can or need to be switched to the first 3GPP access according to the policies provided by the network.

###### 6.1.X.3.1.2	Registration Procedure for Second SUPI

Figure 2.3.1.2-1: Registration procedure for Secondary SUPI

1.	Based on the policy from HPLMN, the DualSteer Device may trigger the registration procedure with the second SUPI. If the DualSteer Device has not stored the policy from the HPLMN yet, the DualSteer Device may attempt the registration procedure.

2.	UE2 sends to AMF 2 via RAN 2 a Registration Request including a DualSteer Capability and the DualSteer Pairing information received by the UE1. The DualSteer Pairing information is shared within the DualSteer device from UE1 to UE2 based on the device implementation.

NOTE:	In the intra-PLMN scenario, it is possible that AMF 2 can be the same AMF as AMF 1.

3.	AMF 2 triggers the authentication and security procedure.

4.	After the UE with SUPI 2 is successfully authenticated, the AMF 2 sends Nudm_UECM_Registration Request to the UDM to register itself as the serving AMF. If the serving AMF 2 supports DualSteer (i.e. the serving network supports DualSteer), the AMF 2 also includes a DualSteer Capability and the DualSteer Pairing information in the request indicating that the UE is registered for DualSteer.

5.	When receiving the indication from the serving AMF 2, UDM is aware of that the serving PLMN supports the DualSteer and further check whether this SUPI is associated with the DualSteer subscription. If this is the second SUPI and the first SUPI has not been registered, the UDM may reject the Registration based on policy (e.g. the second SUPI can only be used concurrently with the first SUPI).

6.	If this SUPI is associated with DualSteer subscription and is the second SUPI, the UDM informs the result of DualSteer authorization (e.g. DualSteer Authorized) to the serving AMF. If the UDM receives the UE Pairing information from the AMF and check that the DualSteer Pairing information is the same as the one allocated to the other associated SUPI (i.e. the first SUPI in this procedure), the UDM informs the result of DualSteer authorization (i.e. DualSteer Pair Authorized) to the serving AMF.

7.	When receiving the DualSteer Authorized result from the UDM, the serving AMF will request to get the DualSteer subscription data from the UDM via Nudm_SDM_Get Request including a DualSteer Indication.

8.	The UDM retrieves the DualSteer subscription based on the indication from the serving AMF. The UDM sends the DualSteer subscription to the serving AMF via Nudm_SDM_Get Response. The UDM indicates that this is secondary SUPI associated with the DualSteer subscription.

9.	The serving AMF sends Registration Accept message to the UE of the second SUPI with DualSteer Pair Authorized Indication, the DualSteer Pairing information and the secondary SUPI indication if received from the UDM. Based on the DualSteer information received from the UDM, the AMF may reject the registration (e.g. in a case where the UE is not allowed to use the second SUPI in the current location).

10.	The DualSteer Device can correlate the two SUPIs to support the DualSteer functionality based on the DualSteer Pair Authorized result and the DualSteer Pairing information.

Once registered on the second 3GPP access, the UE2 of the DualSteer Device informs the UE1 of DualSteer Device its registration status, and if the first 3GPP access is already registered and there are ongoing PDU Sessions on the first 3GPP access, the DualSteer device may verify whether traffic can or need to be switched to the second 3GPP access according to the policies provided by the network.

##### 6.1.X.3.2	Policy Provisioning Procedure

Figure 2.3.2-1: Procedure of policy provisioning

1.	UE of DualSteer Device performs Registration procedure with the first SUPI. During the procedure, policies mentioned in clause 2.2.2 are generated by PCF based on operator policy. In particular, the policies provisioned to the UE of the DualSteer Device also consider the DualSteer Device capability.

The detailed call flows and the corresponding parameters in each message depends on procedure in clause 2.3.1.1 and clause 2.3.1.2.

2.	When the UE of the DualSteer Device performs Registration procedure successfully and is authorized to perform DualSteer traffic steering and DualSteer traffic switching, the UCU procedure is triggered to provision the policies for DualSteer to UE as specified in clause 4.2.4 in TS 23.502 [4].

3.	Based on the policies for registration of Secondary SUPI, the DualSteer Device activates the second SUPI and performs Registration procedure. During the procedure, policies mentioned in clause 6.1.15.1 may be generated or updated by PCF based on operator policy. In particular, the policies provisioned to the UE of the DualSteer Device also consider the DualSteer Device capability.

The detailed call flows and the corresponding parameters in each message depends on procedure in clause 2.3.1.1 and clause 2.3.1.2.

4.	When the UE of the DualSteer Device performs Registration procedure successfully and is authorized to perform DualSteer traffic steering and DualSteer traffic switching, the UCU procedure may be triggered to provision the policies for DualSteer to UE as specified in clause 4.2.4 of TS 23.502 [4].

##### 6.1.X.3.3	Procedure to Support DualSteer Traffic Steering

Figure 2.3.3-1 shows the PDU Session Establishment procedure for DualSteer traffic steering.

Figure 2.3.3-1: PDU Session establishment procedure

0.	UDM stores the linked SUPIs of UE1 and UE2, using the solution on KI#1.1.

1.	UE1 and UE2 registers to network, using the procedure in clause 2.3.1.1. UE1 can be the first UE, and UE2 can be the second UE. AMF1 is the UE1’s serving AMF, and AMF2 is the UE2’s serving AMF. During the registration, it assumes that the AMF1 knows that the UE1 is authorized to use DualSteer and the AMF2 knows that the UE2 is authorized to use DualSteer, and the DualSteer device knows that UE1 and UE2 are authorized to use DualSteer.

2.	UE1 request to establish PDU Session ID#1, with Request Type indicating "DualSteer Request for session switching". Based on the "DualSteer Request for session switching", AMF1 selects SMF supporting DualSteer.

Before step 2, based on the DualSteer policy for traffic steering received during step 1, it assumes that UE1 is selected to transmit the service data, and then the URSP of UE1 is evaluated. The details of DualSteer policy for traffic steering are specified in clause 2.2.2.

The URSP can be enhanced to include a new component "DualSteer traffic switching actions" within Route Selection Descriptors. If the "DualSteer traffic switching actions" exists in the Route Selection Descriptors, the UE1 determines to use the PDU session with DualSteer traffic switching feature to transmit the traffic of the service. The UE1 determines whether an existing PDU session can be reused for the new service or a new PDU session shall be established by taking into account the value of DualSteer traffic switching actions. If the UE1 determines to establish a new PDU session with DualSteer traffic switching feature for the new service, the UE1 sets Request Type indicating "DualSteer Request for session switching". If the "DualSteer traffic switching actions" does not exist in the Route Selection Descriptors, then the UE1 establishes the PDU Session without DualSteer traffic switching feature, and the data services over this PDU session cannot be switched to the target 3GPP access of target UE via DualSteer traffic switching mechanism.

3.	AMF1 sends SUPI of UE1, PDU Session ID#1 and "DualSteer Request for session switching" to SMF. Based on the "DualSteer Request for session switching". If the UE1 is not authorized for perform DualSteer, the AMF shall reject the PDU session.

4.	SMF retrieves, SM subscription data from UDM.

In order to enable the network to correlate the PDU session established by UE2 in the potential DualSteer traffic switching procedure with this PDU session and select the same SMF during the switching procedure, correlation information for the PDU session to be switched is needed and is sent by the SMF to the UDM. The correlation information includes PDU Session ID#1, SMF ID and an indication indicating that the PDU Session ID#1 is allowed to support DualSteer traffic switching to the UDM.

5.	Based on the association between the SUPI of UE1 and the SUPI of UE2 in the subscription, UDM sends the PDU Session ID#1, SUPI of UE1 and SMF ID to the AMF2, if this PDU Session ID#1 is used for DualSteer traffic switching.

AMF2 stores the PDU Session ID#1, SUPI of UE1 and SMF ID.

If the UE1 is not authorized to perform DualSteer (e.g. based on subscription or if SUPI1 and SUPI2 are not used by the same DualSteer Device), UDM does not perform this step.

NOTE:	Step 5 can be performed separately when UE2 registers to the network.

6.	Remaining steps are performed to complete the establishment of PDU Session ID#1, including N3 and N4 establishment. The UPF is the Anchor UPF. SMF may establish SM Policy Association with PCF to get the PCC Rules for PDU Session ID#1 of UE1.

NOTE 1: The PCC Rules are defined in existing specification. There are no new impacts to PCC Rules.

7.	If the SMF decides to accept the "DualSteer Request for session switching", the SMF sends a DualSteer allowed indication to AMF1.

8.	AMF1 sends the DualSteer allowed indication to UE1.

After the PDU Session ID#1 of UE1 is established, the DualSteer device transmits the corresponding data services over the PDU Session ID#1 of UE1.

Apart from using the PDU session #1 of UE1 as correlation information to identify the PDU session for potential DualSteer traffic switching, SMF can also generate a Session correlation ID for network to identify the PDU session to be switched. For this alternative, the procedure of 6.1.15.2.2 can be used with following modifications and clarifications:

-	In step 4, SMF allocates Session correlation ID#1 for the PDU Session ID#1 of UE1, if PDU Session ID#1 is requested for DualSteer. The Session correlation ID shall be unique. SMF sends the Session correlation ID#1 and SMF ID to UDM. SMF stores the association of Session correlation ID#1 and PDU Session ID#1 of UE1.

-	In step 5, UDM sends the Session correlation ID#1 and SMF ID to AMF2. AMF2 stores the Session correlation ID#1 and SMF ID.

-	In step 7, the SMF sends a DualSteer allowed indication and Session correlation ID#1 to AMF1.

-	In step 8, the AMF1 sends the DualSteer allowed indication and Session correlation ID#1 to UE1.

##### 6.1.X.3.4	Procedure to Support DualSteer Traffic Switching

Figure 2.3.4-2 shows the PDU session establishment of the associated PDU session of the source PDU Session for DualSteer traffic switching.

Based on the policies from the HPLMN about when to establish the associated PDU session of the source PDU session for DualSteer traffic switching, the DualSteer Device performs the following procedure in the appropriate time. For example, if the policies indicate the DualSteer Device to establish associated PDU session for DualSteer traffic switching when the switching condition is met, the DualSteer Device will perform following procedure only when the switching condition is met. If the policies allow the DualSteer Device to establish the associated PDU session for DualSteer traffic switching in advance, the DualSteer Device will perform following procedure before the switching condition is met.

Figure 2.3.4-1: PDU Session Switching procedure

0.	After the PDU Session establishment of UE1, PDU Session ID#1 of UE1 is shared from UE1 to UE2 within the device based on the device implementation.

1.	When DualSteer device decides the session switch based on the DualSteer traffic switching policies as the specified in clause 6.1.15 and the current conditions of this DualSteer device (e.g., Primary/Secondary 3GPP access is available or not, current/other 3GPP access status), DualSteer device requests the session switching from PDU Session ID#1 of UE1 to PDU Session ID#2 of UE2. In particular, UE2 sends NAS message to AMF2, includes PDU Session ID#2, PDU Session ID#1 of UE1, session switch indication, and PDU Session establishment request. The PDU Session ID#2 of UE2 has the same parameters (e.g., DNN and S-NSSAI) with the PDU Session ID#1 of UE1.

Based on DualSteer allowed indication received during the establishment of PDU Session ID#1, the DualSteer device knows that the PDU Session ID#1 is allowed to be switched between UEs.

2.	AMF2 may select the SMF based on the information if received from UDM (step 5 of Figure 6.1.X.2.2). If AMF2 does not receive the information for SMF selection, the AMF2 sends the request to UDM to obtain the the SMF ID for PDU Session ID#1 of UE1.

If the UE2 is not authorized to perform DualSteer, the AMF shall reject the PDU session.

3.	AMF2 sends SUPI of UE2, PDU Session ID#2, PDU Session ID#1 of UE1, SUPI of UE1, session switch indication, and PDU Session establishment request.

4.	If the SMF receives the session switch indication from AMF2 and the PDU session is allowed for DualSteer based on SM subscription data, the SMF determines that UE IP address and N4 session associated with PDU Session ID#1 of UE1 can be reused for PDU Session ID#2 of UE2.

Based on operator policy, SMF either triggers the PDU session release procedure to release the PDU Session ID#1 of UE1 or deactivates the UP of the PDU Session#1 after switching has been performed. If SMF determines to release the PDU Session#1, steps 5 and 6 are performed. If the SMF determines to deactivate the UP of the PDU Session#1 of UE1 after switching has been performed, steps 7 to 9 are performed.

5.	Remaining steps are performed to complete the establishment of PDU Session ID#2.

SMF may establish SM Policy Association with PCF to get the PCC Rules for PDU Session ID#2 of UE2.

NOTE 1:	The PCC Rules are defined in existing specification. There are no new impacts to PCC Rules.

NOTE 2: 	The SM Policy Association for PDU Session ID#1 of UE1 and SM Policy Association of PDU Session ID#2 of UE2 are separated, and SMF does not need to select the same PCF. There is no impact on the SM Policy Association

SMF triggers the N4 Session Modification procedure to update the N4 Session Context to replace the AN tunnel info with the AN tunnel info of RAN2. SMF sends the CN tunnel info of the UPF to the RAN2 via N2 SM Information.

After step 5, PDU Session ID#2 of UE2 is established, the DualSteer device and UPF transmits the corresponding services via the PDU Session ID#2 of UE2.

If simultaneous transmission is not allowed based on operator policy, the UDM shall inform the SMF(s) to deactivate/release the PDU Session(s) established by SUPI1/UE1 (if any) based on the UE context in SMF data of the associated SUPI subscription.

6.	SMF triggers the PDU session release procedure to release the PDU Session ID#1 of UE1, including to release SM Policy Association with PCF for PDU Session ID#1 of UE1, and to deregister this PDU Session from the UDM. After the switching, the same IP is only used by the PDU Session ID#2 of UE2.

7.	If the DualSteer Device is allowed to pre-establish the PDU Session for DualSteer traffic switching before switching condition is met, the SMF performs remaining steps to complete the establishment of PDU Session ID#2 but without activating the UP of the PDU Session#2.

8.	DualSteer Device measures the link status based on implementation. When the switching condition is met, the UE#2 sends Service Request to the activate the UP of the PDU Session#2.

9.	Based on the Service Request from the UE#2, SMF determines to activate the UP of the PDU Session#2 and deactivate the UP of the PDU Session#1. The SMF initiates an N4 Session Modification procedure indicating the need to remove AN Tunnel Info for N3 tunnel of the corresponding PDU Session. After the switching, the same IP is only used by the PDU Session ID#2 of UE2.

If the Session correlation ID is used in the PDU session establishment procedure as specified in clause 2.3.3-1, the procedure of 2.3.4-1 can be used with following modifications and clarifications:

-	In step 1, the UE2 sends NAS message to AMF2, includes PDU Session ID#2, Session correlation ID#1, session switch indication, and PDU Session establishment request.

-	In step 2, the AMF2 may send the request to UDM to obtain the the SMF ID for Session correlation ID#1.

-	In step 3, the AMF2 sends SUPI of UE2, PDU Session ID#2, Session correlation ID#1, session switch indication, and PDU Session establishment request.

-	In step 4, the SMF associates PDU Session ID#1 of UE1 and PDU Session ID#2 of UE2 based on Session correlation ID#1.

The procedures in Figure 2.3.3-1 and Figure 2.3.4-1 describe the non-roaming intra-PLMN scenario where both UE1 and UE2 are in the HPLMN, but the same procedure applies to the other scenarios, as follows:

- For the scenario that UE1 is in the HPLMN and UE2 is roaming, the PDU Session for UE2 is set up as HR roaming where the SMF is the H-SMF, and the UPF is the H-UPF. The message in step 3 in Figure 2.3.4-1 is sent to V-SMF and additionally includes H-SMF ID, and the V-SMF sends the information in the step 3 in Figure 2.3.4-13 to the H-SMF.

- For the scenario that UE1 is roaming and UE2 is in the HPLMN, the PDU Session for UE1 is set up as HR roaming where the SMF is the H-SMF, and the UPF is the H-UPF. The messages in both step 3 and step 7 in Figure 2.3.3-1 are transferred via V-SMF.

- For the scenario that both UE1 and UE2 are roaming, the PDU Sessions for UE1 and UE2 are set up as HR roaming where the SMF is the H-SMF, and UPF is the H-UPF. The messages in both step 3 and step 7 in Figure 2.3.3-1 are transferred via V-SMF1. The message in step 3 in Figure 2.3.4-1 is sent to V-SMF and additionally includes H-SMF ID, and the V-SMF sends the information in the step 3 in Figure 2.3.4-1 to the H-SMF.

# * * * * End of changes * * * *