3GPP SA WG2 Meeting #164	S2-2408345

Maastricht, NL, 19-23 August 2024

Title:	[Draft] Reply LS on multi-modality awareness at RAN

Response to:	Reply LS on multi-modality awareness at RAN

Release:	Release 19

Work Item:	FS_XRM_Ph2

Source:	SA2

To:	RAN2

Cc:

Contact Person:

Name:	Yixue Lei

Tel. Number:

E-mail Address:	yixuelei AT tencent DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

SA2 thanks RAN2 LS on multi-modality awareness at RAN. SA2 would like to inform RAN2 the following:

- SA2 has specified the Multi-modal Service ID in Rel-18, see 23.501 clause 5.37.2 for details, which can indicate that certain multi-modal QoS flows belong to the same XRM service.

- SA2 think it is feasible to provide the Multi-modal Service ID to NG-RAN to assist RAN to identify the QoS flows belonging to the same XRM services which may have the same or close synchronization requirement if RAN2 needs it.

- So far, SA2 have not yet specifically work discuss the synchronization thresholds.  However, from end-to-end QoS perspective, SA2 think that whether the synchronization requirement can be met is subject whether the CN-PDB and AN-PDB can be guaranteed and/or whether the packet delay variation between UE and NG-RAN requirement can be guaranteed or not.  If the AN-PDB can be fulfilled by NG-RAN and the packet delay variation for different QoS flows between UE and NG-RAN can be controlled in a certain range, the 5G system has done what it can to meet the synchronization requirement.  Therefore, SA2 think that in Rel-19, NG-RAN can utilize the existing QoS parameters (e.g. AN-PDB) from 5GC and utilize other approaches without relying on additional QoS parameters from 5GC (e.g. synchronization thresholds), including implementation-based approaches, to guarantee the packet delay variation between UE and NG-RAN among different flows are not larger than a certain range, so that the synchronization requirements of different multi-modal flows can be met.

2. Actions:

To RAN2:

ACTION: 	SA2 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #165		14-18 October 2024	Hyderabad, India

TSG-SA2 Meeting #166		18-22 November 2024	Orlando, FL, USA