3GPP TSG-SA WG2 Meeting #164	S2-2408350

19 August - 23 August 2024, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	CATT, Vivo, China Telecom, OPPO，Xiaomi

Title:	Initial conclusions for DualSteer KI#1.2

Document for:	Approval

Agenda Item:	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract: Initial conclusions for DualSteer KI#1.2.

1.	Introduction

This contribution proposes to update the conclusion for KI#1.2.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-54 v1.0.0.

# * * * * First Change * * * *

8.1	Conclusions for DualSteer

8.1.2	Conclusion for KI#1.2: Registration and mobility management for DualSteer

The following principles are proposed to be considered in normative work:

-	The DualSteer Device shall indicate to the AMF its capabilities to support the DualSteer service during registration for each SUPI.

-	The AMF shall indicate to the DualSteer Device whether the network supports the DualSteer capabilities during registration.

-	The network indicates to the DualSteer Device whether the DualSteer is authorized after the registrations for the two SUPIs.

-	The DualSteer Device shall perform separate registration procedures for each of the two SUPIs.

-	The UDM associates registration of two SUPIs based on the association of the two SUPIs.

# * * * * End of change* * * *