3GPP TSG-WG SA2 Meeting #164	S2-240xxxx

Maastricht, NL, 19th Aug –23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on the work split between SA2 and SA5

Document for:	Discussion

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: This SA2 meeting is supposed to resolve the work split issue between SA2 and SA5 regarding energy topic. This document is trying to analyse the different alternatives that are currently known and proposes a way forward.

1. Introduction

The issue of work split between SA2 and SA5 was raised and discussed in SP#104. According to the related discussions, it seems that a description of the basic idea of the following two options is needed:

-	Calculation in SA2 new functionalities (i.e., SA2-based solution), and

-	Calculation in SA5 functionalities (i.e., SA5-based solution);

To progress the discussion, we would like to propose some analysis and comparison of these options.

2. Discussion

2.0	General Issues

2.0.1	Factors for energy consumption

For the calculation, two issues have to be considered, namely a) the equation used for the calculation (input/output parameters), and b) the place for the calculation.

There are no different views on a) as per the discussion. As demonstrated by the NWM discussion, in this release only UPF/RAN node energy consumption for the UE is considered. From Annex A, the equation used for calculating energy consumption of the UE, ECUE, is as follow (for more details see Annex A, where more consideration is provided):

| ECUE= PDU SessionECUE, PDU Session | (0) |
| --- | --- |

For b), it is mainly related to the way of obtaining the input parameter. The following clauses explain and analyse the possible options based on the above equation.

2.0.2	Accuracy issue

In this release, we use equation (0) in clause 2.0.1 to estimate the energy consumption of a certain UE. As analysed in S2-2404793, there are other factors can be considered as well, e.g., 5GC CP NF, transmission network, and even further differentiation at RAN besides data volume.

However, it is argued that in this release such comprehensive (or more completed energy estimation) is not that needed, since the main purpose of calculating the energy consumption is to identify the NF/UE with energy consumption above average, and to trigger subsequent actions for energy saving, e.g., exposure to AF for controlling, or adjusting the PDU session. Note that it should be rather a "soft" enforcement and therefore the accuracy is not an urgent demand. Usage of energy consumption information for charging purposes is not the main focus of the SA2 work (energy consumption information derived by calculation/estimation is therefore sufficient);

2.0.3	Regarding input from other WGs

The general issue of more specific information from RAN is in principle independent from the SA2/SA5 work split and basic architecture debate. If RAN would provide any type of specific UE/PDU session related information, it would be further processed and taken into account by whatever architecture SA2+SA5 can agree on.

Note that if we don't have any architecture (or solution) for EC information collection, any specific RAN information would be useless. Waiting for input from other WGs (e.g., RAN) will cause us not having enough time to use such input in SA2 since RAN answer will be too late and therefore also RAN work will be useless due to lack of usage.

2.1	Options under discussion

2.1.1	Option#1.1: Calculation at SA2 new functionalities – EECF aggregates the energy consumption result

This option assumes that the SMF is enhanced to handle the finer granularity for the calculation of the energy consumption, i.e., energy consumption of specific PDU Session/QoS Flow. SA2 new function EECF is in charge of all other granularities. All necessary information comes from UPF/OAM.

Figure 2.1.1-1: EECF aggregates the energy consumption result

Step 1-2: SMF gets the EC and DV of UPF and RAN (ECUPF, ECRAN, Data VolumeUPF, Data VolumeRAN) from OAM

SMF gets the energy consumption (EC) and data volume (DV) of the certain RAN node(s) and UPF(s) of the target (i.e., the specific PDU Session, or specific QoS Flow) from OAM via EECF.

NOTE 1:	The triggers for the SMF sending the request can be e.g., from EECF asking for the target PDU Session(s) for calculating the UE energy consumption.

NOTE 2:	The UPF(s) and RAN nodes requested by the SMF for the EC/DV information reporting from EECF could be all UPFs and RAN nodes in the SMF service area, to potentially save the interactions.

NOTE 3:	The interaction between OAM and EECF could be for all UPFs and RAN nodes in the whole network, i.e., the OAM provides the EC and DV info for every active UPF and gNB once per time slot, and the EECF can then distribute/replicate the info to the various SMFs according to their service areas.

Step 3: SMF collects DV of the PDU Session from UPF (Data VolumeUE,PDU Session, UPF)

SMF collects the data volume of the PDU Session from UPF from the N4 message.

Step 4: SMF calculates the EC of the PDU Session

SMF estimates the EC of the target (i.e., ECUE, PDU Session in equation (6) in the Annex).

Where Target can be specific PDU Session or specific QoS Flow of a certain UE.

Step 6: EECF calculates the EC of target

EECF can estimates the EC of the UE by using equation (0).

Step 7-8: EECF exposure to the 5GC NF or external AF.

2.1.2	Option#1.2: Calculation at SA2 new functionalities – EECF calculates the energy consumption result

This option assumes the EECF acts as a centralized function that calculates all different granularities. Similar as option#1.1, the calculation is executed at SA2-defined function, and the EC/DV information are all from OAM/UPF.

Figure 2.1.2-1: EECF calculates the energy consumption result

Step 1-2: EECF gets the EC and DV of UPF and RAN from OAM (ECUPF, ECRAN, Data VolumeUPF, Data VolumeRAN)

These steps are similar as step 1-2 of option#1.1.

Step 3: EECF collects DV of the PDU Session from UPF (RAN ID,UPF ID, Data VolumeUE,PDU Session, UPF)

This step is similar as step 3 of option#1.1.

Step 4: EECF calculates the EC of the PDU Session

Similar as option #1.1, equation (0) can be used by EECF.

Step 5-6: EECF exposure to the 5GC NF or external AF.

2.1.3	Option#2.1: Calculation at SA5 functionalities – Charging System calculates the energy consumption result

This option assumes that the Charging System is enhanced with the functionalities of energy-related calculation, which is mainly to be defined by SA5.

Figure 2.1.3-1: Charging System calculates the energy consumption result

Step 1: Charging System derives DV of the PDU Session from CDR (Data VolumeUE,PDU Session, UPF)

Charging System generates CDR based on the charging information report from SMF by using Nchf_ConvergedCharging (for online and offline charging), or Nchf_OfflineOnlyCharging (for offline only charging).

Charging Systems derives DV of the target (e.g., PDU Session, QoS Flow, UE) from CDR.

Step 2-3: Charging System gets the EC and DV of UPF and RAN from OAM (ECUPF, ECRAN, Data VolumeUPF, Data VolumeRAN)

Charging System gets the energy consumption (EC) and data volume (DV) of the certain RAN node(s) and UPF(s) of the target (i.e., the specific PDU Session, or specific QoS Flow) from OAM, by providing the UPF ID and RAN ID.

The UPF ID and RAN ID can be derived from the CDR in step 1:

-	UPF ID: obtained from UPF ID for 5G data connectivity in TS 32.291 [x3];

-	RAN ID: derived from User Location Information for 5G data connectivity, and the leftmost bits can represent the RAN ID.

Step 4: Charging System calculates the EC of the UE

Charging System estimates the EC of the target (e.g., specific UE). Equation (0) can be used.

Step 5-6: Charging System exposure to the 5GC NF or external AF.

2.1.4	Option#2.2: Calculation at SA5 functionalities – OAM calculates the energy consumption result

This option assumes that the OAM is enhanced with the functionalities of energy-related calculation, which is mainly to be defined by SA5.

Figure 2.1.4-1: OAM calculates the energy consumption result

Step 1-2: OAM gets the EC and DV of UPF and RAN, and the DV of the PDU Session (ECUPF, ECRAN, Data VolumeUPF, Data VolumeRAN,Data VolumeUE,PDU Session, UPF)

These steps are similar as step 1-2 of option#2.1.

Similar as option#2.1, Charging System generates CDR based on the charging information reported from SMF, and provides the UPF ID, RAN ID. In addition, Charging System derives the DV of the target from the CDR.

Charging System provides the above information to OAM.

Step 3: OAM calculates the EC of the UE

Similar as option#2.1, equation (0) can used by OAM.

Step 4: OAM returns the calculated EC of the target to the Charging System

Step 5-6: Charging System exposure to the 5GC NF or external AF.

2.2	Analysis

2.2.1	Applicability to different scenarios

The input/output and calculation of SA2-based solution and SA5-based solution are not very different. However, EECF/SMF and OAM/Charging System may have different capabilities of obtaining the necessary information, and also the efforts of enhancement need to be considered as well.

The following table 2.2.1-1 summarizes the further efforts to be done for different scenarios.

Table 2.2.1-1: Applicability to different scenarios

| Scenario | Description | Applicability |
| --- | --- | --- |
| Single PSA with multiple intermediate UPFs | Case#1: URLLC, 5GC may choose to use multiple N3/N9 path for sending the traffic to RAN to guarantee the reliability Case#2: PSA cannot connect to NG-RAN serving the UE directly, I-UPF is needed. | - Option#1: easy to detect the I-UPF of the PDU Session by SMF. - Option#2: information of I-UPF is not available in CHF/OAM, needs some enhancements. For this scenario, the OAM/Charging System needs to know the identity of the I-UPF, which are not available in current CHF. |
| Roaming (or deployments topologies with specific SMF Service Areas) | Although not well touched in this analysis, it has been not discussed whether roaming scenario shall be supported in this release. | Local breakout: - Option#1: EECF/SMF in the VPLMN can figure out ECUE, of roaming UE and EECF exposes to EECF in HPLMN. - Option#2: Charging System can calculate ECUE and CHF of VPLMN exposes to HPLMN. - Note: in case of calculation in OAM there is no mechanism for OMA in VPLMN to expose information to OMA in HPLMN. Home-routed: - Option#1: For Home-routed, EECF/SMF in VPLMN can figure out EC contribution of gNB and UPF in the VPLMN, while HPLMN EECF figures out ECUE by aggregating the parts in VPLMN received from EECF/SMF in VPLMN and EC contribution of UPF in HPLMN respectively. - Option#2: Current interface provided by V-CHF cannot be used directly for H-CHF, N40 (CHF-SMF) and N16 (V-SMF) have to be enhanced. |
| Control Plane energy consumption | Equation (0) considers the energy consumption in 5GC UP part only, the other aspects, e.g., AMF, SMF part are not considered due to the small portion of EC. However, in the future such part is to be added for the cases e.g., IoT, or more accuracy is needed. | Note that for 5GC CP NF, currently there is no determined metric to evaluate the portion of EC of a certain UE/PDU Session/QF of EC, but it is likely to use e.g., # signalling of the UE, to evaluate. - Option#1: EECF/SMF fetches the 5GC CP NF from OAM and gets the information for specific granularity (e.g., # served UEs) directly from 5GC CP NF in SBI manner; - Option#2: to get the information for specific granularity (if not available at Charging System/OAM), Charging System/OAM is supposed to get such info from 5GC CP NF and the new interface for the management/charging field has to be introduced. |

2.2.2	Comparison of the options

The advantage and disadvantage of the solutions of option #1 (i.e. option #1.1 and #1.2) and #2 (i.e. option #2.1 and #2.2) are summarized as follows:

Table 2.2.2-1: Comparison of the options

| Options | Pros | Cons |
| --- | --- | --- |
| Option #1 (#1.1 and #1.2) | - Can be easily enhanced for the potential features introduced in the future (e.g., control plane energy consumption, …); - Exposure via NEF and usage in conductions with existing exposure framework (e.g., QoS influence); - Fast and CN internal decision making and reaction possible; In addition, if Option #1.1 is used: - Achieves scalability for the smaller granularities, i.e., per QoS Flow / per application flow; - Enables re-use of calculation functionality (which is necessary for exposure) when adding further energy related functionality to SMF, like adding energy related info to Charging System CDR, energy credit, (short term) energy limit; - SMF can easily handle/hide all complex user plane scenarios: EC, I-SMF, I-UPF, roaming, offload, ATSSS; | - New NF and/or enhancement of existing NFs with the required functionalities. |
| Option #2 (#2.1 and #2.2) | - Relatively small enhancements for SMF to be done in CN (in order to generate charging data message at UE mobility and to reset the counters as well as to provide additional user plane related information like I-UPF usage); | - Exposure interface would have to be supported by Charging System (run-time, different granularities, security aspect, new services, …); - Possible increased frequency of interaction between Charging System and 5GC to satisfy the purpose of e.g., policy control. - Higher efforts for potential enhancements in the future (e.g., any new use case needs (e.g., CP functions, new requirement in future releases) to go into CDRs and requires SA5 standardization); - Enhancement of charging system for collection/calculation of the Energy consumption information, which is not its original functions. In addition, the processing of a lot of additional information requires performance upgrades or a stretch in time. - Charging System and 5GC may belong to different business unit of operator, it is hard and costly to meet the need of 5GC purpose by depending on charging system. may come from specialized vendor and has different life cycle - Unlikely that SA5 SWG to have a new WID/SID, or updating existing WID to support the feature of option 2 in Rel-19, which means KI#2/KI#3 can also not be supported in this release. - Specific configuration required so that the SMF counts the user plane traffic according to the requirements for energy consumption calculation (e.g. free of charge traffic to be counted). Potential conflicts with configuration for charging to be checked (e.g. combined counting for charging purposes may not be applicable for energy consumption calculation). - CN decision making only with certain delay (as measurements have to be collected and processed by charging system) - If the current accuracy of estimation of EC is considered not suitable for charging purpose, it is a contradiction to perform evaluation of EC in Charging System for a purpose which is limited to actions to be performed in CN and not for the key role of Charging System, i.e. the charging. |

3. Conclusion and proposal(s)

There is no doubt that for calculating the EC for a certain UE/PDU Session/QoS flow, the information from SA5 managed field is needed (i.e., EC/DV of UPFs and gNBs from OAM). However, purely relying on SA5 to provide the final result of EC may face several issues (as highlighted in clause 2.2.2) and would therefore limit the current/future usage of the feature.

As per the above-mentioned analysis, option #1 has advantages with respect to 1) scalability, 2) forward compatibility, 3) Timely provisioning, and 4) more information available for session control.

Therefore, it is proposed to design the EC calculation mechanism based on the principles of option #1 in Rel-19. Ideally, option#1.1 is chosen as baseline for the normative work (as it has the most advantages) but this can also be further discussed and decided next.

Annex A:
Equation for calculating energy consumption

A.1	The way of calculation

Take UE granularity as an example, the ECUE is counted as follows:

| ECUE= ECUE, UPF+ECUE, RAN | (1) |
| --- | --- |

Where ECUE,  UPF is the energy consumption of the UE at UPF, and can be derived as follows:

| ECUE, UPF= PDU SessionECUE, PDU Session, UPF | (2) |
| --- | --- |
| =PDU SessionData VolumeUE,PDU Session, UPFData VolumeUPF×ECUPF | (3) |

Where:

-	ECUE, PDU Session, UPF is the energy consumption at the UPF for serving the PDU Session of the UE;

-	Data VolumeUE,PDU Session, UPF is the data volume of a certain PDU Session of the UE sent by the UPF;

-	Data VolumeUPF is the total data volume sent by the UPF;

-	ECUPF is the total energy consumption of the UPF.

And similarly, ECUE, RAN is the energy consumption of the UE at RAN, and can be derived as follows:

| ECUE, RAN= PDU SessionECUE, PDU Session, RAN | (4) |
| --- | --- |
| =PDU SessionData VolumeUE,PDU Session, RANData VolumeRAN×ECRAN | (5) |

Where:

-	ECUE, PDU Session, RAN is the energy consumption at the RAN for serving the PDU Session of the UE;

-	Data VolumeUE,PDU Session, RAN is the data volume of a certain PDU Session of the UE sent by the RAN;

-	Data VolumeRAN is the total data volume sent by the RAN;

-	ECRAN is the total energy consumption of the RAN.

By combining (3) and (5):

| ECUE= PDU SessionData VolumeUE,PDU Session, UPFData VolumeUPF×ECUPF+Data VolumeUE,PDU Session, RANData VolumeRAN×ECRAN | (6) |
| --- | --- |

Note that (6) is also equal to the following

| ECUE= PDU SessionECUE, PDU Session | (7) |
| --- | --- |

And ECUE, PDU Session is the energy consumption of the network for serving the PDU Session of the UE;

And it can be assumed that Data VolumeUE,PDU Session, RAN is almost the same as Data VolumeUE,PDU Session, UPF.

The evaluation of the EC for UE may also be performed considering for the Data Volume per UE per UPF as in (8), but the Data Volume of UE in the UPF is equal to the sum of the Data Volume of all PDU sessions of the UE in the UPF in (3).

| ECUE, UPF= Data VolumeUE, UPFData VolumeUPF×ECUPF=PDU Session,UPFData VolumeUE, PDU session, UPFData VolumeUPF×ECUPF = PDU Session, UPFData VolumeUE,PDU Session, UPFData VolumeUPF×ECUPF | (8) |
| --- | --- |