3GPP TSG-WG SA2 Meeting #164	S2-2408355

Maastricht, NL, 19th Aug – 23rd Aug, 2024	(revision of S2-240xxxx)

Source:	Huawei, HiSilicon

Title:	KI#3 Conclusion: Update of conclusion of KI#3

Document for:	Approval

Agenda Item:	19.4

Work Item / Release:	FS_EnergySys / Rel-19

Abstract: It is proposed to update KI#3 to clarify the use case.

# 1. Introduction/Discussion

The agreed conclusion of KI#3 in the last meeting provides a good basis for the normative work.

We think it is also beneficial to support the analytic feature in this release – it is a low hanging fruit and the use case and solution is clear. However, we think it is not proper to provide such information to AMF/SMF for making the MM/SM decision, since those 5GC NF are assume to have a more "deterministic" output.

PCF and AF could be a proper destination for making use of the analytics provided by NWDAF.

-	On PCF, there are already some existing mechanisms for PCF to use output from NWDAF for policy decision (see clause 7.1 of TS 23.288).

-	On AF, according to the requirement in clause 6.15a.5.2 of TS 22.261, "Based on operator’s policy and agreement with 3rd party, the 5G system shall be able to expose energy consumption information and prediction on energy consumption of the 5G network per application service to the 3rd party."

Regarding the output, we suggest to make it more focused, i.e., prediction on energy consumption of the 5G network.

Meanwhile, the agreed slice admission control part was removed from the conclusion of KI#3 in the last meeting. One discussion was on whether operator is able to change the maximum number of UEs/PDUs of a network slice. The other discussion was on the differences between the slice Admission control by NSACF on the PDU session and related enforcement at the SMF. This paper clarifications the difference and proposes some rewording on the original text in the conclusion.

-	For the first discussion, the maximum number of UEs/PDUs of a network slice are defined in the SLA. According to operator policy and agreement in the SLA, the maximum number of PDUs of a network slice may use a minimum number to be guaranteed, a range, or multiple levels subject to different charging policy. It is therefore proposed to add “if allowed by SLA” as a pre-condition for EC related maximum number of UEs/PDUs adjustment.

-	For the second discussion, NSACF has already supported the checking against maximum number of UEs/PDUs of a network slice, it is straight forward to extend that to support also the checking against energy consumption related thresholds of a network slice. Such checking results, however, is on the slice level. The individual session management decision (e.g., admit/reject a PDU session, potentially with the modification of the admitted PDU session or release of an established PDU session in a network slice), should still be made and enforced by the SMF based on related policy. It is therefore proposed to keep the original sentence with small editorial changes for readability.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-66.

# * * * * First change * * * *

## 8.3	Conclusion for Key Issue #3: 5GS enhancements for network energy saving and efficiency

It is proposed to procced to normative work based on the following principles:

1)	For Enhancements on NF discovery and (re-)selection based on energy related information:

-	NF profile may be extended (e.g. include the new energy related information or reuse existing NF profile parameters) to allow an operator to influence NF discovery and selection based on their energy strategy.

NOTE 1:	Which energy related information is included in the NF profile is to be further discussed in the normative phase. Only parameters captured in existing solutions will be considered. Coordinated with SA WG5 may be needed.

NOTE 2:	The information within NF profile is expected not to change frequently, e.g. constantly reflecting changes in energy consumption. The procedure for updating the information will be addressed in normative phase.

-	NF discovery and (re-)selection is enhanced to consider the energy related information from the NF profiles and/or discovery request from the NF consumer.

2)	For enhancements on existing operations and procedures for energy saving and energy efficiency:

-	UP path of PDU session may be adjusted.

-  The management of the PDU Session belongs to the network slice subject to energy control may consider the energy related information.

-	Maximum number of UEs and maximum number of PDU sessions for a network slice maybe adjusted by considering energy related information if allowed by SLA.

NOTE 3:	The energy related decision will also consider operator’s policy.

3)	For NWDAF-Based Energy Analytics for network energy saving and efficiency:

-	Analytics information from NWDAF is enhanced to assist the PCF in making network energy saving related decisions.

-	Analytics information from NWDAF may be used to assist an AF to make energy saving related decisions.

The input and output for analytics are energy consumption of individual UE(s) and their location during historical period of time, and prediction of energy consumption of UE or group of UEs or areas, respectively.

NOTE 4:	The detail use cases on energy related analytics, corresponding input and output data and the energy consumption of generating analytics and energy saving mechanisms will be addressed in the normative phase.

NOTE 5: Energy related information required and obtained to support 5GS enhancement for energy saving and energy efficient is dependent on KI#1 conclusions.

# * * * * End of changes * * * *