3GPP TSG-WG SA2 Meeting #164	S2-240835

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | draftCR | rev | X | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Energy system support |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | Dummy |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Support the energy system functionalities per conclusion of TR 23.700-66 in v1.0.0. The changes are based on the following assumptions resumed d in the figure and the list below: - Evaluation of energy related parameter for UE granulalrity is performed in a a new NF called EECF and for PDU session and QoS flow granulalrity in SMF. - the energy related parameter for NF are passed by MnS to the EECF |  |  |  |  |  |  |  |  |
| Summary of change: |  | The following additions are included: - in clause 4.2.2 new NF called EECF - new clause 4.2.x with the reference Architecture to support Energy System |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2, 4.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 4.2.2	Network Functions and entities

The 5G System architecture consists of the following network functions (NF):

-	Authentication Server Function (AUSF).

-	Access and Mobility Management Function (AMF).

-	Data Network (DN), e.g. operator services, Internet access or 3rd party services.

-	Unstructured Data Storage Function (UDSF).

-	Network Exposure Function (NEF).

-	Network Repository Function (NRF).

-	Network Slice Admission Control Function (NSACF).

-	Network Slice-specific and SNPN Authentication and Authorization Function (NSSAAF).

-	Network Slice Selection Function (NSSF).

-	Policy Control Function (PCF).

-	Session Management Function (SMF).

-	Unified Data Management (UDM).

-	Unified Data Repository (UDR).

-	User Plane Function (UPF).

-	UE radio Capability Management Function (UCMF).

-	Application Function (AF).

-	User Equipment (UE).

-	(Radio) Access Network ((R)AN).

-	5G-Equipment Identity Register (5G-EIR).

-	Network Data Analytics Function (NWDAF).

-	CHarging Function (CHF).

-	Time Sensitive Networking AF (TSN AF).

-	Time Sensitive Communication and Time Synchronization Function (TSCTSF).

-	Data Collection Coordination Function (DCCF).

-	Analytics Data Repository Function (ADRF).

-	Messaging Framework Adaptor Function (MFAF).

-	Non-Seamless WLAN Offload Function (NSWOF).

NOTE:	The functionalities provided by DCCF and/or ADRF can also be hosted by an NWDAF.

-	Edge Application Server Discovery Function (EASDF).

-	Energy Efficency Control Function (EECF)

The 5G System architecture also comprises the following network entities:

-	Service Communication Proxy (SCP).

-	Security Edge Protection Proxy (SEPP).

The functional descriptions of these Network Functions and entities are specified in clause 6.

-	Non-3GPP InterWorking Function (N3IWF).

-	Trusted Non-3GPP Gateway Function (TNGF).

-	Wireline Access Gateway Function (W-AGF).

-	Trusted WLAN Interworking Function (TWIF).

# * * * * Second change * * * *

### 4.2.x	Architecture to support Energy System

In order to support the Energy System feature, the 5G System Architecture is extended as shown this clause. The additional functionality that is supported by the network functions shown in these figures is specified in clause 5.x. In summary:

#### 4.2.x.1	Non-roaming architecture

Figure 4.2.x.1-1 depicts the non-roaming architecture for Energy Efficeincy Control Function (EECF) using SBI representation. The figure shows only the NFs enhanced to support the Energy System functionality.

Figure 4.2.x.1-2 depicts the non-roaming architecture for Energy Efficeincy Control Function (EESF) using reference point representation. The figure shows only the direct interface of EECF with other NFs.

The interface between the EECF and the MnS enables the retrival of the required informaiton in order to perfomr the evalution of the energy related parameters as defined in clause 5.x.

Editor’s Note: the defintion of the interface between the MnS and EECF is subsject to further discussion with SA5.

Figure 4.2.3-1: Non-Roaming 5G System Architecture

Figure 4.2.x.1-2: Non-Roaming Architecture for Energy System Function in reference point representation

Editor’s note: the reference point name from Nx1 to Nx6 in Figure 4.2.3-x shall be replace with the proper number from N110 onward.

NOTE x:	In Figure 4.2.3-x, The reference points between the other NFs are not shown.

#### 4.2.x.2	Roaming architecture

Editor’s note: the Roaming reference architecture is FFS.

#### 4.2.x.4	Service-based interfaces

The 5G System Architecture contains the following service-based interfaces:

Nesf:	Service-based interface exhibited by ESF

Editor’s Note: the defintion of the interface between the MnS and EECF is subsject to further discussion with SA5.

#### 4.2.x.5	Reference points

The description of the reference points specific for Energy System are:

Nx1:	Reference point between EECF and UDM.

Nx2:	Reference point between EECF and AMF.

Nx3:	Reference point between EECF and SMF.

Nx4:	Reference point between EECF and PCF.

Nx5:	Reference point between EECF and NEF.

Nx6:	Reference point between EECF and MnS.

Editor’s note: the reference point name from Nx1 to Nx6 in Figure 4.2.3-x shall be replaced with the proper number from N110 onward.

Editor’s note: The Nx6 reference point between the ESF and the MnS is defined in coordination with SA5 and it is FFS.

# * * * * End of changes * * * *