3GPP TSG-WG SA2 Meeting #164	S2-2408384

# Maastricht, NL, 19th Aug – 23rd Aug, 2024		(revision of S2-240xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 5572 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of UE-satellite-UE communication via UPF and IMS-AGW deployed on satellite |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_ARCH |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on 5GSAT_Ph3_ARCH WID work task, the functionality of UE-satellite-UE communication via UL CL/BP, local PSA UPF and IMS-AGW deployed on satellite should be capatured by TS 23.501. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new paragraph added to describe how to use procedures defined in TS 23.502 to support of the functionality of UE-satellite-UE communication via UL CL/BP, local PSA UPF and IMS-AGW deployed on satellite in regenerative architecture. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how to support UE-satellite-UE communication via UL CL/BP, local PSA UPF and IMS-AGW deployed on satellite. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 5.4.11	Support for integrating NR satellite access into 5GS

#### 5.4.11.x	Support of UE-satellite-UE communication via UL CL/BP, local PSA UPF and IMS-AGW deployed on satellite in regenerative architecture

When two UEs are served by the same PLMN in their home network and both served by NR satellite access using regenerative architecture, their IMS media traffic may be routed by UL CL/BP(s) and local PSA UPF(s) deployed on satellite(s) to their IMS-AGW(s) which are also deployed on satellite(s).

-	During IMS PDU session establishment procedure of the UE with NR satellite access, the SMF shall select a on-ground UPF as the PSA UPF.

-	During the IMS call setup procedure, after receiving a request from the P-CSCF for access network information report, the PCF for the UE subscribes to Access Network Information report from the SMF for the IMS PDU Session, as described in clause 6.1.3 of TS 23.503 [45].

-	Based on Application Function influence on traffic routing procedures described in clause 4.3.6 of TS 23.502 [3], upon receiving a Npcf_SMPolicyControl_UpdateNotify message from the PCF that contains an IP address of the IMS-AGW and DNAI associated with the satellite serving the UE, the SMF inserts an UL CL/BP and local PSA UPF on the satellite associated to the DNAI and installs the corresponding N4 rules to direct the IMS media traffic to the IMS-AGW on satellite.

NOTE 1: It is assumed that each satellite is assigned dedicated DNAI(s).

-	When the gNB onboard the satellite serving one of the two UEs involved in a UE-Satellite-UE communication changes (e.g. due to the satellite movement), the UL CL/BP, L-PSA and IMS-AGW shall be relocated to a target satellite or removed from satellite. The SMF should report user plane management event (i.e., change of DNAI(s) on satellite(s) or to ground) to P-CSCF based on the procedure of notification of user plane management event described in clause 4.3.6.3 of TS 23.502 [3].

-	To coordinate the IMS-AGW relocation at the P-CSCF with the UL CL/BP and L-PSA relocation to a target satellite, the simultaneous change of UL CL/BP, L-PSA procedure in clause 4.3.5.6 or 4.3.5.7 of TS 23.502 [3] is performed at step 3 of Figure 4.3.6.3-1 in TS 23.502 [3].

-	To coordinate the IMS-AGW relocation at the P-CSCF with the UL CL/BP and L-PSA removal, the procedure of UL CL/BP and L-PSA removal in clause 4.3.5.5 of TS 23.502 [3] is performed at step 3 of Figure 4.3.6.3-1 in TS 23.502 [3].

# * * * * End of changes * * * *