SA WG2 Meeting #S2-164	S2-2408393

19 - 23 August, Maastricht, Netherlands	(revision of S2-240xxxx)

Source:	OPPO, vivo, China Telecom, Xiaomi, CATT

Title:	FS_MASSS Interim conclusion for KI #1.4 on policy enhancement for DualSteer

Document for:	Approval

Agenda Item: 	19.13

Work Item / Release:	FS_MASSS /Rel-19

Abstract of the contribution: This paper proposes interim conclusion for KI #1.4 for the FS_MASSS TR 23.700-54.

1	Discussion

This paper proposes interim conclusion for KI #1.4 for Dual Steer.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V1.0.0.

* * * * Start of Changes (All text new) * * * *

8.1.X Interim Conclusions for Key Issue #1.4

The following interim conclusions are agreed for principles of Key Issue #1.4 "Policy enhancements for DualSteer":

UE policy for DualSteer (e.g. new defined Access Selection Policy or enhanced URSP) can be used to select a registered access network (i.e. registered UE) for traffic steering. The URSP rule is used to associate the Application traffic to a PDU session based on the evaluation as defined in clause 6.6.2.3 of TS 23.503[5].

For provisioning of UE policy for DualSteer:

UCU procedure is used for the delivery of UE policy for DualSteer for the two SUPIs in same device.

NOTE 1:	Whether extends URSP or uses new UE policy, e.g., ASP, and what information (e.g., updated policy or authorization information) is notified to PCF is determined in normative phase.

For the content of UE policy for DualSteer:

The DualSteer device performs DualSteer traffic steering based on the UE policy. The UE policy includes the following information:

-	Traffic descriptors;

-	Access network selection descriptors: PLMN ID and/or RAT under each Traffic descriptor for DualSteer traffic steering.

NOTE 2:	Other information, e.g., validity information, DualSteer indication etc., is determined in normative phase.

-	Priority level for each traffic descriptor for non-simultaneous transmission cases;

* * * * End of Changes * * * *