SA WG2 Meeting #164	S2-2408398

Maastricht, NL, 19 - 23 August, 2024	revision of S2-240yyyy

Source:	vivo, China Telecom, OPPO, CATT, Xiaomi

Title:	Interim conclusions for DualSteer KI#1.3

Document for:	Approval

Agenda Item: 	19.13.1

Work Item / Release:	FS_MASSS / Rel-19

Abstract of the contribution: This paper proposes interim conclusions on KI#1.3.

1	Discussion

Please see S2-2408395 for discussion on the way forward for DualSteer.

This paper proposes interim conclusions on KI#1.3.

2	Proposal

It is proposed to include the following changes in TR 23.700-54 V1.0.0.

* * * * Start of Changes (all new text) * * * *

### 8.1.3	Conclusions on Key Issue #1.3

The following interim conclusions are agreed for principles of Key Issue #1.3 "Session management aspects for DualSteer":

A. Establishment of a PDU Session for DualSteer:

1)	If access network selection information (e.g., RAT list) for a data service is indicated in the UE policy for DualSteer, the DualSteer device selects one of the two registered UEs for transferring the data service based on the access network selection information.

2)	When establishing a PDU Session according to the UE policy for DualSteer, the DualSteer device indicates to the AMF that the PDU Session is for DualSteer. The AMF selects an SMF supporting DualSteer and sends the indication for DualSteer to the SMF.

3)	During the PDU Session Establishment procedure, if the indication for DualSteer is received, the SMF indicates authorization result to the DualSteer device based on the authorization information (e.g., switch allowed or not) specific to the DNN/S-NSSAI of the PDU Session in subscription data.

B. Establishment of a target PDU Session for DualSteer traffic switching:

1)	For performing DualSteer traffic switching, the DualSteer device establishes a target PDU Session of target SUPI with same (DNN, S-NSSAI) combination as the source PDU Session established by a source SUPI.

2)	During the PDU Session Establishment procedure, the DualSteer device provides source PDU Session ID to the AMF. The AMF selects SMF, which is same as source PDU Session, based on "UE context in SMF" subscription data of the source SUPI (i.e., linked SUPI in subscription data) provided by the UDM and the source PDU Session ID.

3)	During the PDU Session Establishment procedure, the AMF sends session association information, i.e., the source PDU Session ID and the source SUPI, to the SMF. Based on the subscription data and the session association information, the SMF anchors the target PDU Session on the same UPF of the source PDU Session and allocates same IP address as the source PDU Session.

4)	During the PDU Session Establishment procedure, the DualSteer device may indicate pre-establishment or not to the SMF.

a)	In case of pre-establishment, the SMF does not reserve UP resources of the target PDU Session.

b)	Otherwise, the SMF instructs the UPF to switch the DL traffic through the target PDU Session.

5)	During the PDU Session Establishment procedure, the DualSteer device may indicate dual-active or not to the network. In case that dual-active and pre-establishment are not indicated, the SMF deactivates or releases the source PDU Session.

C. Switching when target PDU Session for DualSteer exists and is inactive (e.g., in the case of switching back):

1)	For performing DualSteer traffic switching, the DualSteer device initiates Service Request to activate the target PDU Session(s). The SMF(s) deactivates or releases the source PDU Session(s), and instructs the UPF(s) to switch the DL traffic through the target PDU Session(s).

* * * * End of Changes * * * *