3GPP TSG-SA2 Meeting #164	S2-2408402

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 4968 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TEI19 User consent management for MSISDN exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI19 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, SA2 agreed to support MSISDN exposure, considered as sensitive information, via NEF Nnef_UEId service and to support UDM-based user consent check for the MSISDN exposure. However, the following issues on user consent management for MSISDN are identified. - Upon revocation of the user consent for MSISDN exposure, the notification for user consent revocation to the AF (that had retrieved MSISDN via Nnef_UEId) is not supported. Some regional regulation or operator’s policy require the enforcement of user consent revocation notification from CN to AF. This requirement is also addressed in Annex V of TS 33.501, but the corresponding procedure is not supported for the MSISDN exposure. - Rel-18 user consent check for MSISDN exposure relies on the manually preconfigured user consent in UDM. As the user consent for MSISDN exposure can be given or revoked, it is required to support a procedure for user consent setting in UDM, similarly to UE location privacy profile setting in TS 23.273. The procedure for the UE location privacy profile seeting can be reused for user consent setting for MSISDN exposure. To address the above issues, this CR proposes (i) to update Nnef_UE_Id service to include user consent revocation notification address information as input parameter and (ii) to introduce user consent setting procedure for MSISDN exposure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Nnef_UE_Id service Introduce user consent setting procedure for MSISDN exposure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User consent management for MSISDN exposure remains uncomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.10A, 4.15.10B (new), 5.2.6.4.2, 5.2.3.6.2, 5.2.6.27.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR is for TEI19 WID (S2-2408403) |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# *** 1st Change ***

4.15.10A	 MSISDN retrieval

This clause contains the detailed description and procedures for UE ID retrieval in the GPSI format of MSISDN as defined in TS 23.003 [33] by AFs. Depending on operator policy and local regulation, GPSI in MSISDN format may be exposed through the NEF to an authenticated and authorized AF. Depending on operator policy and local regulation, user consent may be required when exposing MSISDN. For the corresponding user consent check, one of the followings is used:

-	if CAPIF, defined in TS 23.222 [54], is deployed and RNAA feature is supported, then RNAA, defined in clause 6.5.3 of TS 33.122 [95], is used; or

-	the UDM is used to check user consent for MSISDN exposure.

The figure and procedures of clause 4.15.10 shall be applicable for MSISDN retrieval with the following differences:

-	description of the AF specific UE ID retrieval is replaced as the MSISDN retrieval;

-	description of the AF specific UE Identifier represented as an External Identifier is replaced as the UE Identifier in the GPSI form of MSISDN; and

-	in step 10, the UDM checks the user consent for MSISDN exposure based on the operator policy.

The user consent for MSISDN exposure may be provided and updated (i.e., revoked), e.g., after the MSISDN was exposed to an AF. If user consent is required when exposing MSISDN, upon the revocation of the user consent, the notification for user consent revocation is sent by NEF to the AF based on the operator policy or local regulation. To support this, the AF provides the User Consent Revocation notification target address to the NEF when invoking the Nnef_UEId_Get for MSISDN retrieval. The NEF identifies the notification target based on the User Consent Revocation notification target address.

The user consent for MSISDN exposure may be revoked via the procedure for the User Consent Setting for MSISDN retrieval as specified in 4.15.10B or by operator’s configuration update of UDM.

# *** Next Change ***

4.15.10B			User Consent setting for MSISDN retrieval

4.15.10B.1	General

This clause contains the detailed description and procedures for User Consent Setting for MSISDN retrieval.

The UE and/or AF can provide and update User Consent parameter to the core network as an update to the UDM/UDR. The User Consent parameter indicates whether the user consent is given for the MSISDN exposure or not.

NOTE:	The User Consent parameter is stored and updated as in clause 4.15.10B.2, if CAPIF is not used. If CAPIF is used for user consent management as specified in TS 33.122 [95], the user consent can be set in the CAPIF core function, which is out of scope of the present document.

4.15.10B.2	User Consent setting initiated by UE

The procedure is specified by reusing the procedure, UE Location Privacy Setting Procedure Initiated by UE, in clause 6.12.1 of TS 23.273 to provide or update the User Consent parameters for MSISDN exposure with the following differences:

-	description of the UE Location Privacy Indication is replaced as the User Consent parameters for MSISDN exposure;

-	description of the UE LCS privacy profile is replaced as the User Consent Information for MSISDN exposure;

-	At step 1, The UE generates or updates the User Consent parameters instead of UE LCS privacy profile updates, and sends the User Consent parameters for MSISDN exposure to the AMF via User Consent setting request in N1 NAS message.

-	At step 2, the AMF invokes a Nudm_ParameterProvision_Update (User Consent parameters for MSISDN exposure) service operation towards the UDM and the service operation carries the User Consent parameters for MSISDN exposure. The UDM stores or updates the User Consent Information for MSISDN exposure in the UDR.

4.15.10B.3	User Consent setting initiated by AF

The procedure is specified by using the procedure in clause 4.15.6.2 of TS 23.502 to provide or update the User Consent parameters for MSISDN exposure.

# *** Next Change ***

5.2.6.27.2	Nnef_UEId_Get operation

Service operation name: Nnef_UEId_Get

Description: Get the UE identifier.

Inputs, Required: GPSI or UE address (i.e. IPv4/IPv6 address or MAC address).

Inputs, Optional: DNN, S-NSSAI, Port number (e.g. TCP or UDP port), IP domain, Application port ID, MTC Provider Information, AF Identifier, User Consent Revocation notification target address (e.g., callback URI).

NOTE 1:	User Consent Revocation notification target address (e.g., callback URI) shall be present if AF hasn’t provided it in advance (e.g. via NEF local configuration or as part of AF onboarding procedure onto CAPIF/NEF).

Outputs, Required: Result, GPSI either as an AF specific UE Identifier represented in the form of an External Identifier, or in the form of a MSISDN or SUPI.

Outputs, Optional: HPLMN DNN, HPLMN S-NSSAI.

NOTE 2:	SUPI can only be exposed to roaming partners, i.e. V-NEF, to support HR-SBO Sessions as described in clause 4.3.6.5.

NOTE 3:	The AF specific UE Identifier in GPSI form of MSISDN can only be exposed to an AF when allowed and authorized by the operator as described in clause 4.15.10A.

Outputs, Optional: None.

*** Next Changes ***

5.2.6.4.2	Nnef_ParameterProvision_Update service operation

Service operation name: Nnef_ParameterProvision_Update

Description: The consumer updates the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, ECS Address Configuration Information) or 5G VN Group related information (e.g. 5G VN group data, 5G VN membership management), or for Multicast MBS related information or DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI.

Inputs, Required: AF Identifier, Transaction Reference ID.

Inputs, Optional: GPSI or UE addressing information, External Group ID, at least one of the Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or at least one of the Application-Specific Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or at least one of the Network Configuration parameters or 5G VN related information or ECS Address Configuration Information, Validity Time or Location Privacy Indication parameters, MTC Provider Information, or Multicast MBS related information, DNN and S-NSSAI specific Group Parameters, DNN, S-NSSAI, PLMN IDs, User Consent parameters for MSISDN exposure.

Outputs, Required: Operation execution result indication.

Outputs, Optional: Transaction specific parameters, if available. External Identifier (representing an AF specific UE Identifier).

For Multicast MBS related information, refer to TS 23.247 [78].

*** Next Changes ***

5.2.3.6.2	Nudm_ParameterProvision_Update service operation

Service operation name: Nudm_ParameterProvision_Update.

Description: The consumer updates the UE related information (e.g. Expected UE Behaviour, Network Configuration parameters, Location Privacy Indication parameters, Ranging/Sidelink Positioning Privacy Indication parameters, Enhanced Coverage Restriction Information, ECS Address Configuration Information), 5G VN group related information (5G VN group data, 5G VN membership management), Multicast MBS group related information, or Application-Specific UE Behaviour.

Inputs, Required: AF Identifier, Transaction Reference ID(s).

Inputs, Optional: GPSI or SUPI, External Group ID, DNN, S-NSSAI, at least one of the Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or at least one of the Application-Specific Expected UE Behaviour parameters (optionally with associated confidence and/or accuracy levels) or at least one of the Network Configuration parameters or 5G VN group related information, MTC Provider Information, Validity Time or Location Privacy Indication parameters or Ranging/Sidelink Positioning Privacy Indication parameters or Enhanced Coverage Restriction Information or ECS Address Configuration Information, Multicast MBS related information, or Application-Specific UE Behaviour, User Consent parameters for MSISDN exposure.

Outputs, Required: Transaction Reference ID(s), Operation execution result indication, or DNN and S-NSSAI specific Group Parameters.

Outputs, Optional: Transaction specific parameters, if available.

For Multicast MBS related information, refer to TS 23.247 [78].

*** End of Changes ***