3GPP TSG-WG SA2 Meeting #164	S2-2408406

# Maastricht, Netherlands, 19th Aug 2024 - 23rd Aug 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1171 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration and Discovery procedure for Vertical Federated Learning among NWDAF(s) and/or AF(s) with NWDAF as the VFL server |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ICS, OPPO, InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Introducing a new FL variant (i.e. Vertical Federated Learning) in 5GC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - [1st Change] Updateting FL capability type to indicate VFL client/server in clause 5.2. - [2nd Change] Adding a new subsection to describe required procedures to perform VFL Registration and Discovery when NWDAF(s) and/or AF(s) are the VFL client with a VFL Server NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new feature that was concluded in the study phase will not be adopted in the related specs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 6.2X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.501, TS 23.502 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

## 5.2 NWDAF Discovery and Selection

In order to discover an NWDAF containing MTLF with Federated Learning (FL) capability via NRF, in addition to the procedures described above for discovering NWDAF containing MTLF:

-	An NWDAF containing MTLF supporting FL as a server shall additionally include FL capability type (i.e. HFL server or VFL server) and may include Time interval supporting FL as FL capability information during the registration in NRF.

-	An NWDAF containing MTLF supporting FL as a client shall additionally include, FL capability type (i.e. HFL client or VFL client) and may include Time interval supporting FL as FL capability information during the registration in NRF, and it may also include, NF type(s) and NWDAF Serving Area information and/or NF set ID(s) of the data source(s) where data can be collected as input for local model training.

NOTE 6:	An NWDAF containing MTLF may indicate to support both FL server and FL client in the FL capability for specific Analytics ID.

-	During the discovery of NWDAF containing MTLF as FL server, a consumer (e.g. a NWDAF containing MTLF) may include in the request the FL capability type as HFL server or VFL server and may include Time Period of Interest and ML Model Filter information for the trained ML Model(s) per Analytics ID(s), if available. The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

-	During the discovery of NWDAF containing MTLF as FL client, a consumer (e.g. an FL server) may include in the request FL capability type as HFL client or VFL client, and may include Time Period of Interest, a list of NF type(s) and/or NF set ID(s) of the data source(s). The NRF returns one or more NF profiles of candidate instances of NWDAF satisfying the query parameters.

NOTE 7:	The service consumer to discover an NWDAF containing MTLF with FL capability is limited to NWDAF containing MTLF in this Release.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

In the roaming architecture, the NWDAF with roaming exchange capability (RE-NWDAF) to request analytics or input data is discovered via the NRF. A consumer in the same PLMN as the RE-NWDAF discovers the RE-NWDAF(s) by querying for NWDAF(s) where the roaming exchange capability is indicated in its (their) NF profile. A consumer in a peer PLMN (i.e. RE-NWDAF) discovers the RE-NWDAF(s) by querying for NWDAF(s) in the target PLMN that is (are) supporting the specific services defined for roaming. A RE-NWDAF discovers the RE-NWDAF(s) in a different PLMN (i.e. HPLMN or VPLMN) using the procedure defined in clause 4.17.5 (if delegated discovery is not used) or clause 4.17.10 (if delegated discovery is used) of TS 23.502 [3], where the detailed parameters are determined based on the analytics request or subscription from the consumer 5GC NF, operator policy, user consent and/or local configuration.

# * * * * 2nd change * * * *

#### 6.2X.X.X	Registration and Discovery procedure for Vertical Federated Learning among NWDAF(s) and/or AF(s) with NWDAF as the VFL server

Figure 6.2X.X.X-X: Registration and Discovery procedure for Vertical Federated Learning when NWDAF is the VFL server and NWDAF(s) and/or AF(s) are the VFL client.

Steps 1 to 3 are the NWDAF and AF Registration procedures when the VFL server is NWDAF.

1-3.	VFL Server/Client NWDAF (containing MTLF)  and VFL Server/Client AF (containing model training capability) registers to NRF with its NF profile, which includes NF Type (see clause 5.2.7.2.2 of TS 23.502 [X]), Analytics ID(s), Address information of NWDAF/AF, Service Area, FL capability type (e.g. VFL server and/or VFL client), VFL capability information (e.g. Vendor ID(s), ML Model ID(s), [optional] VFL feature description information), and Time interval supporting FL as described in clause 5.2.

For an untrusted AF, the NEF registers based on configuration at the NRF within its NF profile information about the AF as specified in clause 6.2.2.3 and includes other information related to the AF capability including FL capability type (e.g. VFL server and/or VFL client), VFL capability information (Vendor ID(s)), supported ML model(s), [optional] VFL feature description information).

NOTE: The VFL feature description information may be locally configured in the NWDAF or the AF, and it is negotiated between VFL server and VFL client when performing feature alignment which is an optional feature in the current release.

Steps 4 to 6 are the NWDAF and AF Discovery procedures when the VFL server is NWDAF.

4-6.	NWDAF containing MTLF (as the VFL server) determines that the ML Model requires VFL based on operator policy (e.g., a pre-configured list of ML Models), Analytic ID, Service Area/DNAI, data can not be obtained directly from data producer NF (e.g., due to privacy reasons).

NOTE: Step 4 in Figure 6.2X.X.X-X may be triggered at the VFL Server NWDAF by VFL server NWDAF itself or a request from an analytics consumer, such as an NWDAF containing AnLF. An analytics consumer may also provide a list of candidate samples (UE IDs) per Analytics ID to the VFL Server NWDAF, or the network operator may pre-configure a list of candidate samples (UE IDs) per Analytics ID at the VFL Server NWDAF.

If the NWDAF containing MTLF can not perform as VFL Server NWDAF, the MTLF first discovers and selects VFL Server NWDAF from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the VFL ML Model required, ML Model filter information as defined in clause 6.2A.2, Time Period of Interest, Service Area, FL capability Type (e.g. VFL server and/or VFL client), and VFL capability information if available (e.g. Vendor ID(s), supported ML model(s)).

Once the VFL Server NWDAF is determined, the VFL Server NWDAF discovers other NWDAF(s) containing MTLF and/or AF(s) as VFL Client from NRF by invoking the Nnrf_NFDiscovery_Request service operation. The following criteria might be used: Analytic ID of the ML Model required, FL capability Type (i.e. VFL client), VFL capability information if available (e.g. Vendor ID(s), supported ML model(s), feature list), Service Area, NF type(s) of data sources from which the VFL Client NWDAF is able to collect data for local ML Model training, Time Period of Interest, and optional sample list (i.e. list of UE IDs to engage in VFL operations). The sample list (i.e. a list of UE IDs to engage in VFL operations) can also be pre-configured at the VFL Server NWDAF by the operator, or it can be received by a VFL analytics consumer (e.g. an NWDAF containing AnLF). Once the sample list is provided, the NRF discovers the VFL Client NWDAF(s) and AF(s), which are serving corresponding UEs in the sample list.

7. 	When the VFL Server NWDAF receives Nnrf_NFDiscovery_Request Response, it may select a set of VFL Client NWDAF(s) and/or VFL Client AF(s) according to some criteria such as a sample list (i.e. UE IDs) if available.

8.	VFL Server NWDAF sends Federated Learning preparation request to the VFL NWDAF Client(s), using Nnwdaf_MLModelTraining_Subscribe or Nnwdaf_MLModelTrainingInfo_Request service, and/or the VFL AF Client(s), using Naf_MLModelTraining_Subscribe or Naf_MLModelTrainingInfo_Request service, with the ML Preparation Flag, to check if the VFL Client NWDAF(s) and VFL Client AF(s) can meet the ML Model training requirement (e.g. Analytics ID, Data Availability requirement, FL Availability time requirement (time span needed for the FL process), etc.). Data Availability requirement includes a list of Event IDs of the local data for training, and may also include the dataset statistical properties, the time window of the data samples and the minimum number of data samples. In the case of untrusted AF Client(s), the Nnef _MLModelTraining_Subscribe or NnefMLModelTrainingInfo_Request service can be used by the VFL NWDAF Server with the ML Preparation Flag.

NOTE:	Vertical Federated Learning preparation procedure (i.e. steps 8-10) can be skipped if the VFL Server NWDAF can decide that the VFL Client(s) supports the VFL procedure to be performed, e.g. based on information acquired from previous VFL procedures or from the NRF or based on local configuration.

9.	VFL Client NWDAF(s) and/or VFL Client AF(s) checks if it can meet the ML Model training requirement and decides whether to join the Federated Learning process based on operator policy (e.g. pre-configured list of ML Models) and/or implementation. Example criteria used by VFL Client NWDAF(s) may be based on its data availability and time availability, computation and communication capability, etc.

10.	VFL Client NWDAF(s) invokes Nnwdaf_MLModelTraining_Notify or Nnwdaf_MLModelTraining_Subscribe response or Nnwdaf_MLModelTrainingInfo_Request response service operation, and/or the VFL Client AF(s) invokes Naf_MLModelTraining_Subscribe response, Naf_MLModelTraining_Subscribe response, to indicate to the VFL Server NWDAF whether it will join the VFL procedure and may include the reason in the response message if it cannot join the VFL process. The untrusted VFL Client AF(s) invokes Nnef_MLModelTraining_Notify or Nnef_MLModelTraining_Subscribe response or Nnef_MLModelTrainingInfo_Request response service operation.

11.	VFL Server NWDAF determines the VFL Client NWDAF(s) and/or VFL Client AF(s) to be involved in the VFL procedures based on the information received in step 6 and other information received in step 10 (if available).

Editor's note:	Steps 8-11 may be further updated depending on agreements on how the sample and feature alignment procedure should be implemented.

# * * * * End of changes * * *