3GPP TSG-WG2 Meeting #164	S2-2408407

# Maastricht, Netherlands, August 19 - 23, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1445 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#1: Supporting IMS events in the context of DC communication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2024-08-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | TR 23.700-77 has concluded the KI#1 - Extensible IMS mechanism supporting IMS events in the context of DC communication. The CR provides the necessary changes to TS 23.228. The study concludes two kinds of events: subscriber specific and non-subscriber specific. In the former case, the subscribed events are stored in HSS, and delivered to the IMS AS upon user registration. In the latter case, the subscribed events are delivered directly to the IMS AS (every IMS AS in the PLMN). This leads to two different sets of procedures, thus complicating the solution. It is proposed to simplify the above by noticing that "non-subscriber specific events" can be implemented by a "subscriber specific event" that is targeted to any user in the network (e.g. wildcarded user identity set to e.g. *@operator.com). The events are stored in both cases in the HSS in the same manner. It should be noted that the TR has concluded that also "subscriber specific event" can be targeted to a wildcarded user identity, in which case the data must be provided to all relevant HSS instances. The study concludes that upon user registration to the IMS, the IMS AS registers its address to the HSS, and HSS "pushes" the subscribed session events to the registered IMS AS address. This is unnecessarily complex; a simpler model is to let the IMS AS to "pull" the session events from HSS when the user registers to IMS. It is proposed that the IMS AS obtains the session events from HSS along rest of the user data using the existing the Nhss_ImsSDM_Get service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Session events for the Nimsas_SessionEventControl service are stored in HSS as a repository data (using the Service Indication = SessionEventControl as a Data Sub Key). NEF stores the session events to HSS via Nxx reference point. IMS AS can retrieve the session events using N71/Sh interface. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AA.1.2, AA.2.4.2.1, AC.2.2.6, AC.7.2.1, AC.7.x (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************************** Start of Changes ***************************

## AA.1.2	Reference point to support SBA in IMS

Following reference points are realized by service-based interfaces in IMS:

N5:	Reference point between the PCF and an AF.

NOTE:	P-CSCF acts as an AF from PCF point of view. N5 Reference point is defined in TS 23.501 [93].

N70:	Reference point between an SBI capable I/S-CSCF and an SBI capable HSS.

N71:	Reference point between an SBI capable IMS AS and an SBI capable HSS.

DC1:	Reference point between an SBI capable IMS AS and DCSF.

DC2:	Reference point between an SBI capable IMS AS and MF.

N72:	Reference point between DCSF and an SBI capable HSS.

Nxx:	Reference point between NEF and an SBI capable HSS.

*************************** Next Change ***************************

#### AA.2.4.2.1	General

Service description: This service enables the consumer to be notified about session events when served IMS subscribers take part in IMS sessions.

The following operations are available for this service:

-	Explicit subscription to receive session events.  The subscription service data including the subscribed Event IDs can be optionally stored in HSS as repository data (using the Service Indication =  SessionEventControl as a Data Sub Key) by NEF via Nxx reference point. IMS AS can retrieve the service data using N71/Sh interface.

-	Notifying IMS session control events of a specific IMS subscriber to NFs, e.g. DCSF, based on local configuration in the IMS AS.

IMS AS processing of the session event is paused until the consumer acknowledges reception of the session event notification. The consumer is given the possibility to apply session media control and policies by invoking the Nimsas_MediaControl service before giving IMS AS a session event notification acknowledgement.

If the consumer does not apply media control/policies to a session event, IMS AS continues processing and forwards the event as is (including the media state) to calling/called party.

*************************** Next Change ***************************

### AC.2.2.6	HSS

In order to support data channel service, the HSS is additionally enhanced with the following functionalities:

-	The HSS stores IMS Data Channel subscription data as transparent data.

-	The HSS interacts with DCSF during service data retrieval by DCSF via N72/Sc.

-	The HSS interacts with IMS AS during service data retrieval by IMS AS via N71/Sh.

-	The HSS interacts with NEF during service data update by NEF via Nxx/Sh.

*************************** Next Change ***************************

# AC.3	IMS Data Channel Service Subscription

Service subscription for IMS Data Channel shall be an extension to the MMTEL related service profile in HSS. The Data Channel subscription data shall be used by the IMS AS and S-CSCF during IMS registration, session initiation or update to authorize subscribers to use the DC service.

IMS AS Service specific data is enhanced with DC specific service data, optionally stored in HSS (e.g. as repository data) and retrieved by IMS AS using N71/Sh interface. The IMS AS service specific data includes the subscription service data for the Nimsas_SessionEventControl service.

DCSF service specific data used by DCSF for data channel management, may be stored (e.g. as repository data) and retrieved from HSS using the N72/Sc interface. DCSF service specific data are out of scope of 3GPP.

*************************** Next Change ***************************

### AC.7.2.1	Person to Person (P2P) Application Data Channel Setup

Figure AC.7.2.1-1 depicts a signalling flow diagram for establishing an application data channel in a person-to-person use case. In this scenario, the MF is not used to anchor the Application data channel.

In the call flow the UEs have already established an IMS audio session, and the originating UE is updating the IMS audio/video session to an IMS data channel session.

Figure AC.7.2.1-1: Person-to-Person Application Data Channel set up Signalling Procedure

The steps in the call flow are as follows:

0.	IMS registration has been performed. During the IMS registration procedure, the IMS AS retrieves the user subscription data from HSS via N71/Sh interface. The user subscription data includes the service data for the Nimsas_SessionEventControl service.

IMS session and bootstrap data channel have been established. Selected data channel application(s) have been downloaded to UE#1 and possibly UE#2.

1.	UE#1 sends the SIP reINVITE request with an updated SDP to IMS AS, through originating network P-CSCF and S-CSCF. The updated SDP contains the bootstrap data channel information, as well as the requested application data channel and the associated DC application binding information, according to TS 26.114 [76].

NOTE 1:	The SIP re-INVITE can be initiated either from UE#1 or UE#2.

2.	The IMS AS validates user subscription data and uses the service data for the Nimsas_SessionEventControl service (as received in Step 0) to determine whether the media change request event should be notified to the DCSF.

3.	The IMS AS notifies the DCSF, via Nimsas_SessionEventControl_Notify (MediaChangeRequest Event, Session ID, Event Direction, Event initiator, Media Info List) of the media change request event.

4.	After receiving the session event notification, the DCSF determines the policy about how to process the application data channel establishment request based on the related parameters (i.e. associated DC application binding information) in the notification and/or DCSF service specific policy.

5.	The DCSF determines that the added application data channel media descriptor of the SDP offer takes UE#2 as target endpoint and does not require anchoring on the local MF or MRF. If MF or MRF needs to anchor application data channel, DCSF would have used the Nimsas_MediaControl service operation to instruct IMS AS to allocate data channel media resources of the MF or MRF.

6.	DCSF responds to the notification received in step 3.

7-8.	IMS AS sends the re-INVITE to the originating S-CSCF and then to the terminating network side and UE#2.

9-11.	UE#2 and terminating network returns a 200 OK response with SDP answer for application data channel to originating network. Based on the received DC application binding information in the SDP offer of the re-INVITE, UE#2 may need to download the corresponding DC Application signalled in the SDP offer, if not done already and associate it with the requested application DC.

NOTE 2:	The UE at the terminating side is capable to determine if to use the DC application based on the received DC application binding information.

12.	IMS AS notifies the DCSF of the successful data channel modification.

13.	DCSF responds to the notification received in step 12.

14-15.	The IMS AS sends 200 OK response to the originating S-CSCF and P-CSCF.

16.	The originating network P-CSCF executes QoS procedure for application data channel media based on the SDP answer information from the 200 OK response.

17.	P-CSCF returns the 200 OK response to UE#1.

18.	UE#1 sends ACK to the terminating network.

19.	The application data channel between UE#1 and UE#2 is established. In this example, it is not anchored in MF/ MRF.

*************************** Next Change ***************************

## AC.7.x	AF providing the session events

Figure AC.7.x-1 depicts a signalling flow diagram for the AF (e.g. DCAS, DCSF) submitting the service data for the Nnef_SessionEventControl service. The procedure can be used to submit subscriber specific events (i.e. events that are targeted to one or more distinct IMPUs), and non-subscriber specific events (i.e. events that are targeted to any user). In the latter case, the request is targeted to a wildcarded IMPU where the user portion ruled out, e.g. sip:*@operator.com.

Figure AC.7.x-1: AF providing the session events to the NEF

The steps in the call flow are as follows:

1.	The AF (e.g. DSCF or DCAS) uses the Nnef_SessionEventControl_Create/Update service operation to provide subscribed session events for the IMS AS. The service data contains the subscribed Event IDs and the targeted users. The targeted users can be either one distinct IMPU, or multiple users given as a list of distinct IMPUs or as a wildcarded IMPU. The Event IDs in the request are common for all targeted subscribers. The service data contains also the notification target address pointing to the AF.

2. 	The NEF authorizes the request. The NEF may use the NRF service to discover the HSS(s) that are serving the targeted users. The NEF uses the Nhss_ImsSDM_Update service operation to submit the service data (using the Service Indication =  SessionEventControl as a Data Sub Key) to the discovered HSS(s).

3. 	The HSS stores the service data and responds to the NEF.

4.	The NEF responds to the AF.

5.	The user registers to the IMS. The IMS AS receives a 3rd party registration from S-CSCF.

6.	The IMS AS uses the Nhss_ImsSDM_Get service operation to retrieve the user subscription data from HSS.

7.	The HSS responds to the IMS AS, and returns the user subscription data to the IMS AS. The user subscription data includes the service data (session events) for the Nimsas_SessionEventControl service.

8. 	The IMS AS uses the Nhss_ImsSDM_Subscribe service operation to subscribe for the changes in the user subscription data in HSS.

9.	The HSS responds to the IMS AS.

10.	Whenever the AF updates the session events or unsubscribes for the session events, the NEF updates the service data in the HSS.

11. HSS notifies any change in the user data to the IMS AS.

12. IMS AS responds to the HSS.

*************************** End of Changes ****************************